| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12July 2019 | Document: JVET-O0157 |
| --- | --- |

| Title: | Non-CE10: A CNN based in-loop filter for intra frame |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Jianqing Zhu Kimihiko Kazui | Tel: Email: | +86-21-633350606 xuluhang@cn.fujitsu.com |
| Source: | Fujitsu R&D Center |  |  |

_____________________________

- Abstract

A sub-sampled Convolution Neural Network filter is proposed to replace the de-blocking filter (DBF) in VVC. This CNN based in-loop filter is trying to reduce neural network complexity while keeping coding efficiency. It is a light weight CNN and with an effective way to control the CNN filtering strength for processing in different quantization condition. It is only for intra frames. On top of VTM-5.0, simulation results reportedly show 1.44%/2.51%/3.39% BD-rate saving for Y, Cb and Cr components respectively in AI configuration. The decoding time is 1040%. Subjective quality improvement can also be observed.

Introduction

In VVC, De-Blocking Filter (DBF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) are used to remove the artifacts and improve the quality of reconstructed frames

In Geneva meeting, CE10 was set up to test several CNN based in-loop filter. These CNN based filters could be categorized into online training and offline training. There are two concerns raised during discussion in Geneva meeting. One is coding gain and the other is complexity.

This contribution focuses on reducing the complexity of neural network while keeping coding efficiency. It is proposed a light weight CNN and with an effective way to control the CNN filtering strength for processing in different quantisation condition.

Proposed method

Proposed Decoding scheme for intra frame

Figure 2-1 shows the CNN based in-loop filter including CNN filter, SAO and ALF for intra frame process. For inter frame, filters are same as VTM-5.0 [1].

Figure 2-1 – CNN based in-loop filter

CNN in-loop filter

In this section, the network structure, the strength control of filtering and training details are provided.

Network Structure

The proposed CNN structure and residual block structure are shown in Figure 2-2 and Figure 2-3, where N and M denotes the number sub-sampling stride and number of convolution kernel's channel. The core idea of this structure is sub-sampling that to save computation and strength control that to train single model for all QP condition. With the inspiration from WDSR [2] which is a high performance CNN with light weight. The structure in Figure 2-2 is proposed. For further complexity reduction, the 3x3 convolution kernels were replaced by 3x3 depth-wise separable convolution kernel in residual blocks.

Figure 2-2 – CNN structure

Figure 2-3 – Residual block structure

With the outside skip connection, the reconstructed image is compensated by the residual signal that output by CNN.

The division and multiplication of Qstep aims to control the filtering strength.

CNN process:

- The (N, N, M) convolutional layers with a sub-sampling scale N (stride = N) are designed for extracting the features.

- The features are sent to several residual blocks for enhancing the filtering.

- The (3, 3, NxN) convolutional layers outputs NxN feature maps and the DepthToSpace function shuffles feature maps into an input's size image.

Filtering Strength

A simple example of scalar quantization is the process of rounding a fractional number to the nearest integer. A more general example of a uniform quantizer and inverse quantizer are shown in equation (1) and (2):

|  | FQ=round(XQstep) | (1) |
| --- | --- | --- |
|  | Y=FQ×Qstep | (2) |

Where Qstep is a quantization 'step size'.

Round process using different Qstep causes different levels of distortion were found from the equations. Hence the reconstructed image was divided by normalized Qstep before sending to CNN to control the distortion level and multiplied by normalized Qstep after CNN process.

For Qstep approximation, VTM has quantization g_quantScale and inverse quantization g_invquantScale lists. Qstep will be doubled with the increment by 6 of Qp. The relationship between Qp and Qstep and the g_invquantScale were used to approximate the corresponding Qstep in VTM.

The method for approximating normalized Qstep is shown in Table 2-1.

Table 2-1 – Normalized Qstep calculation

1. Get QP from current slice

2.baselist[6] = g_invquantScale[6] / 26

3. a = mod(QP, 6); b = int(QP/6)

4. Qstep = baselist[a] x 2b

5. NormalizedQstep = Qstep / 228.0

Training details

The DIV2K dataset [3] including 800 images is utilized to generate training data for proposed network. Each image of the dataset are converted from RGB to YUV. The loop filters in VTM-5.0[1] was disabled to generate reconstructed images under All Intra (AI) configuration. 800,000 patches of Y component and 800,000 patches of U and V components have been cropped randomly for training two sets of parameters.

Table 2-2 – Training setup

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | 1e-4 |
|  |  | optimizer: | ADAM |
|  |  | batch size: | 500 |
|  |  | epoch: | 400 |
|  |  | loss function: | L2 |
|  |  | training GPU: | GTX TITAN X |
|  |  | training time: | each model 30h |
|  |  | framework: | TensorFlow |
| Optional | Other Settings: |  |  |
|  |  | Patch size | 64x64 |
|  |  | Dataset | DIV2K |
|  |  | QP set for generating training dataset | 22,27,32,37 |

Simulation results

The proposed CNN based in-loop filter is integrated into VTM-5.0 [1] as the replacement of De-Blocking filter. The results of experiments running by CPU are shown in Table 3-1.

Table 3-1 – Proposed Method (N = 4, M = 32)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -0.16% | -2.45% | 103% | 1297% |
| Class A2 | -1.59% | -1.83% | -2.35% | 105% | 1068% |
| Class B | -1.11% | -2.42% | -3.87% | 103% | 1021% |
| Class C | -1.57% | -3.26% | -5.46% | 99% | 791% |
| Class E | -2.18% | -4.66% | -1.81% | 99% | 1204% |
| Overall | -1.44% | -2.51% | -3.39% | 102% | 1040% |
| Class D | -2.03% | -2.81% | -4.21% | 94% | 838% |
| Class F | 0.49% | -2.89% | -3.63% | 103% | 978% |

Table 3-2 shows the process time comparison with other proposed pre-trained CNN based in-loop filter.

The proposal N0513, N0169 and N0254 are the implementation on VTM-4.0 and O0079 and O0157 are on VTM-5.0.

Table 3-2 –Test results’ Comparison

| N0513 | -0.87% | -0.44% | -0.56% | 106% | 1912% |
| --- | --- | --- | --- | --- | --- |
| N0169 | -4.65% | -6.73% | -7.92% | 139% | 37956% |
| N0254 | -1.52% | -2.12% | -2.73% | 107% | 4667% |
| O0079 | -3.93% | -7.89% | -7.29% | 134% | 74517% |
| O0157 | -1.44% | -2.51% | -3.39% | 102% | 1040% |

Figure 3-1 shows one example of the subjective quality improvement by the proposed CNN in-loop filter. A slight block on the man's neck is removed when the proposed filter is used.

| (a) VTM-5.0 | (b) Proposed |
| --- | --- |

Figure 3-1 – Subjective quality improvement (QP=37)

Conclusions

This contribution presents a pre-trained CNN filter which replaces existing deblocking filter in VTM-5.0. Simulation results shows 1.44%/2.51%/3.39% BD-rate savings for luma and two chroma components compared with VTM-5.0 in AI configuration. The decoding time is 1040%.

Reference

- [1] VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.

- [2] WDSR, https://arxiv.org/pdf/1808.08718.pdf.

- [3] DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

Patent rights declaration(s)

Fujitsu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).