| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0158 |
| --- | --- |

| Title: | Non-CE5: Deblocking based on DMVR refined motion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

VTM-5.0 use the motion information before DMVR refinement which make deblocking unaware of differences in motion information that can cause blocking artefacts. Since DMVR refinement is performed on a sub-block size of 16x16 inside a coding unit using DMVR this can also introduce blocking artefacts inside DMVR coding blocks.

This contribution proposes to use DVMR refined motion for deblocking. This gives a significant benefit in subjective quality for some sequences.

Objective results are only tested for random access since DMVR is not used for other coding configurations.

BDR results of -0.03% for RA with ALF on and -0.07% for RA with ALF off are reported.

Encoding time of 100% for RA with ALF on and 103% for RA with ALF off are reported.

Decoding time of 102% for RA with ALF on and 102% for RA with ALF off are reported.

# Introduction

When a coding block use DMVR the motion vectors of the coding block are refined on a subblock size of 16x16. The refined motion vectors are used for motion compensation of the coding block but are not used for deblocking. This makes deblocking take action on coding block boundaries based on non-refined motion that not have been used to generate the prediction. Since there is no variation in non-refined motion vectors inside the coding block, deblocking is unaware of potential subblock boundaries inside the coding block.

This contribution suggests using refined motion for deblocking to combat these issues.

# Problem description

A good show case of the problems in VTM-5.0 due to having deblocking being unaware of DMVR refined motion is the CE5 [1] test sequence Kimono as can be seen in Figure 1.

Figure 1: Kimono, RA, QP 37 Left: POC 133 with ALF OFF, Right: POC 137 with ALF ON

# Proposed solution

It is proposed to deblock boundaries of DMVR refined motion.

# Specification change

The change in the VVC specification [2] is attached with the contribution and can be summarized as follows:

- Remove part of the note in section 8.5.1 that says that un-refined motion is used for deblocking.

- Modify the definition of the number of subblocks to cover all inter subblock tools in section 8.8.3.4.

# Results

## Subjective examples

Example with problem for both ALF on and ALF off and how it is fixed when the proposal only is applied on the problem POC can be seen in Figure 2 and Figure 3.

Figure: Kimono, POC 133, RA QP 37, ALF OFF. Left: VTM-5.0, Right: Proposed

Figure: Kimono, POC 137, RA QP 37, ALF ON. Left: VTM-5.0, Right: Proposed

## Objective performance

The objective performance for the proposal is shown below in comparison to VTM-5.0 [3] and also in comparison to VTM-5.0 with ALF disabled on both anchor and test for random access configuration [4] where DMVR is used. The objective results show a small gain. Encoding time is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running test and anchor on same machine without yuv output.

### Objective results over CTC

|  | Random access Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,06% | -0,02% | 100% | 103% |
| Class A2 | -0,04% | 0,04% | 0,00% | 100% | 104% |
| Class B | 0,00% | -0,02% | -0,02% | 101% | 102% |
| Class C | -0,03% | 0,03% | -0,10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,00% | -0,04% | 100% | 102% |
| Class D | -0,03% | 0,01% | 0,03% | 100% | 102% |
| Class F | 0,00% | 0,01% | 0,01% | 96% | 103% |

### Objective results without ALF

|  | Random access Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,18% | -0,01% | -0,02% | 103% | 102% |
| Class A2 | -0,07% | 0,01% | 0,04% | 104% | 104% |
| Class B | -0,04% | -0,02% | 0,00% | 103% | 102% |
| Class C | -0,03% | 0,00% | -0,02% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,07% | 0,00% | 0,00% | 103% | 102% |
| Class D | -0,04% | 0,03% | 0,01% | 100% | 100% |
| Class F | 0,00% | -0,02% | 0,01% | 105% | 102% |

# Conclusions

We recommend adoption of deblocking of DMVR refine motion to VVC draft. This ensures that blocking artifacts that origins from DMVR also are deblocked.

# References

- C-Y. Chen, A. Norkin, "Description of Core Experiment 5 (CE5): Loop filtering", Document of Joint Video Experts Team, JVET-N1025.

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)", Document of Joint Video Experts Team, JVET-N1001.

- VTM-5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).