| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0159 |
| --- | --- |

| Title: | Non-CE5: Deblocking tC table defined for 10-bit video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

VTM-5.0 uses a tC table defined for 8-bit video which then is upscaled to the bit-depth a video is encoded at.

This contribution proposes to instead define the tC table for 10-bit video to get a better linear dependency between quantization step size and tC for 10-bit video and higher.

BDR results of 0.07%/-0.03%/-0.09%/-0.07% for AI/RA/LDB/LDP with ALF on and 0.06%/-0.03%/-0.04%/-0.04% for AI/RA/LDB/LDP with ALF off are reported.

Encoding time and decoding time unchanged

# Introduction

tC is used to restrict the amount of modification from deblocking according to the QP. The tC table in VTM is defined for 8-bit video and is same as in HEVC but extended in same fashion to cover higher QPs [1]. For 10-bit video the tC values are scaled by 4. Due to the definition of the tC table for 8-bit video the tC values have a quantization noise which can be seen when plotted versus quantization step size 2^(QP/6) where it really should increase linearly with the quantization step size. This gives a noticeable step wise increase of tC at lower QP where tC is small.

This contribution suggests to instead use tC values defined for 10-bit video.

# Proposed solution

The tC table in VVC is defined for 8-bit video and looks as follows:

tC=[0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,1,2,2,2,2,3,3,3,3,4,4,4,5,5,6,6,7,8,9,10,11,13,14,16,18,20,22,25, 28, 31, 35, 39, 44, 50, 56, 63, 70, 79, 88, 99]

For encoding video at 10-bit the tC for the given QP is scaled from 8 to 10-bit before being used by deblocking, all upscaled values are shown below:

tC*4= [0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,4,4,4,4,4,4,4,4,4,8,8,8,8,12,12,12,12,16,16,16, 20,20,24,24,28,32,36,40,44,52,56,64,72,80,88,100,112,124,140,156,176,200,224,252,280,316,352,396]

We propose to instead define the tC table for 10-bit video to get tC to increase linearly with quantization step size also for smaller quantization step sizes for 10-bit video and higher. The proposed tC table looks as follows:

tC=[0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,3,4,4,4,4,5,5,6,6,7,7,8,9,9,10,11,13,14,15,17, 19,21,24,26,29,33,36,41,45,51,57,64,71,80,89,100,112,126,141,158,177,198,222,250,280,314,352,395]

A comparison between tC defined for 8-bit video and tC defined for 10-bit video can be seen in Figure 1 where a zoom in on the range of QPs from 0 to QP equal to 50 is shown. It can be seen that the 8-bit tC (black circle) fluctuate around a linear slope (~+-2) where the proposed tC (green diamond) removes most fluctuations and keep a linear shape. In Figure 2 tC versus QP is shown for both VVC and the proposal.

Figure 1: Plot of tC versus quantization step size for VVC (black circle) and proposed (greed diamond).

Figure 2: Plot of tC versus QP for VVC (black circle) and proposed (greed diamond).

# Specification change

The change in the VVC specification [2] is to update the values of the tC table with the proposed tC values and to generate tC for the current bit-depth starting from 10-bit instead of 8-bit.

tC’ = bitDepht < 10 ? (tC+2)>>(10-bitDepth) : tC<<(bitDepth-10)

# Results

## Objective performance

The objective performance for the proposal is shown below in comparison to VTM-5.0 [3] and also in comparison to VTM-5.0 with ALF disabled on both anchor and test [4]. The objective results show a small gain. Encoding time is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running test and anchor on same machine without yuv output.

### Objective results over CTC

|  | All intra Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,11% | 0,38% | 0,20% | 93% | 100% |
| Class A2 | 0,08% | 0,44% | 0,57% | 93% | 99% |
| Class B | 0,07% | 0,44% | 0,41% | 93% | 101% |
| Class C | 0,04% | 0,43% | 0,44% | 100% | 100% |
| Class E | 0,05% | 0,25% | 0,10% | 92% | 101% |
| Overall | 0,07% | 0,40% | 0,36% | 95% | 100% |
| Class D | 0,04% | 0,37% | 0,35% | 98% | 102% |
| Class F | 0,03% | 0,47% | 0,52% | 94% | 102% |
|  | Random access Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,29% | 0,06% | 99% | 99% |
| Class A2 | -0,03% | -0,13% | -0,30% | 99% | 103% |
| Class B | -0,02% | -0,30% | -0,29% | 98% | 101% |
| Class C | -0,05% | -0,56% | -0,65% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,33% | -0,32% | 99% | 101% |
| Class D | -0,01% | -0,35% | 0,00% | 98% | 101% |
| Class F | 0,00% | -0,62% | -0,70% | 103% | 101% |
|  | Low delay B Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,06% | 0,11% | 0,11% | 99% | 103% |
| Class C | -0,07% | 0,07% | -0,07% | 102% | 100% |
| Class E | -0,16% | 0,17% | -0,09% | 97% | 101% |
| Overall | -0,09% | 0,11% | 0,00% | 99% | 101% |
| Class D | -0,09% | 0,07% | 0,07% | 102% | 100% |
| Class F | -0,11% | -0,24% | -0,28% | 101% | 99% |
|  | Low delay P Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,07% | 0,01% | -0,03% | 102% | 100% |
| Class C | -0,02% | -0,47% | -0,08% | 100% | 101% |
| Class E | -0,13% | 0,69% | -0,19% | 99% | 100% |
| Overall | -0,07% | 0,02% | -0,09% | 101% | 100% |
| Class D | -0,04% | -0,31% | 0,16% | 101% | 99% |
| Class F | -0,06% | -0,05% | 0,05% | 104% | 103% |

### Objective results without ALF

|  | All intra Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,10% | 0,37% | 0,18% | 100% | 100% |
| Class A2 | 0,07% | 0,50% | 0,62% | 99% | 101% |
| Class B | 0,07% | 0,46% | 0,43% | 99% | 101% |
| Class C | 0,04% | 0,47% | 0,47% | 98% | 101% |
| Class E | 0,04% | 0,27% | 0,12% | 96% | 100% |
| Overall | 0,06% | 0,42% | 0,38% | 98% | 101% |
| Class D | 0,04% | 0,39% | 0,37% | 94% | 102% |
| Class F | 0,03% | 0,49% | 0,55% | 98% | 99% |
|  | Random access Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | -0,18% | -0,25% | 102% | 100% |
| Class A2 | -0,05% | -0,23% | -0,34% | 102% | 102% |
| Class B | -0,01% | -0,44% | -0,39% | 101% | 100% |
| Class C | -0,03% | -0,42% | -0,53% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,34% | -0,39% | 102% | 100% |
| Class D | -0,05% | -0,62% | -0,59% | 102% | 100% |
| Class F | -0,02% | -0,80% | -0,75% | 104% | 100% |
|  | Low delay B Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,17% | -0,46% | 100% | 100% |
| Class C | -0,05% | -0,08% | -0,47% | 102% | 98% |
| Class E | -0,07% | -0,60% | -0,24% | 103% | 101% |
| Overall | -0,04% | -0,25% | -0,41% | 102% | 100% |
| Class D | -0,03% | 0,17% | 0,00% | 103% | 98% |
| Class F | -0,16% | -0,25% | 0,20% | 102% | 100% |
|  | Low delay P Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | -0,36% | -0,46% | 104% | 99% |
| Class C | -0,06% | -0,36% | -0,29% | 99% | 99% |
| Class E | 0,00% | 0,18% | 0,68% | 103% | 98% |
| Overall | -0,04% | -0,23% | -0,12% | 102% | 99% |
| Class D | -0,05% | 0,18% | -0,55% | 101% | 101% |
| Class F | -0,07% | -0,53% | -0,71% | 104% | 99% |

# Conclusions

It is proposed to adopt the proposed tC table to VVC draft and the VTM software.

# References

- A. Norkin, "CE11-related: On deblocking tC table", Document of Joint Video Experts Team, JVET-L0410.

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)", Document of Joint Video Experts Team, JVET-N1001.

- VTM-5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.