| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0160 |
| --- | --- |

| Title: | Non-CE3: Sample value clipping on MIP reduced prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a method to apply the sample value clipping on samples in MIP reduced prediction when the current block is predicted with matrix based intra prediction (MIP). The proposed method reduces the sample value clipping operations from the number of samples in the current block to the number of samples in MIP reduced prediction. The proposed method also eliminates the sign value check for rounding when deriving a MIP prediction sample by linear interpolation. The worst case when the current block is a 64 × 64 block, the number of sample value clipping operations reduces from 4096 to 64, the number of sign value checks reduces from 4032 to 0. The proposed method has a luma BDR impact for AI / RA / LDB of -0.00% / -0.00% / -0.03% compared to VTM5.0 and decoding time impact for AI / RA / LDB of 100% / 100% /100% compared to VTM5.0.

# Introduction

In the current VVC, when given a certain block predicted with MIP, the predicted samples are derived based on the following steps:

- Two or four samples are extracted by averaging for each boundary (bdrytop and bdryleft) depending on the dimension of the current block. The extracted averaged boundary samples are named as reduced boundary bdryred.

- A matrix vector multiplication is carried out with the extracted averaged boundary samples as input. The output is a reduced prediction signal consisting of a set of predicted sample values where each predicted sample corresponds to a position in the current block, and where the set of positions is a subset of all positions of the current block. The output reduced prediction signal is named as predred.

- The prediction sample values for the remaining positions in the current block that is not in the set of positions are generated from the reduced prediction signal by linear interpolation which is a single step linear interpolation in each direction (vertical and horizontal). The prediction signal consists of all prediction sample values for the block.

- If H > W, the horizontal linear interpolation is first applied by using the reduced left boundary samples which are named as bdryredleft or bdryredIIleft depending on the current block dimension. The vertical linear interpolation is applied after horizontal linear interpolation by using the original top boundary bdrytop.

- If H ≤ W, the vertical linear interpolation is first applied by using the reduced top boundary samples which are named as bdryredtop or bdryredIItop depending on the current block dimension. The horizontal linear interpolation is applied after vertical linear interpolation by using the original left boundary bdryleft.

- The predicted samples are finally derived by clipping on the predicted signal.

Figure 1 shows an example of MIP process for a 16 × 16 block:

Fig 1. An example of MIP process for a 16 × 16 block.

The reduced prediction signal predred could have one or several samples with a value out of sample value range. The prediction signal at the remaining positions of the current block that is generated from the predred by linear interpolation could have one or several samples with a value out of sample value range. Therefore, the last step of MIP process is to apply sample value clipping on all samples of the MIP predicted samples. Given a W × H MIP predicted block, the number of sample value clipping operations equals to W × H. The worst case of a MIP predicted block needs 4096 sample value clipping operations.

Since the samples in MIP reduced prediction could have a value less than 0, sign value check for rouding is needed when deriving a MIP prediction sample by linear interpolation. Given a W × H MIP predicted block, the dimension of predred is predW × predH, where predW ≤ W and predH ≤ H. The number of sign value checks for rounding equals to (W × H)  (predW × predH). The worst case of a MIP predicted block needs 4032 sign value checks for rounding.

# Proposed method

This contribution proposes to apply sample value clipping on MIP reduced prediction signal predred. The proposed method reduces the number of sample value clipping operations from number of samples in the current block to number of samples in MIP reduced prediction. The worst case when the current block is a 64 × 64 block, the number of sample value clipping operations reduces from 4096 to 64.

Given two samples p[0] and p[2N], where N ≥ 1, the samples p[k] between p[0] and p[2N] are derived by linear interpolation as follows:

p[k] = (k × p[0] + (2N  k) × p[2N] + 2N  1) >> N, where k = 1…(2N1)

- The derived p[k] ≥ minimum(p[0], p[2N])

- The derived p[k] ≤ maximum(p[0], p[2N])

Given the input sample values (p[0] and p[2N]) to the linear interpolation range from 0 to 2bitDepth1, the samples p[k] which are generated by linear interpolation have a value also range from 0 to 2bitDepth1.

With the proposed method, the sample value clipping is applied to the MIP reduced prediction signal predred. The samples of Clip(predred) have a sample value range from 0 to 2bitDepth1. The samples of bdrytop and bdryleft also have a sample value range from 0 to 2bitDepth1. Since all input samples to linear interpolation have a value range from 0 to 2bitDepth1, therefore, it is not necessary to apply sample value clipping on the samples at the remaining positions of MIP prediction block that are derived by linear interpolation.

Table 1 shows the number of sample value clipping operations is reduced by the proposed method for a W × H MIP predicted block:

| Block dimension W × H | Current VTM | Proposed |
| --- | --- | --- |
| 4 × 4 | 16 | 16 |
| 4 × 8 or 8 × 4 | 32 | 16 |
| 4 × 16 or 16 × 4 | 64 | 32 |
| 4 × 32 or 32 × 4 | 128 | 32 |
| 4 × 64 or 64 × 4 | 256 | 32 |
| 8 × 8 | 64 | 16 |
| 8 × 16 or 16 × 8 | 128 | 64 |
| 8 × 32 or 32 × 8 | 256 | 64 |
| 8 × 64 or 64 × 8 | 512 | 64 |
| 16 × 16 | 256 | 64 |
| 16 × 32 or 32 × 16 | 512 | 64 |
| 16 × 64 or 64 × 16 | 1024 | 64 |
| 32 × 32 | 1024 | 64 |
| 32 × 64 or 64 × 32 | 2048 | 64 |
| 64 × 64 | 4096 | 64 |

Table 1. The number of sample value clipping operations for the proposed method compared to current VVC method

Another advantage by using the proposed method is to eliminate the sign value check for rounding when deriving a MIP prediction sample by linear interpolation. As shown in above example, the samples p[k] between p[0] and p[2N] are derived by linear interpolation as:

sum[k] = k × p[0] + (2N  k) × p[2N]

If p[0] and / or p[2N] are negative values, the p[k] is further derived by the rounding and shift as:

- If sum[k] ≥ 0, p[k] = (sum[k] + 2N  1) >> N.

- If sum[k] < 0, p[k] = (sum[k] + 2N  1  1) >> N.

With the proposed method, the negative value samples in MIP reduced prediction have been clipped to 0. Therefore, the rounding can be performed without checking the sign value of sum[k].

The number of sign value checks for rounding equals to the number of samples derived by linear interpolation. The worst case when the current block is a 64 × 64 block, the number of sign value checks for rounding reduces from 4032 to 0. Table 2 shows the number of sign value checks for the proposed method compared to current VVC method.

| Block dimension W × H | Current VTM | Proposed |
| --- | --- | --- |
| 4 × 4 | 0 | 0 |
| 4 × 8 or 8 × 4 | 16 | 0 |
| 4 × 16 or 16 × 4 | 32 | 0 |
| 4 × 32 or 32 × 4 | 96 | 0 |
| 4 × 64 or 64 × 4 | 224 | 0 |
| 8 × 8 | 48 | 0 |
| 8 × 16 or 16 × 8 | 64 | 0 |
| 8 × 32 or 32 × 8 | 192 | 0 |
| 8 × 64 or 64 × 8 | 448 | 0 |
| 16 × 16 | 192 | 0 |
| 16 × 32 or 32 × 16 | 448 | 0 |
| 16 × 64 or 64 × 16 | 960 | 0 |
| 32 × 32 | 960 | 0 |
| 32 × 64 or 64 × 32 | 1984 | 0 |
| 64 × 64 | 4032 | 0 |

Table 2. The number of sign value checks for the proposed method compared to current VVC method.

The current VVC WD is changed as:

8.4.5.2.1 Matrix-based intra sample prediction

…..

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-63)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * boundarySize − 1, y = 0..predC * predC − 1is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-XX.

- The bias vector vBias[ y ] with y = 0..predC * predC − 1 is derived using sizeId and modeId as specified in Table 8-XX.

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-8.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = 1 << ( sW − 1 )				 	 	 (8-64)

sB = BitDepthY – 1						(8-65)

incW = ( predC > mipW )  ?  2  :  1			(8-66)

incH = ( predC > mipH )  ?  2  :  1			(8-67)

predMip[ x ][ y ] = Clip1Y (

( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW ) >> sW )	(8-68)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-69)

predMip =predTemp						(8-70)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If needUpsBdryVer is equal to TRUE or needUpsBdryHor is equal to TRUE, the MIP prediction upsampling process as specified in clause 8.4.5.2.4 is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the upsampling boundary width upsBdryW, the upsampling boundary height upsBdryH, the top upsampling boundary samples upsBdryT, and the left upsampling boundary samples upsBdryL as inputs, and the output is the predicted sample array predSamples.

- Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

- The predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are clipped as follows:

predSamples[ x ][ y ] = Clip1Y( predSamples[ x ][ y ] )	(8-71)

8.4.5.2.2 MIP prediction upsampling process

The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If nTbH is greater than nTbW, the following ordered steps apply:

      - When upHor is greater than 1, horizontal upsampling for all sparse positions ( xHor, yHor ) = ( m * upHor − 1, n * upVer − 1 ) with m = 0..predW − 1, n = 1..predH is applied with dX = 1..upHor − 1 as follows:

sum = ( upHor − dX ) * predSamples[ xHor ][ yHor ] + dX * predSamples[ xHor + upHor ][ yHor ]	(8-84)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 − ( sum < 0  ?  1  :  0 ) ) / upHor	(8-85)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 ) / upHor	(8-85)

      - Vertical upsampling for all sparse positions ( xVer, yVer ) = ( m, n * upVer − 1 ) with m = 0..nTbW − 1, n = 0..predH − 1 is applied with dY = 1..upVer − 1 as follows:

sum = ( upVer − dY ) * predSamples[ xVer ][ yVer ] + dY * predSamples[ xVer ][ yVer + upVer ]	(8-86)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 − ( sum < 0  ?  1  :  0 ) ) / upVer	(8-87)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 ) / upVer	(8-87)

- Otherwise, the following ordered steps apply:

      - When upVer is greater than 1, vertical upsampling for all sparse positions ( xVer, yVer ) = ( m * upHor − 1, n * upVer − 1 ) with m = 1..predW, n = 0..predH − 1 is applied with dY = 1..upVer − 1 as follows:

sum = ( upVer − dY ) * predSamples[ xVer ][ yVer ] + dY * predSamples[ xVer ][ yVer + upVer ]	(8-88)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 − ( sum < 0  ?  1  :  0 ) ) / upVer	(8-89)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 ) / upVer	(8-89)

      - Horizontal upsampling for all sparse positions ( xHor, yHor ) = ( m * upHor − 1, n ) with m = 0..predW − 1, n = 0..nTbH − 1 is applied with dX = 1..upHor − 1 as follows:.

sum = ( upHor − dX ) * predSamples[ xHor ][ yHor ] + dX * predSamples[ xHor + upHor ][ yHor ]	(8-90)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 − ( sum < 0  ?  1  :  0 ) ) / upHor	(8-91)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 ) / upHor	(8-91)

# Simulation result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,03% | -0,02% | 100% | 100% |
| Class A2 | 0,00% | -0,01% | 0,00% | 99% | 100% |
| Class B | 0,00% | 0,02% | 0,03% | 100% | 100% |
| Class C | 0,00% | 0,02% | -0,04% | 99% | 100% |
| Class E | 0,00% | 0,01% | 0,04% | 100% | 100% |
| Overall | 0,00% | 0,01% | 0,00% | 100% | 100% |
| Class D | 0,00% | 0,02% | -0,01% | 100% | 100% |
| Class F | 0,01% | -0,05% | 0,05% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,09% | -0,02% | 100% | 100% |
| Class A2 | 0,00% | 0,02% | 0,02% | 100% | 100% |
| Class B | 0,00% | -0,04% | 0,05% | 100% | 100% |
| Class C | -0,02% | -0,09% | 0,00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | -0,05% | 0,01% | 100% | 100% |
| Class D | 0,01% | 0,05% | 0,05% | 100% | 100% |
| Class F | 0,00% | -0,11% | -0,04% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | 0,05% | 0,59% | 100% | 100% |
| Class C | -0,02% | 0,04% | -0,11% | 100% | 99% |
| Class E | -0,08% | -0,56% | -0,15% | 99% | 100% |
| Overall | -0,03% | -0,10% | 0,17% | 100% | 100% |
| Class D | 0,05% | 0,05% | 0,27% | 100% | 100% |
| Class F | -0,02% | -0,21% | 0,70% | 100% | 99% |

# Conclusions

We propose to adopt sample value clipping on MIP reduced prediction to VVC and VTM to reduce the complexity for a MIP predicted block.

# References

[1]. Versatile Video Coding (Draft 5)

[2]. CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).