| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0161 |
| --- | --- |

| Title: | Non-CE3: Simplification on MIP boundary sample downsampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a method to simplify the matrix based intra prediction (MIP) boundary sample downsampling process when the current block is predicted by MIP. The proposed method 1) enable parallel derivation of the reduced boundary for matrix multiplication and the reduced boundary for interpolation; 2) reduced the number of samples to be averaged to derive one reduced boundary sample. For the first aspect, the proposed method derives the reduced boundary for interpolation without averaging. For the second aspect, the proposed method simplifies the derivation of reduced boundary for matrix multiplication by averaging two original boundary samples to derive one reduced boundary sample. The proposed method has a luma BDR impact for AI / RA / LDB of -0.01% / -0.01% / 0.01% compared to VTM5.0 and decoding time impact for AI / RA / LDB of 100% / 100% /100% compared to VTM5.0.

# Introduction

In the current VVC, when given a certain block predicted with MIP, the predicted samples are derived based on the following steps:

- Two or four samples are extracted by averaging for each boundary (bdrytop and bdryleft) depending on the dimension of the current block. The extracted averaged boundary samples are named as reduced boundary bdryred.

- A matrix vector multiplication is carried out with the extracted averaged boundary samples as input. The output is a reduced prediction signal consisting of a set of predicted sample values where each predicted sample corresponds to a position in the current block, and where the set of positions is a subset of all positions of the current block. The output reduced prediction signal is named as predred.

- The prediction sample values for the remaining positions in the current block that is not in the set of positions are generated from the reduced prediction signal by linear interpolation which is a single step linear interpolation in each direction (vertical and horizontal). The prediction signal consists of all prediction sample values for the block.

- If H > W, the horizontal linear interpolation is first applied by using the reduced left boundary samples which are named as bdryredleft or bdryredIIleft depending on the current block dimension. The vertical linear interpolation is applied after horizontal linear interpolation by using the original top boundary bdrytop.

- If H ≤ W, the vertical linear interpolation is first applied by using the reduced top boundary samples which are named as bdryredtop or bdryredIItop depending on the current block dimension. The horizontal linear interpolation is applied after vertical linear interpolation by using the original left boundary bdryleft.

- The predicted samples are finally derived by clipping on the predicted signal.

Figure 1 shows an example of MIP process for a 16 × 16 block:

Fig 1. An example of MIP process for a 16 × 16 block.

One problem of the MIP boundary sample downsampling process is the two-steps downsampling process to firstly derive the reduced boundary bdryredII for linear interpolation and secondly derive the reduced boundary bdryred for matrix multiplication from bdryredII. The two-steps process prevents parallel derivation of bdryred and bdryredII. Moreover, the samples of bdryredII are derived from average of N boundary samples. This gives filtered boundary samples centered in-between the two boundary samples in the middle and not at the reduced prediction predred in the horizontal or vertical direction. Figure 2 shows an example of the mis-alignment between bdryredIItop and predred.

Fig 2. An example of mis-alignment between bdryredIItop and predred for a 32 × 32 block.

Another problem is the mis-alignment between reduced boundary bdryred for matrix multiplication and reduced prediction predred. The samples of bdryred are derived from average of N boundary samples. This gives filtered boundary samples centered in-between the two boundary samples in the middle. That position is closer to the first of two samples of reduced prediction. This means that the sample of bdryred is biased towards first, third, fifth, etc sample of predred. Fgure 3 shows an example of the mis-alignment between bdryred and predred.

Fig 3. An example of mis-alignment between bdryred and predred for a 32 × 32 block.

# Proposed method

This contribution proposes two aspects of downsampling process to derive the MIP reduced boundary. Aspect one is about the downsampling process to derive the reduced boundary bdryredII for linear interpolation. Aspect two is about the downsampling process to derive the reduced boundary bdryred for matrix multiplication.

First of all, we change the two-step process to derive the reduced boundary bdryred from bdryredII. The samples of bdryred are derived from downsampling the original boundary samples. The downsampling process to derive bdryred and bdryredII can be performed in parallel.

## Aspect one

Downsapling process to derive reduced boundary bdryredII for linear interpolation by selecting a sample from the original boundary without averaging. The samples of bdryredII align with samples of reduced prediction predred in linear interpolation direction.

Given a W × H block, the reduced prediction dimension is predW × predH, where predW < W and predH < H:

- When H ≤ W, the vertical linear interpolation is firstly applied to the reduced prediction predred. The samples of bdryredIItop are derived by selecting every K-th of the original top boundary samples without averaging, where K = W ∕ predW. Figure 4 shows an example of bdryredIItop derivation for an 8 × 8 block.

- When H > W, the horizontal linear interpolation is firstly applied to the reduced prediction predred. The samples of bdryredIIleft are derived by selecting every K-th of the original left boundary samples without averaging, where K = H ∕ predH. Figure 5 shows an example of bdryredIIleft derivation for a 16 × 32 block.

Fig 4. An example of bdryredIItop derivation for an 8 × 8 block.

Fig 5. An example of bdryredIIleft derivation for a 16 × 32 block.

## Aspect two

Downsapling process to derive reduced boundary bdryred for matrix multiplication by averaging two samples from the original boundary. The samples to be used for averaging align with samples of reduced prediction predred in linear interpolation direction.

Given a W × H block, the length of the top reduced boundary bdryredtop is LenW, the downsampling factor bDwn = W ∕ LenW.

- When bDwn = 1, the bdryredtop is set equal to the original top boundary.

- When bDwn > 1, the samples of bdryredtop are derived by averaging two samples of every Mtop-th sample of the original top boundary, where Mtop = bDwn ∕ 2.

Given a W × H block, the length of the top reduced boundary bdryredleft is LenH, the downsampling factor bDwn = H ∕ LenH.

- When bDwn = 1, the bdryredleft is set equal to the original left boundary.

- When bDwn > 1, the samples of bdryredleft are derived by averaging two samples of every Mleft-th sample of the original left boundary, where Mleft = bDwn ∕ 2.

It can be detected that:

- when the current block width W ≤ 8, the samples of bdryredtop derived by aspect 2 are same as current VVC. When the current block width W ≥ 16, the downsampling process for bdryredtop reduces number of samples to be averaged and the samples of bdryredtop centered in-between the two reduced prediction samples in vertical direction.

- when the current block height H ≤ 8, the samples of bdryredleft derived by aspect 2 are same as current VVC. When the current block height H ≥ 16, the downsampling process for bdryredleft reduces number of samples to be averaged and the samples of bdryredleft centered in-between the two reduced prediction samples in horizontal direction.

Figure 6 is an example of bdryred derivation for a 32 × 16 block. The length of the top reduced boundary LenW equals to the length of the left reduced boundary LenH equals to 4.  The Mleft = H ∕ LenH ∕ 2 = 16 ∕ 4 ∕ 2 = 2. The Mtop = W ∕ LenW ∕ 2 = 32 ∕ 4 ∕ 2 = 4.

Fig 6. An example of bdryred derivation for a 32 × 16 block.

## The draft text change when both aspect one and two are applied

The current VVC WD is changed as:

8.4.5.2.3   MIP boundary sample downsampling process

Inputs to this process are:

- a variable nTbS specifying the transform block size,

- reference samples refS[ x ] with x = 0..nTbS − 1,

- a variable boundarySize specifying the downsampled boundary size,

- a flag needUpsBdry specifying whether intermediate boundary samples are required for upsampling,

- a variable upsBdrySize specifying the boundary size for upsampling.

Outputs of this process are the reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 and upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1.

The upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1 are derived as follows:

- If needUpsBdry is equal to TRUE and upsBdrySize is less than nTbS, the following applies:

uDwn = nTbS / upsBdrySize		(8-74)

upsBdryS[ x ] = ( i = 0uDwn - 1refS x*uDwn+i  + ( 1 << ( Log2( uDwn ) − 1 ) )) >> Log2( uDwn )	(8-75)

upsBdryS[ x ] = refS[ x * uDwn + uDwn – 1]				(8-75)

- Otherwise (upsBdrySize is equal to nTbS), upsBdryS[ x ] is set equal to refS[ x ].

The reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 are derived as follows:

- If boundarySize is less than upsBdrySize, the following applies:

bDwn = upsBdrySize / boundarySize					(8-76)

redS[ x ] = ( i = 0bDwn - 1upsBdryS x*bDwn+i  + ( 1 << ( Log2( bDwn ) − 1 ) )) >> Log2( bDwn )	(8-77)

Otherwise (boundarySize is equal to upsBdrySize), redS[ x ] is set equal to upsBdryS[ x ].

- If boundarySize is less than nTbS, the following applies:

bDwn = nTbS / boundarySize							(8-76)

bDwnHalf = bDwn / 2								(8-77)

redS[ x ] = ( refS[ x * bDwn + bDwnHalf – 1] + refS[ x * bDwn + bDwn – 1 ] + 1) >> 1	(8-78)

- Otherwise (boundarySize is equal to nTbS), redS[ x ] is set equal to refS[ x ].

# Simulation result

## Results when both aspect one and aspect two are applied

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,06% | -0,09% | 100% | 100% |
| Class A2 | 0,00% | -0,11% | -0,09% | 100% | 100% |
| Class B | -0,02% | -0,07% | -0,02% | 100% | 100% |
| Class C | 0,00% | -0,08% | 0,04% | 99% | 100% |
| Class E | -0,02% | -0,17% | 0,01% | 100% | 100% |
| Overall | -0,01% | -0,09% | -0,02% | 100% | 100% |
| Class D | 0,00% | 0,11% | -0,11% | 100% | 100% |
| Class F | -0,01% | -0,01% | 0,07% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,29% | 0,05% | 101% | 100% |
| Class A2 | 0,02% | 0,04% | -0,02% | 100% | 100% |
| Class B | 0,00% | -0,06% | -0,18% | 99% | 100% |
| Class C | -0,03% | -0,09% | -0,20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,10% | -0,11% | 100% | 100% |
| Class D | 0,02% | -0,06% | 0,21% | 100% | 100% |
| Class F | -0,03% | -0,08% | 0,12% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | 0,10% | 0,02% | 100% | 100% |
| Class C | 0,03% | 0,14% | 0,25% | 99% | 100% |
| Class E | 0,01% | 0,25% | 0,91% | 99% | 100% |
| Overall | 0,01% | 0,15% | 0,32% | 100% | 100% |
| Class D | 0,02% | 0,35% | 0,60% | 100% | 99% |
| Class F | -0,08% | -0,15% | 0,81% | 100% | 100% |

## Results when only aspect one is applied

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | -0,11% | -0,07% | 100% | 99% |
| Class A2 | -0,01% | -0,09% | -0,11% | 100% | 100% |
| Class B | -0,02% | -0,08% | 0,03% | 100% | 99% |
| Class C | 0,00% | -0,08% | 0,10% | 99% | 101% |
| Class E | -0,01% | -0,07% | -0,03% | 99% | 103% |
| Overall | -0,01% | -0,09% | 0,00% | 100% | 100% |
| Class D | 0,01% | 0,12% | -0,20% | 100% | 102% |
| Class F | 0,00% | 0,05% | 0,04% | 100% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,24% | 0,01% | 100% | 101% |
| Class A2 | 0,00% | 0,07% | 0,04% | 100% | 99% |
| Class B | -0,01% | -0,04% | -0,27% | 100% | 100% |
| Class C | 0,00% | -0,08% | -0,30% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,07% | -0,16% | 100% | 100% |
| Class D | -0,01% | 0,09% | 0,27% | 100% | 100% |
| Class F | 0,03% | -0,04% | 0,18% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | 0,22% | -0,07% | 102% | 101% |
| Class C | 0,07% | 0,11% | 0,18% | 101% | 100% |
| Class E | 0,06% | -1,18% | 0,66% | 100% | 96% |
| Overall | 0,04% | -0,17% | 0,19% | 101% | 99% |
| Class D | -0,01% | -0,96% | 0,22% | 100% | 99% |
| Class F | -0,01% | 0,00% | -0,67% | 99% | 103% |

# Conclusions

We propose to adopt the proposed methods to VVC and VTM to simplify the MIP boundary sample downsampling process for a MIP predicted block.

# References

[1]. Versatile Video Coding (Draft 5)

[2]. CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).