| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0166 |
| --- | --- |

| Title: | CE8-related: BDPCM for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare b<>com 1219 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France Félix Henry Orange Labs 4 rue Clos Courtel 35510 Cesson-Sévigné, France | Tel: Email: | +332991245558 gordon.clare@b-com.com felix.henry@orange.com |
| Source: | Orange, b<>com |  |  |

_____________________________

# Abstract

In the current design of the VVC specification, BDPCM is applied on the luma component only. This contribution proposes to apply it also to the 	chroma components. A single additional flag is used to signal the use of BDPCM on chroma components. When BDPCM is used on chroma components, a single direction flag is sent for all chroma components at block level.

The proponents report the following BD-rate change on TGM444 1080p formats in single tree configuration:

All Intra:

Luma		Chroma1	Chroma2 	EncT 	DecT

YUV	-0,58%		-3,23%		-3,45%		103%	97%

RGB	-3,55%		-5,57%		-5,93%		103%	96%

Random Access:

Luma		Chroma1	Chroma2 	EncT 	DecT

YUV 	-0,46%		-2,04%		-2,08%		100%	102%

RGB 	-2,49%		-3,83%		-4,05%		101%	100%

# Introduction

The current design of VVC uses Block-DPCM (or BDPCM) as adopted from JVET-N0413 [1]. This consists of a vertical or horizontal prediction for the quantized residues of a luma block.

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller than or equal to 32. This flag indicates whether regular intra coding or BDPCM is used for that CU.

If BDPCM is used, a bdpcm_dir flag is transmitted to indicate if BDPCM will be applied in the vertical or horizontal direction for that luma CU.

In the current design, BDPCM is not applied to chroma components.

# Normative Proposal

The present contribution proposes to modify the design of VVC by allowing BDPCM to be used on the chroma components. The proposed modifications are the following.

A syntax element  sps_bdpcm_enable_flag is added to the SPS in order to enable the use of BDPCM (this part is essentially a bugfix and is independently proposed in JVET-O0165 [4])

If sps_bdpcm_enable_flag is 1, a syntax element sps_bdpcm_chroma_enable_flag is added to the SPS.

The flags have the following behaviour, as indicated in table 1

| sps_bdpcm_enable_flag | sps_bdpcm_chroma_enable_flag | behaviour |
| --- | --- | --- |
| 0 | not written | BPDCM is not used in the sequence |
| 1 | 0 | BDPCM is available for luma only |
| 1 | 1 | BDPCM is available for luma and chroma |

When BDPCM is available for luma only, the current behaviour is unchanged.

When BDPCM is also available for chroma, a bdpcm_chroma_flag is sent for each chroma block. This indicates whether BDPCM is used on the chroma blocks. When it is on, BDPCM is used for both chroma componenents, and an additional bdpcm_dir_chroma flag is coded, indicating the prediction direction used for both chroma components.

# Common Testing Conditions Proposal

In the current CTCs, BDPCM is active only for Screen Content (Class F and TGM), and only for the luma components.

It is proposed that, if 4:4:4 becomes part of the future CTCs, BDPCM is active both for luma and for chroma and only on 4:4:4 sequences.

It is also proposed that BDPCM is active for chroma in 4:4:4 screen content core experiment CTCs.

# Results

The proponents have tried to stay as close as possible to the test conditions of CE8 as described in [2].

The reference uses VTM5 with CE8 configuration (IBC on, HashME on, BDPCM on).

The tested software uses the same configuration with the proposed modifications.

The sequences used for 4:4:4 YUV are the ones recommended by CE8 [2].

The sequences used for 4:4:4 RGB are from JCTVC-U1015 [3].

The following sections report the results for All Intra and Random Access. Other configurations are reported in the joint excel sheet.

## Single tree YUV

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0,48% | -2,91% | -3,05% | 102% | 97% |
| TGM 720p | 0,36% | -1,78% | -2,30% | 103% | 97% |
| Animation | 0,32% | -0,35% | -0,33% | 103% | 99% |
| Mixed content | 0,60% | -2,14% | -2,26% | 103% | 98% |
| Camera-Captured content | 0,25% | -0,07% | -0,10% | 103% | 98% |
| Overall | 0,16% | -1,73% | -1,93% | 103% | 98% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0,46% | -2,04% | -2,08% | 100% | 102% |
| TGM 720p | 0,00% | -1,33% | -1,80% | 101% | 103% |
| Animation | 0,10% | -0,29% | -0,18% | 101% | 101% |
| Mixed content | 0,08% | -2,38% | -2,24% | 101% | 101% |
| Camera-Captured content | 0,04% | 0,17% | 0,24% | 101% | 102% |
| Overall | -0,09% | -1,39% | -1,47% | 101% | 102% |

## Single tree RGB

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -3,55% | -5,57% | -5,93% | 103% | 97% |
| TGM 720p | -0,66% | -2,30% | -2,20% | 104% | 98% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2,49% | -3,83% | -4,05% | 101% | 100% |
| TGM 720p | -1,11% | -2,28% | -2,08% | 101% | 103% |

## Dual tree YUV

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0,08% | -2,45% | -2,39% | 103% | 95% |
| TGM 720p | 0,41% | -1,48% | -1,75% | 104% | 97% |
| Animation | 0,31% | 0,52% | 0,81% | 104% | 95% |
| Mixed content | 0,38% | -1,66% | -1,55% | 103% | 97% |
| Camera-Captured content | 0,08% | 0,35% | 0,44% | 103% | 97% |
| Overall | 0,22% | -1,27% | -1,24% | 103% | 96% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0,60% | -2,06% | -1,96% | 101% | 100% |
| TGM 720p | 0,10% | -1,35% | -1,43% | 101% | 100% |
| Animation | 0,22% | 0,14% | 0,26% | 101% | 100% |
| Mixed content | 0,16% | -1,79% | -1,45% | 101% | 100% |
| Camera-Captured content | 0,03% | 0,12% | 0,05% | 101% | 101% |
| Overall | -0,07% | -1,23% | -1,15% | 101% | 100% |

## Dual tree RGB

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2,93% | -4,29% | -4,58% | 104% | 95% |
| TGM 720p | -0,38% | -1,55% | -1,39% | 105% | 96% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2,12% | -3,32% | -3,52% | 101% | 101% |
| TGM 720p | -0,75% | -1,63% | -1,48% | 101% | 100% |

# Bibliography

[1] JVET-N0413 	CE8-related: Quantized residual BDPCM 	M. Karczewicz, M. Coban (Qualcomm)

[2] JVET-N1028 	Description of Core Experiment 8 (CE8): Screen Content Coding Tools 	X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

[3] JCTVC-U1015 Common Test Conditions for Screen Content Coding, 	H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)

[4] JVET-O0165 CE8-related: SPS flag for BDPCM 	F. Henry (Orange), G. Clare (bcom)

# Patent rights declaration(s)

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).