#### Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- the availability flag availableFlagA1 of the neighbouring coding unit,

- the reference index refIdxLXA1 of the neighbouring coding unit with X being 0 or 1,

- the prediction list utilization flag predFlagLXA1 of the neighbouring coding unit with X being 0 or 1,

- the motion vector in 1/16 fractional-sample accuracy mvLXA1 of the neighbouring coding unit with X being 0 or 1.

Outputs of this process are:

- the availability flag availableFlagSbCol,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the reference indices refIdxL0SbCol and refIdxL1SbCol,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0SbCol[ xSbIdx ][ ySbIdx ] and mvL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the prediction list utilization flags predFlagL0SbCol[ xSbIdx ][ ySbIdx ] and predFlagL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The availability flag availableFlagSbCol is derived as follows.

- If one or more of the following conditions is true, availableFlagSbCol is set equal to 0.

- slice_temporal_mvp_enabled_flag is equal to 0.

- sps_sbtmvp_enabled_flag is equal to 0.

- cbWidth is less than 8.

- cbHeight is less than 8.

- Otherwise, the following ordered steps apply:

- The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows:

xCtb = ( xCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-514)

yCtb = ( yCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-515)

-

-

- The derivation process for subblock-based temporal merging mv shift as specified in clause 8.5.5.4 is invoked with the location ( xCtb, yCtb ),  the availability flag availableFlagA1, and the prediction list utilization flag predFlagLXA1, and the reference index refIdxLXA1, and the motion vector mvLXA1, with X being 0 and 1 as inputs and the temporal motion vector tempMv as outputs.

Intialize availableFlagSbCol to 1,

- The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows:

numSbX  =  cbWidth >> 3		(8-518)

numSbY  =  cbHeight >> 3		(8-519)

sbWidth  =  cbWidth / numSbX		(8-520)

sbHeight  =  cbHeight / numSbY		(8-521)

refIdxLXSbCol  =  0			(8-522)

For xSbLoopIdx = 0..numSbX − 1 and ySbLoopIdx = 0 .. numSbY − 1, and if availableFlagSbCol equals to 1,

- Set the sub block index.

xSbIdx =( xSbLoopIdx+( numSbX>>1))% numSbX

ySbIdx==( ySbLoopIdx+( numSbY>>1))% numSbY

- the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth + (sbWidth>>1)	(8-523)

ySb  =  yCb + ySbIdx * sbHeight + (sbHeight>>1)	(8-524)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

xColSb = Clip3( xCtb, 
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(8-525)
 	xSb + ( tempMv[0]  >>  4 ) )

yColSb = Clip3( yCtb, 
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ),	(8-526)
 	ySb + ( tempMv[1]  >>  4 ) )

- The variable currCb specifies the luma coding block covering the current coding subblock inside the current picture.

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColSb >> 3 ) << 3, ( yColSb >> 3 ) << 3 ) inside the ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.5.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL0 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL0SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL0SbCol.

- The derivation process for collocated motion vectors as specified in clause 8.5.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL1 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL1SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL1SbCol.

- If (xSbLoopIdx == 0) and (ySbLoopIdx == 0 )

The variable availableFlagSbCol is derived as follows:

–	If both availableFlagL0SbCol and availableFlagL1SbCol are equal to 0, availableFlagSbCol is set equal to 0.

–	Otherwise, set the base motion data as

ctrMvLX = mvLXSbCol[ xSbIdx ][ ySbIdx ] 						(8-52x)

ctrPredFlagLX = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]                                                           (8-52x)

with X being 0 and 1.

- Otherwise ((xSbLoopIdx != 0) or (ySbLoopIdx == 0 )), and when availableFlagL0SbCol and availableFlagL1SbCol are both equal to 0, the following applies for X being 0 and 1:

mvLXSbCol[ xSbIdx ][ ySbIdx ] = ctrMvLX		(8-527)

predFlagLXSbCol[ xSbIdx ][ ySbIdx ] = ctrPredFlagLX	(8-528)

#### Derivation process for subblock-based temporal merging mv shift

Inputs to this process are:

- the location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block,

- the availability flag availableFlagA1 of the neighbouring coding unit,

- the reference index refIdxLXA1 of the neighbouring coding unit,

- the prediction list utilization flag predFlagLXA1 of the neighbouring coding unit,

- the motion vector in 1/16 fractional-sample accuracy mvLXA1 of the neighbouring coding unit.

Outputs of this process are:

- ,

- the temporal motion vector tempMv.

The variable tempMv is set as follows:

tempMv[ 0 ] = 0		(8-529)

tempMv[ 1 ] = 0		(8-530)

The variable currPic specifies the current picture.

When availableFlagA1 is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMv is set equal to mvL1A1:

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0,

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- Otherwise if all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0.

-

-

-

-

