| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0167 |
| --- | --- |

| Title: | Non-CE4: On SbTMVP base motion data derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In Subblock-based TMVP, base motion data is firstly derived. When sub-CU MV is not available, base motion data is used as sub-CU’s motion data. Both base motion data and sub-CU motion data derivation are similar to TMVP and require temporal motion vector scaling. It is asserted that a separate base motion data derivation process is unnecessary. This contribution proposes to simplify SbTMVP base motion data derivation by aligning it with sub-CU motion derivation position, and reuse motion data of a sub-CU as base motion data instead. This reduces the number of MV scaling and reduces the SbTMVP complexity. Results show that there is no loss. Two tests are done:

(1) Use MV of sub-CU near center of the CU as base motion data. Results are: In RA configuration, 0.00%, -0.03%, -0.03% for Y,U,V respectively; in LD configuration,  0.00%, 0.08%, -0.08% for Y,U,V respectively

(2) Use MV of the top-left sub-CU as base motion data. Results are: In RA configuration, 0.00%, -0.01%, 0.03% for Y,U,V respectively; In LD configuration, 0.04%, 0.09%, 0.00% for Y,U,V respectively.

# Introduction

In VTM5, Subblock-based temporal motion vector prediction (SbTMVP) may be applied to blocks of size 8x8 and larger. SbTMVP uses the motion field in the collocated picture to improve motion vector prediction and merge mode for CUs in the current picture.

The SbTVMP process is illustrated in Figure 1.  SbTMVP predicts the motion vectors of the sub-CUs within the current CU in three steps:

- Obtain MV shift from neighbor A1.

- If A1 has a motion vector that uses the collocated picture as its reference picture, this motion vector is selected to be the motion shift.

- Otherwise, the motion shift is set to (0, 0).

- This MV shift is added to the coordinates to current CU or sub-CU to find the collocated CU or sub-C.

- Obtain base motion data. Base motion data is obtained from the corresponding block that covers the center of the collocated CU (with MV shift).

- If the corresponding block is not coded in inter mode, availableFlagSbCol is set as not available and SbTMVP process stops.

- Otherwise, the motion information of the corresponding block is used to derive the base motion data.

- If availableFlagSbCol is true, Obtain sub-CU motion information. Sub-CU is of size 8x8.

- For each sub-CU, the motion information of its corresponding block that covers the center sample of the collocated sub-CU is used to derive the motion information for the sub-CU.

- If motion data of sub-CU is not available, uses base motion data as the sub-CU’s motion data.

Fig. 1 The SbTMVP process in VVC

In both step 2 and 3, after the motion information of the collocated block is identified, it is converted to the motion vectors and reference indices of the current CU or sub-CU in a similar way as the TMVP process, where temporal motion scaling is applied to align the reference pictures of the temporal motion vectors to those of the current CU.

# Proposed method

| - a 32x32 CU | - an 8x16 CU |
| --- | --- |

Fig. 2. Examples of sample positions for Base and sub-CU motion data derivations. Dark circle indicates sample position for base motion data derivation, and ‘x’ indicates sample position for sub-CU motion derivation.

In VVC5 SbTMVP, when availableFlagSbCol is true and sub-CU motion does not exist, it is filled with base motion data. The scaling processes of base motion data and Sub-CU motion data are the same except that they are having different corresponding block positions. The base motion data is derived from the block covering the center of the collocated CU (shown as the dark dot in Fig. 2), while sub-CU motion data is derived from the block covering the center of each collocated sub-CU ( shown as ‘x’ sign in Fig. 2) . The center of the collocated CU is different from centers of any of the sub-CU. Therefore, to obtain base motion data, it needs a separate derivation process. In this process, for each list, it needs to get the collocated motion vectors, and performs a MV scaling based on POC distance. It is asserted that this separate base motion data derivation process is unnecessary.

We propose to align the sample position for base motion data derivation with the sample position for a sub-CU motion data derivation. In another word, re-use motion data of a sub-CU as the base motion data. Then it does not need a separate base motion data derivation process.

In terms of which sub-CU to use for base motion data, two options are tested:

- Use the sub-CU near the center of the CU. Given numSbX and numSbY as the number of sub-CUs in X and Y directions, sub-CU at (numSbX>>1, numSbY>>1) is used.

- Use the top left sub-CU.

The top-left sub-CU is the first block in sub-CU processing order. So if use the top-left sub-CU as base motion data, no need to changed existing sub-CU processing order. If using the center sub-CU as base motion data, sub-CU at (numSbX>>1, numSbY>>1) needs to be checked first. Since there are no dependencies between sub-CUs, sub-CU processing order can be re-arranged to start from (numSbX>>1, numSbY>>1). This can be easily done by arranging the loop index and sub-CU index. Details are shown in the propsoed draft text change, where many redundant text in 8.5.5.4 “Derivation process for subblock-based temporal merging base motion data” are removed.

After deriving the motion data of the selected sub-CU for base-motion data, the following is done:

- If the sub-CU motion data is not avaialble, set availableFlagSbCol to be false, stops SbTMVP process. This is same as the existing work flow when base motion data is not available.

- Otherwise, set this sub-CU motion as base motion data and continue motion data derivation for other sub-CUs.

In this way, base motion data is set after coding a sub-CU. It does not need an extra process to derive the base motion data, and saves the process to determine and clip the center CU positions and saves the MV temporal scaling used to derive the base motion data.

# Experimental Results

Proposed methods have been implemented into VTM5.0 and tested following CTC test condition [1]. Two tests are done.

- Use motion data of the center sub-CU at (numSbX>>1, numSbY>>1) as base motion data

- Use motion data of the top-left sub-CU as base motion data.

Table 1. Result of using Center Sub-CU as SbTMVP base motion data

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.19% | -0.09% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.04% | -0.04% | 100% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.05% | -0.01% | 100% | 100% | -0.01% | 0.20% | 0.17% | 99% | 95% |
| Class C | -0.01% | -0.01% | 0.00% | 100% | 98% | 0.01% | -0.05% | -0.25% | 99% | 96% |
| Class E |  |  |  |  |  | -0.01% | 0.03% | -0.25% | 100% | 101% |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 100% | 0.00% | 0.08% | -0.08% | 99% | 97% |
| Class D | 0.01% | -0.03% | 0.04% | 100% | 99% | 0.01% | 0.02% | 0.64% | 100% | 102% |
| Class F | 0.00% | 0.01% | -0.01% | 100% | 100% | 0.03% | 0.24% | 1.18% | 100% | 98% |

Table 2. Result of using Top-left Sub-CU as SbTMVP base motion data

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.15% | 0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | 0.05% | 0.07% | 99% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.07% | 0.05% | 100% | 101% | 0.01% | 0.28% | -0.13% | 99% | 95% |
| Class C | -0.02% | -0.03% | -0.04% | 100% | 99% | 0.02% | -0.05% | 0.06% | 100% | 97% |
| Class E |  |  |  |  |  | 0.12% | -0.06% | 0.14% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.03% | 100% | 100% | 0.04% | 0.09% | 0.00% | 100% | 97% |
| Class D | 0.01% | 0.02% | 0.09% | 100% | 99% | 0.01% | -0.08% | 0.63% | 100% | 100% |
| Class F | 0.00% | -0.02% | -0.05% | 100% | 100% | 0.07% | 0.23% | 0.41% | 100% | 98% |

As shown the results, using sub-CU motion data as SbTMVP base motion data has no loss. Using the center sub-CU has slightly better performance than using the top-left sub-CU.

# Conclusion

This contribution proposes to simplify SbTMVP base motion data derivation by using motion data of a sub-CU as base motion. This removes the separate base motion data derivation step, reduces unnecessary motion vector scalings.

Under CTC test condition, results of using the center sub-CU for base motion data are 0.00%, -0.03%, -0.03% for Y,U,V respectively in RA configuration; and 0.00%, 0.08%, -0.08% for Y,U,V respectively in LD configuration. Results of using the top-left sub-CU for base motion data are 0.00%, -0.01%, 0.03% for Y,U,V in RA configuration; and 0.04%, 0.09%, 0.00% for Y,U,V in LD configuration.

We propose to adopt using the center sub-CU for base motion data derivation into VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N0010, Geneva, CH, Mar 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).