| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0169 |
| --- | --- |

| Title: | CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the whole MPM derivation for the MIP cases. So no MPM mode would be added in encoder side. Also all the MIP modes are coded with truncated binary code.

The reported result is 0.01%, 0.00 % and 0.02 % luma BD-rates differences for the AI, RA, and LB settings respectively, under common test conditions.

# Introduction

In VTM5.0, if an MIP mode is to be applied, the index predmode of the MIP mode is signaled using a MPM-list with 3 MPMS [1]. The derivation of the MPMs is performed using the intra-modes of the above and the left PU as follows. There are three fixed tables map_angular_to_alwipidx, idx∈{0,1,2} that assign to each conventional intra prediction mode predmodeAngular an MIP mode

predmodeALWIP= map_angular_to_alwipidx[predmodeAngular].

For each PU of width W and height H one defines an index

idxPU=idx(W,H)∈{0,1,2}

that indicates from which of the three sets the MIP-parameters are to be taken as in Section 1.3 above.

If the above Prediction Unit PUabove is available, belongs to the same CTU as the current PU and is in intra mode, if idxPU=idx(PUabove) and if MIP is applied on PUabove with MIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwipidx(PUabove)[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode modeALWIPleft.

Finally, three fixed default lists listidx, idx∈{0,1,2} are provided, each of which contains three distinct MIP modes. Out of the default list listidx(PU) and the modes  modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.

Based on our experiment the whole MPM derivation process in all the cases (either encoder decision or MIP index coding) does not justify its complexity considering its negligible effect on the performance.

# Proposed Method

In this contribution we propose to remove the MPM derivation for MIP cases. Currently there are 35, 19, and 11 modes for MIP index 0,1, and 2 respectively. We code all the MIP modes using truncated binary code without any context coded bin.

Also in the encoder we remove the part of adding MPM modes for the Full RD in check. So no where in the code MPM mode will be derived for MIP cases.

# Experimental results

The proposed method has been implemented based on VTM 5.0 reference software according to common test conditions defined in JVET-N1010 [1].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.08% | 99% | 100% |
| Class A2 | 0.02% | -0.07% | -0.04% | 100% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 99% | 100% |
| Class C | 0.00% | -0.01% | 0.06% | 99% | 99% |
| Class E | 0.02% | 0.00% | 0.01% | 99% | 99% |
| Overall | 0.01% | -0.01% | 0.02% | 99% | 100% |
| Class D | -0.01% | 0.15% | -0.03% | 99% | 99% |
| Class F | 0.01% | 0.00% | 0.07% | 100% | 99% |
| Class SCC | 0.01% | 0.03% | 0.02% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.10% | 100% | 101% |
| Class A2 | 0.01% | 0.07% | 0.00% | 100% | 101% |
| Class B | 0.01% | 0.06% | -0.07% | 100% | 100% |
| Class C | -0.02% | -0.10% | -0.16% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class D | 0.01% | 0.22% | 0.17% | 99% | 97% |
| Class F | -0.01% | -0.03% | -0.03% | 100% | 100% |
| Class SCC | -0.04% | -0.01% | -0.08% | 99% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.20% | 101% | #NUM! |
| Class C | 0.03% | 0.11% | 0.01% | 101% | 109% |
| Class E | 0.04% | -0.35% | 0.40% | 101% | 106% |
| Overall | 0.02% | 0.05% | 0.19% | 101% | #NUM! |
| Class D | 0.02% | -0.84% | 0.28% | 100% | 103% |
| Class F | 0.04% | 0.82% | -0.28% | 100% | 103% |
| Class SCC | -0.04% | -0.04% | -0.05% | 101% | 107% |

# Conclusion

In the contribution it is proposed to remove the whole MPM derivation for the MIP cases. The proposed method has negligible change in the performance. It is recommended that the method be adopted in the next VTM.

# Reference

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0207, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).