| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0171 |
| --- | --- |

| Title: | Non-CE3: Simplifications for MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, two simplifications are proposed for Matrix based Intra Prediction (MIP) mode. Firstly, it is proposed to always map MIP mode to Planar mode when constructing the MPM list of a conventional intra coding unit so that the look-up table mapping MIP modes to conventional intra prediction modes is saved. Secondly, it is proposed to directly signal the mode index of MIP mode with truncated binary codeword to avoid constructing a separate MPM list for MIP mode. It is reported that both the proposed methods obtain 0.01% luma BD-rate change for the AI configuration on top of VTM-5.0 with similar encoding and decoding run-time.

# Introduction

Matrix based intra prediction mode (MIP) [1] was adopted in VTM-5.0. For a WxH CU, MIP defines an index, which is listed below:

idxW,H=0for W=H=41for maxW,H=82formaxW,H>8.                          (1)

MIP has different number of modes for CU with different indices

number of MIP modes=35idxW,H=019idxW,H=111idxW,H=2.                     (2)

For prediction process, MIP modes are mapped to MatrixId as follows:

MatrixId=modefor idxW,H=0 and mode<18mode-17for idxW,H=0 and mode≥18modefor idxW,H=1 and mode<10mode-9for idxW,H=1 and mode≥10modefor idxW,H=2 and mode<6mode-5for idxW,H=2 and mode≥6.                                  (3)

For each intra Coding Unit (CU), if an MIP mode flag is signaled, the index predmode of the MIP mode is signaled using an MPM-list with 3 MPMs. The derivation of the MPMs is performed using the intra modes of the above and the left CUs. There are three fixed tables map_angular_to_mipidx, idx∈{0,1,2} that assign each conventional intra prediction mode predmodeAngular to a MIP mode

predmodeMIP= map_angular_to_mipidx[predmodeAngular].              (4)

For the MPM list derivation of a conventional intra CU, another three fixed tables map_mip_to_angularidx, idx∈{0,1,2}, are used to map an MIP-mode predmodeMIP to one of the conventional intra-prediction modes

predmodeAngular=map_mip_to_angularidx(CU)[predmodeMIP].            (5)

# Proposed methods

Method #1: it is proposed to always map MIP mode to Planar mode for the MPM list derivation of a conventional intra CU. As a result, the look-up table in equation (5) can be saved.

Method #2: it is proposed to directly signal the mode index of MIP mode with truncated binary codeword. As a result, the MPM derivation process for MIP mode is avoided and the look-up table in equation (4) is also saved.

# Test results

The proposed two methods in this contribution are implemented on top of VTM5.0 software [2]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [3]. Four tests are conducted to evaluate the performance of the proposed methods which are listed below:

Table 2: Results of proposed method #1 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 100% | 101% | 0.01% | -0.02% | 0.22% | 100% | 101% |
| Class A2 | 0.03% | 0.02% | -0.02% | 100% | 100% | 0.07% | 0.21% | 0.06% | 100% | 101% |
| Class B | 0.01% | -0.04% | 0.09% | 99% | 97% | 0.01% | 0.04% | 0.01% | 99% | 97% |
| Class C | 0.00% | -0.12% | 0.05% | 100% | 100% | -0.02% | 0.15% | 0.01% | 99% | 96% |
| Class E | 0.03% | 0.01% | 0.03% | 100% | 99% |  |  |  |  |  |
| Overall | 0.01% | -0.02% | 0.04% | 100% | 99% | 0.01% | 0.09% | 0.06% | 99% | 98% |
| Class D | -0.03% | 0.19% | 0.00% | 99% | 102% | 0.00% | 0.07% | 0.00% | 99% | 97% |
| Class F | -0.02% | -0.02% | 0.08% | 99% | 96% | 0.00% | 0.13% | 0.16% | 98% | 96% |

Table 3: Results of proposed method #2 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.08% | 99% | 100% | 0.01% | -0.05% | 0.10% | 99% | 101% |
| Class A2 | 0.02% | -0.07% | -0.04% | 99% | 100% | 0.01% | 0.06% | 0.00% | 99% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 99% | 98% | 0.01% | 0.06% | -0.07% | 99% | 98% |
| Class C | 0.00% | -0.01% | 0.06% | 99% | 100% | -0.02% | -0.10% | -0.16% | 99% | 97% |
| Class E | 0.02% | 0.00% | 0.01% | 99% | 100% |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.02% | 99% | 100% | 0.00% | 0.00% | -0.05% | 99% | 99% |
| Class D | -0.01% | 0.15% | -0.03% | 99% | 102% | 0.01% | 0.22% | 0.17% | 99% | 99% |
| Class F | 0.01% | 0.00% | 0.07% | 99% | 98% | -0.01% | -0.03% | -0.03% | 99% | 97% |

# Conclusion

This contribution presents two simplifications for MIP mode to reduce the complexity. It is reported that the proposed method obtains 0.01% luma BD-rate change in AI configurations on top of VTM-5.0 respectively with similar encoding or decoding run-time. It is proposed to adopt this proposal.

# References

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, and T. Wiegand “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0217, Geneva, CH, 19–27 March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).