| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0172 |
| --- | --- |

| Title: | On rectangular slice address signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee 117, Bundangnaegok-ro,Bundang-gu, Seongnam-si, Gyeonggi-do | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr |
| Source: | Xris Corporation |  |  |

_____________________________

- Abstract

This contribution proposed to modify some slice address information such as top_left_idx[i].

Following is proposed

- Proposal 1: It is proposed to replace syntax element top_left_brick_idx[i] into top_left_brick_idx_minus1[i]

- Proposal 2: It is proposed to replace syntax element top_left_brick_idx[i] into top_left_brick_delta[i] that is difference between top_left_brick_idx[i] and top_left_brick_idx[i-1]

- Proposal 3: It is propsed to replace syntax element top_left_brick_idx[i] into top_left_brick_delta_minus1[i] that is minus1 from top_left_brick_delta[i] of proposal 2 in this contribution

Introduction

VVC WD Draft5 [1] includes signalling for brick, tile and slices. Information of rectangular slices is mainly signaled in PPS. This contribution proposes three modification of slice address signaling

Proposal

Proposal 1

It is asserted that top_left_brick_idx[i] is signalled for i-th slice where i is not equal to 0, if rectangular slice mode is used. Therefore, top_left_brick_idx[i] cannot be equal to 0.

This contribution proposes to replace top_left_brick_idx[i] into top_left_brick_idx_minus1[i] as shown highlighted below in the following Table 1

      - Picture parameter set RBSP syntax

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_brick_idx_minus1[ i ] | u(v) |
| bottom_right_brick_idx[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

      - Picture parameter set RBSP semantics

…

top_left_brick_idx_minus1 [ i ] +1 specifies the brick index of the brick located at the top-left corner of the i-th slice .When not present, the value of top_left_tile_idx_delta[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic 1)) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx_minus1[ i ] + 1 + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-36)
					BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

Proposal 2

It is proposed to specify the difference between brick index of the brick located at the top-left corner in i-th slice and brick index of the brick located at the top-left corner in (i-1)th slice

It is asserted that since brick index of the brick located at the top-left corner in i-th slice is greater than or equal to brick index of the brick located at the top-left corner in (i-1)-th slice, coding in this manner is always possible and that it saves bits.

It is proposed to change top_left_brick_idx[i] into top_left_brick_idx_delta[i] and he proposed changes are shown highlighted below in the following Table 2.

      - Picture parameter set RBSP syntax

Table 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_brick_idx_delta[ i ] | u(v) |
| bottom_right_brick_idx[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

      - Picture parameter set RBSP semantics

…

top_left_brick_idx_delta[ i ] specifies the difference between the brick index of brick located at the top-left corner of i-th slice and the brick index of brick located at the top-left corner of (i-1)th slice When not present, the value of top_left_tile_idx_delta[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic – TopLeftBrickIdx(i-1))) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
TopLeftBrickIdx(0) = 0

TopLeftBrickIdx(i) = TopLeftBrickIdx(i-1) + top_left_brick_Idx_delta[i]

botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]

botRightBkIdx(i) = TopLeftBrickIdx(i) + bottom_right_brick_idx_delta[i]

for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ TopLeftBrickIdx(i)]  &&
					BrickColBd[ j ]  <=  BrickColBd[ TopLeftBrickIdx(i) ]  &&
					BrickRowBd[ j ]  >=  BrickRowlBd[ TopLeftBrickIdx(i) ]  &&	
					BrickRowBd[ j ] <= BrickColBd[ botRightBkIdx(i) ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

Proposal 3

It is proposed to specify the difference between the brick index of the brick located at the top-left corner in i-th slice and the brick index of the brick located at the top-left corner in (i-1)th slice

It is asserted that since brick index of the brick located at the top-left corner in i-th slice is greater than brick index of the brick located at the top-left corner in (i-1)-th slice. Therefore, it is proposed to change top_left_brick_Idx[i] into top_left_birck_idx_delta_minus1[i] and the proposed changes are shown highlighted below in the following Table 3

      - Picture parameter set RBSP syntax

Table 3

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_brick_idx_delta_minus1[ i ] | u(v) |
| bottom_right_brick_idx[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

      - Picture parameter set RBSP semantics

…

top_left_brick_idx_delta_minus1[ i ] +1 specifies the difference between the brick index of brick located at the top-left corner in i-th slice and the brick index brick located at the top-left corner in (i-1)th slice When not present, the value of top_left_tile_idx_delta_minus1[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic – TopLeftBrickIdx(i-1)-1)) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
TopLeftBrickIdx(0) = 0

TopLeftBrickIdx(i) = TopLeftBrickIdx(i-1) + top_left_brick_Idx_delta_minus1[i] + 1

botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]

botRightBkIdx(i) = TopLeftBrickIdx(i) + bottom_right_brick_idx_delta[i]

for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ TopLeftBrickIdx(i)]  &&
					BrickColBd[ j ]  <=  BrickColBd[ TopLeftBrickIdx(i) ]  &&
					BrickRowBd[ j ]  >=  BrickRowlBd[ TopLeftBrickIdx(i) ]  &&	
					BrickRowBd[ j ] <= BrickColBd[ botRightBkIdx(i) ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, May 2019

Patent rights declaration(s)

XRis Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).