| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0173 |
| --- | --- |

| Title: | On brick information signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee 117, Bundangnaegok-ro,Bundang-gu, Seongnam-si, Gyeonggi-do, Republic of Korea | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr |
| Source: | Xris Corporation |  |  |

_____________________________

- Abstract

VVC WD Draft 5 [1] includes signaling for brick, tile and slices in PPS.

This contribution proposed to modify some brick information such as num_brick_row_minus1 and brick splitting present information for each tile column in the PPS.

Following is proposed

- Proposal 1: It is proposed to replace syntax element num_brick_rows_minus1 into num_brick_rows_minus2

- Proposal 2: It is proposed to signal brick_column_split_present_flag[i] in PPS which specifies that at least one tile in any i-th tile column split into two or more bricks

Introduction

This contribution proposes two modification of brick information signaling in the PPS.

One is simple modification for signaling of number of brick row.

Another one is adding one more syntax element which is brick splitting information for each tile column. The motivation is described as below

Brick splitting information for each tile column

Tiles are always in tiles rows and tile columns. Tile column runs through from the top of a picture to the bottom of the picture.

There is scenario [2] that would benefit from a signalling of brick splitting present information for each tile column rather than signalling of brick splitting information for every tile.

The first such scenario is the one that is documented in subclause D.6.2 of [3]

The scheme is illustrated with Figure 1, and explained in the following:

- Encoding: The content is encoded at two spatial resolutions, such as 5120×2560 and 2560×1280, and with 4×2 tile grid. An MCTS is coded for each tile position.

- Encapsulation of coded data: Each coded MCTS sequence is stored as a sub-picture track or a tile track.

- Viewport-adaptive MCTS selection: Each range of viewing orientations that causes a different selection of high- and low-resolution MCTSs is considered. Four MCTSs of the high-resolution bitstream are selected the same way as in the example above. Moreover, the four low-resolution MCTSs that are not overlapping with the selected high-resolution MCTSs are also selected.

- Creation of extractor tracks: An extractor track is created for each distinct viewport-adaptive MCTS selection. In each sample of the extractor track, one extractor is created for each MCTS, extracting data from the tile or sub-picture track that contains the selected high- or low-resolution MCTSs. Each tile encapsulating content from the low-resolution bitstream contains two slices. RegionWisePackingBox is included in the extractor track to create a mapping between the packed picture and a projected picture of the ERP format. In the presented example, the bitstreams resolved from the extractor tracks have resolution 3200×2560. Consequently, a 4K-capable decoder could decode content where the viewport is extracted from a coded bitstream with 5K (5120×2560) resolution.

Figure 1 – Example of extractor track combining MCTSs from bitstreams of different resolution

In this use case, brick is not used for some tile columns and brick is used for some tile columns as shown by the right hand side of Figure 1,

Proposal

Proposal 1

If brick_split_flag is equal to 1, a tile is composed of two or more bricks. That means number of minimum brick is 2 in a tile when brick_split_flag is equal to 1. So, it is proposed to replace syntax element num_brick_row_minus1 into num_brick_row_minus2 as following Table 1

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus2 [ i ]+1; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

7.4.3.4	Picture parameter set RBSP semantics

num_brick_rows_minus12[ i ] plus 12 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1[ i ] shall be in the range of 12 to RowHeight[ i ] − 1, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred as specified in 6.5.1.

Proposal 2

It is proposed to signal brick splitting information for each tile column. The proposed methods would give more bit saving for some use case such as above mentioned.

It is proposed to signal brick_column_split_present_flag[ i ] which specifies that at least one tile in the i-th tile column is splitted into two or more bricks.

For example, any tile in 0th tile column and 1st tile column, 2nd tile column is not splitted into two or more bricks in Figure 2, so brick_column_split_present_flag[0], brick_column_split_present_flag[1] and brick_column_split_present_flag[2] are equal to 0. However, at least one tile in 3rd tile column is splitted into two or more bricks, so brick_column_split_present_flag[3] is equal to 1.

Figure 2

The following Table 2 and following sentences shows proposed syntax and semantics change in PPS.

Table 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( j = 0; brick_splitting_present_flag && j <= num_tile_columns_minus1; j++ ) { |  |
| brick_column_split_present_flag[ j ] | u(1) |
| } |  |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| if (brick_column_split_present_flag[ i % (num_tile_columns_minus1+1)]) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … | u(1) |
| } |  |

7.4.3.4 Picture Parameter set RBSP semantics

brick_column_split_present_flag[ i ] equal to 1 specifies that at least one tile in the i-th tile column is splitted into two or more bricks. brick_column_split_present_flag[i] equal to 0 specifies that any tile in the i-th tile column is not splitted into two or more bricks.

Proposal 3

It is proposed to include conditionally signaling of syntax elements brick_column_split_present_flag which are proposed in proposal 2.

If brick_column_split_present_flag[i] is equal to 0 for each i (i < num_tile_columns_minus1), then brick_column_split_present_flag[num_tile_columns_minus1] is not signaled and it value of brick_column_present_flag[num_tile_columns_minus1] is inferred to 1.

The following Table 3 and following sentences shows proposed syntax and semantics change in PPS.

Table 3

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| isSplitpresnt = 0 |  |
| for(j = 0; brick_splitting_present_flag && j <= num_tile_columns_minus1; j++ ) { |  |
| if (!(isSplitPresent == 0 && j == num_tile_columns_minus1)) |  |
| brick_column_split_present_flag[ j ] | u(1) |
| if (brick_column_split_present_flag[ j ]) |  |
| isSplitPresent ++ ; |  |
| } |  |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| if (brick_column_split_present_flag[ i % (num_tile_columns_minus1+1)]) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … | u(1) |
| } |  |

Analysis of bit saving

The following Table 4 shows bit saving comparison of tile and brick signalling in VVC Draft5, O0173 and O1075 V2. Analysis of bit saving in JVET-O1075 v2 [4] is reused in the Table 4

Table 4

|  | Effective 6K ERP |  | 6K effective CMP 3×6 tile grid |  | Effective 6K CMP 6×3 tile grid |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # bits | #bits saving | # bits | #bits saving | # bits | #bits saving |
| VVC Draft 5 + O0236 | 74 |  | 63 |  | 84 |  |
| O0173 Prop #2 | 72 | 2.7% | 54 | 15% | 78 | 7% |
| O0173 Prop #3 | 72 | 2.7% | 53 | 17% | 78 | 7% |
| JVET-O1075 v2 | 39 | 47 % | 34 | 56 % | 35 | 58 % |

| a) 6K effective ERP scheme of OMAF clause D.6.3. 6×2 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and tile row heights of 2048 and 256 luma samples | b) 6K effective CMP scheme of OMAF clause D.6.4. 3×6 tile grid with tile column widths of 768, 768, and 384 luma samples and all tile row heights equal to 768 luma samples. | c) 6K effective CMP scheme. 6×3 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and all tile row heights equal to 768 luma samples. |
| --- | --- | --- |

Figure 3. VVC tile and brick partitioning use case in

References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, May 2019

[2] JVET-N0111, AHG12: Flexible tiling, Geneva, May 2019

[3] N18393, WD5 of ISO/IEC 23090-2 OMAF 2nd edition

[4] JVET-1072 v2

Patent rights declaration(s)

XRis Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).