| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0176 |
| --- | --- |

| Title: | On Tiles, Bricks and Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modification are proposed to bricks signalling and slice ID signalling

Following is proposed:

- Proposal 1: It is proposed to signal the signalled_slice_id_length_minus1 syntax element with an offset.

- Proposal 2: It is proposed to signal number of brick rows in the case of non-uniform bricks with a minus2 coding or with a minus3 coding. Additionally range of valid values is defined for brick_row_height_minus1[ i ][ j ] syntax element.

- Proposal 3: It is proposed to move the signalling location of single_brick_per_slice_flag and rect_slice_flag before signalling of brick related syntax elements and to conditionally signal the brick related syntax elements based on rect_slice_flag.

- Proposal 4: It is proposed to relax the restriction regarding the order of slice_address values when the slice_id values are explicitly signalled in PPS (i.e. when signalled_slice_id_flag is equal to 1).

- Proposal 5: It is proposed to signal slice_pic_order_cnt_lsb before the conditional signaling of recovery_poc_cnt.

- Proposal 6: It is proposed to signal if a rectangular slice is a motion constrained tile set.

# Introduction

VVC WD [1] includes signalling for tiles, bricks and slices.

We propose modifications to brick signaling and to the explicit slice ID signalling.

Following is proposed:

- Proposal 1: It is proposed to signal the signalled_slice_id_length_minus1 syntax element with an offset.

- Proposal 2: It is proposed to signal number of brick rows in the case of non-uniform bricks with a minus2 coding or with a minus3 coding. Additionally range of valid values is defined for brick_row_height_minus1[ i ][ j ] syntax element.

- Proposal 3: It is proposed to move the signalling location of single_brick_per_slice_flag and rect_slice_flag before signalling of brick related syntax elements and to conditionally signal the brick related syntax elements based on rect_slice_flag.

- Proposal 4: It is proposed to relax the restriction regarding the order of slice_address values when the slice_id values are explicitly signalled in PPS (i.e. when signalled_slice_id_flag is equal to 1).

- Proposal 5: It is proposed to signal slice_pic_order_cnt_lsb before the conditional signaling of recovery_poc_cnt.

- Proposal 6: It is proposed to signal if a rectangular slice is a motion constrained tile set.

# Proposals

## Proposal 1

It is proposed to signal the signalled_slice_id_length_minus1 syntax element with an offset. This saves bits.

In VVC WD it is a requirement of bitstream conformance that:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

Additionally, in the rectangular slice mode following specification text defines behavior of slice_address and in turn the behavior of signalled slice_id values:

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

Because of the above bitstream conformance requirement, each slice in the rectangular mode needs to have unique slice_id value that is used as slice_address. As a result when explicitly signalling slice_id values in PPS at least num_slices_in_pic_minus1 + 1  values must be signalled. As a result it is proposed to code the signalled_slice_id_length_minus1 instead with signalled_slice_id_length_minus1_offset.

The proposed changes are shown highlighted below compared to VVC WD:

signalled_slice_id_length_minus1_offset plus (Ceil( Log2( num_slices_in_pic_minus1 + 1 ))) specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 to 15-((Ceil( Log2( num_slices_in_pic_minus1 + 1 )))), inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) − 10.

Alternative Proposal 1:

Alternatively, it is proposed to restrict the range of values signalled in signalled_slice_id_length_minus1 syntax element

signalled_slice_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) – 1 to 15, inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) − 1.

## Proposal 2

It is proposed to signal number of brick rows in the case of non-uniform bricks with a minus2 coding or with a minus3 coding. This saves bits.

Two alternative options are proposed. One with minus 2 coding and another with minus 3 coding.

Additionally range of valid values is defined for brick_row_height_minus1[ i ][ j ] syntax element. This is currently missing.

### Proposal 1 - Option 1: Minus2 coding

It is proposed to signal number of brick rows in the case of non-uniform bricks with a minus 2 coding.

In VVC the syntax element num_brick_rows_minus1[ i ] is only signalled when a tile is further split into more than one brick. As a result it is signalled only when brick_split_flag[ i ] is equal to 1. Since this means that there are two or more bricks in the tile, it is asserted that it is better to code this syntax element with a minus 2 coding. As this saves bits for this ue(v) coded syntax element.

The proposed modifications for this change are shown highlighted below comapred with VVC WD5.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus1minus2 [ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Picture parameter set RBSP semantics

pps_pic_parameter_set_id identifies the PPS for reference by other syntax elements. The value of pps_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

pps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the active SPS. The value of pps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated slice headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated slice headers.

single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS.

NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick.

It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are activated within a CVS.

uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1.

tile_cols_width_minus1 plus 1 specifies the width of the tile columns excluding the right-most tile column of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_cols_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_cols_width_minus1 is inferred to be equal to PicWidthInCtbsY – 1.

tile_rows_height_minus1 plus 1 specifies the height of the tile rows excluding the bottom tile row of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_width_minus1 is inferred to be equal to PicHeightInCtbsY – 1.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_columns_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred as specified in clause 6.5.1.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred as specified in clause 6.5.1.

The variable NumTilesInPic is set equal to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ).

When single_tile_in_pic_flag is equal to 0, NumTilesInPic shall be greater than 1.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs.

brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks.

brick_split_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more bricks. brick_split_flag[ i ] equal to 0 specifies that the i-th tile is not divided into two or more bricks. When not present, the value of brick_split_flag[ i ] is inferred to be equal to 0.

uniform_brick_spacing_flag[ i ] equal to 1 specifies that brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1minus2[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1.

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] – 1.

num_brick_rows_minus1minus2[ i ] plus 12 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1minus2[ i ] shall be in the range of 1 0 to RowHeight[ i ] − 12, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1minus2 [ i ] is inferred to be equal to 0 -1. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus2[ i ] is inferred as specified in 6.5.1.

brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0. brick_row_height_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ i ] – 2, inclusive.

The following variables are derived, and, when uniform_tile_spacing_flag is equal to 1, the values of num_tile_columns_minus1 and num_tile_rows_minus1 are inferred, and, for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus2[ i ] is inferred, by invoking the CTB raster and brick scanning conversion process as specified in clause 6.5.1:

–	the list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs,

–	the list CtbAddrRsToBs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the brick scan,

–	the list CtbAddrBsToRs[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the brick scan to a CTB address in the CTB raster scan of a picture,

–	the list BrickId[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in brick scan to a brick ID,

–	the list NumCtusInBrick[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick index to the number of CTUs in the brick,

–	the list FirstCtbAddrBs[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick ID to the CTB address in brick scan of the first CTB in the brick.

single_brick_per_slice_flag equal to 1 specifies that each slice that refers to this PPS includes one brick. single_brick_per_slice_flag equal to 0 specifies that a slice that refers to this PPS may include more than one brick. When not present, the value of single_brick_per_slice_flag is inferred to be equal to 1.

rect_slice_flag equal to 0 specifies that bricks within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that bricks within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When single_brick_per_slice_flag is equal to 1 rect_slice_flag is inferred to be equal to 1.

num_slices_in_pic_minus1 plus 1 specifies the number of slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_brick_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumBricksInPic − 1.

top_left_brick_idx[ i ] specifies the brick index of the brick located at the top-left corner of the i-th slice. The value of top_left_brick_idx[ i ] shall not be equal to the value of top_left_brick_idx[ j ] for any i not equal to j. When not present, the value of top_left_brick_idx[ i ] is inferred to be equal to i. The length of the top_left_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) bits.

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a subset of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowlBd[ top_left_brick_idx[ i ] ]  &&	(7-34)
					BrickRowBd[ j ] <= BrickColBd[ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

loop_filter_across_bricks_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across brick boundaries in pictures referring to the PPS. loop_filter_across_bricks_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across brick boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_bricks_enabled_flag is inferred to be equal to 1.

loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across slice boundaries in pictures referring to the PPS. loop_filter_across_slice_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across slice boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_slices_enabled_flag is inferred to be equal to 0.

signalled_slice_id_flag equal to 1 specifies that the slice ID for each slice is signalled. signalled_slice_id_flag equal to 0 specifies that slice IDs are not signalled. When rect_slice_flag is equal to 0, the value of signalled_slice_id_flag is inferred to be equal to 0.

signalled_slice_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) − 1.

slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits.  When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive.

## Scanning processes

### CTB raster scanning, tile scanning, and brick scanning process

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag ) {
	remainingWidthInCtbsY = PicWidthInCtbsY
	i = 0
	while( remainingWidthInCtbsY > ( tile_cols_width_minus1 + 1 ) ) {
		colWidth[ i++ ] = tile_cols_width_minus1 + 1
		remainingWidthInCtbsY  −=  ( tile_cols_width_minus1 + 1 )
	}
	colWidth[ i ] = remainingWidthInCtbsY
	num_tile_columns_minus1 = i
} else {
	colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		colWidth[ i ] = tile_column_width_minus1[ i ] + 1
		colWidth[ num_tile_columns_minus1 ]  −=  colWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag ) {
	remainingHeightInCtbsY = PicHeightInCtbsY
	i = 0
	while( remainingHeightInCtbsY > ( tile_rows_height_minus1 + 1 ) ) {
		RowHeight[ i++ ] = tile_rows_height_minus1 + 1
		remainingHeightInCtbsY  −=  ( tile_rows_height_minus1 + 1 )
	}
	RowHeight[ i ] = remainingHeightInCtbsY
	num_tile_rows_minus1 = i
} else {
	RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
		RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

The list tileColBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, is derived as follows:

for( tileColBd[ 0 ] = 0, i = 0; i  <=  num_tile_columns_minus1; i++ )
		tileColBd[ i + 1 ] = tileColBd[ i ] + colWidth[ i ]	(6-3)

The list tileRowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs, is derived as follows:

for( tileRowBd[ 0 ] = 0, j = 0; j  <=  num_tile_rows_minus1; j++ )
	tileRowBd[ j + 1 ] = tileRowBd[ j ] + RowHeight[ j ]	(6-4)

The variable NumBricksInPic, specifying the number of bricks in a picture referring to the PPS, and the lists BrickColBd[ i ], BrickRowBd[ i ], BrickWidth[ i ], and BrickHeight[ i ] for i ranging from 0 to NumBricksInPic − 1, inclusive, specifying the locations of the brick column boundaries in units of CTBs, the locations of the brick boundaries in units of CTBs, the widths of the brick columns in units of CTBs, and the heights of brick columns in units of CTBs, are derived, and for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus2[ i ] is inferred, as follows:

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )
	if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]	(6-5)
		brickIdx++
	} else {
		if( uniform_brick_spacing_flag[ i ] ) {
			remainingHeightInCtbsY = RowHeight[ tileY ]
			j = 0
			while( remainingHeightInCtbsY > ( brick_height_minus1[ i ] + 1 ) ) {
				rowHeight2[ j++ ] = brick_height_minus1[ i ] + 1
				remainingHeightInCtbsY  −=  ( brick_height_minus1[ i ] + 1 )
			}
			rowHeight2[ j ] = remainingHeightInCtbsY
			num_brick_rows_minus1minus2 [ i ] = j − 1
		} else {
			rowHeight2[ num_brick_rows_minus1minus2[ i ] +1] = RowHeight[ tileY ]
			for( j = 0; j <= num_brick_rows_minus1minus2[ i ]; j++ ) {
				rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
				rowHeight2[ num_brick_rows_minus1minus2[ i ] +1 ]  −=  rowHeight2[ j ]
			}
		}
		for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus1minus2[ i ]+1; j++ )
			rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
		for( j = 0; j <= num_brick_rows_ minus1minus2[ i ] + 1; j++ ) {
			BrickColBd[ brickIdx ] = tileColBd[ tileX ]
			BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
			BrickWidth[ brickIdx ] = colWidth[ tileX ]
			BrickHeight[ brickIdx ] = rowHeight2[ j ]
			brickIdx++
		}
	}
}
NumBricksInPic = brickIdx

The list CtbAddrRsToBs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in brick scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	tbX = ctbAddrRs % PicWidthInCtbsY
	tbY = ctbAddrRs / PicWidthInCtbsY
	brickFound = FALSE
	for( bkIdx = NumBricksInPic − 1, i = 0; i < NumBricksInPic − 1  &&  !brickFound; i++ ) {
		brickFound = tbX  <  ( BrickColBd[ i ] + BrickWidth[ i ] )  &&
					tbY  <  ( BrickRowBd[ i ] + BrickHeight[ i ] )
		if( brickFound )		(6-6)
			bkIdx = i
	}
	CtbAddrRsToBs[ ctbAddrRs ] = 0
	for( i = 0; i < bkIdx; i++ )
		CtbAddrRsToBs[ ctbAddrRs ]  +=  BrickHeight[ i ] * BrickWidth[ i ]
	CtbAddrRsToBs[ ctbAddrRs ]  +=
				( tbY − BrickRowBd[ bkIdx ] ) * BrickWidth[ bkIdx ] + tbX − BrickColBd[ bkIdx ]
}

The list CtbAddrBsToRs[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in brick scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ )	(6-7)
	CtbAddrBsToRs[ CtbAddrRsToBs[ ctbAddrRs ] ] = ctbAddrRs

The list BrickId[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in brick scan to a brick ID, is derived as follows:

for( i = 0, i < NumBricksInPic; i++ )
	for( y = BrickRowBd[ i ]; y < BrickRowBd[ i ] + BrickWidth[ i ]; y++ ) 	(6-8)
		for( x = BrickColBd[ i ]; x < BrickColBd[ i ] + BrickHeight[ i ]; x++ )
			BrickId[ CtbAddrRsToBs[ y * PicWidthInCtbsY+ x ] ] = i

The list NumCtusInBrick[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick index to the number of CTUs in the brick, is derived as follows:

for( i = 0; i < NumBricksInPic; i++ ) 		(6-9)
	NumCtusInBrick[ i ] = BrickWidth[ i ] * BrickHeight[ i ]

The list FirstCtbAddrBs[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick ID to the CTB address in brick scan of the first CTB in the brick are derived as follows:

for( ctbAddrBs = 0, brickIdx = 0, brickStartFlag = 1; ctbAddrBs < PicSizeInCtbsY; ctbAddrBs++ ) {
	if( brickStartFlag ) {
		FirstCtbAddrBs[ brickIdx ] = ctbAddrBs		(6-10)
		brickStartFlag = 0
	}
	brickEndFlag = ( ctbAddrBs  = =  PicSizeInCtbsY − 1 )  | |
					( BrickId[ ctbAddrBs + 1 ]  !=  BrickId[ ctbAddrBs ] )
	if( brickEndFlag ) {
		brickIdx++
		brickStartFlag = 1
	}
}

### Proposal 2 - Option 2: Minus 3 coding

Alternatively, it is proposed to signal number of brick rows in the case of non-uniform bricks with a minus3 coding.

It is asserted that if there are two bricks in an i-th tile, it is more efficient to code this information using uniform_brick_spacing_flag[ i ] equal to 1 which takes one bit followed by one syntax element brick_height_minus1[ i ] compared to coding this with a uniform_brick_spacing_flag[ i ] equal to 0 followed by num_brick_rows_minus1[ i ] equal to 1 (which needs 3 bits) followed by brick_row_height_minus1[ i ][ 0 ] which needs same number of bits as brick_height_minus1[ i ]. Thus in this case of two bricks or brick rows in the tile, setting uniform_brick_spacing_flag[ i ] equal to 1 saves 3 bits. Thus it is proposed to not allow signalling the two bricks or brick rows in a tile case using  uniform_brick_spacing_flag[ i ] equal to 0, thus the else part of the loop is proposed to be only used for signalling a case when there are more than 2 bricks or brick rows  in a tile. Thus in this case the syntax element indicating number of bricks or brick rows in the tile can be coded with a minus 3 coding. This minus 3 coding also further saves bits – for example when there are three or more bricks or brick rows in the tile and they are coded with non-uniform brick spacing.

The proposed modifications for this change are shown highlighted below comapred with VVC WD5.

| brick_splitting_present_flag | u(1) |
| --- | --- |
| for( i = 0; brick_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1minus3[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus1minus3 [ i ]+1; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

uniform_brick_spacing_flag[ i ] equal to 1 specifies that brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1minus3[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1.

num_brick_rows_minus1minus3[ i ] plus 13 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1minus3[ i ] shall be in the range of 1 0 to RowHeight[ i ] − 13, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1minus3[ i ] is inferred to be equal to 0 -2. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus3[ i ] is inferred as specified in 6.5.1.

brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0. brick_row_height_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ i ] – 3, inclusive.

The following variables are derived, and, when uniform_tile_spacing_flag is equal to 1, the values of num_tile_columns_minus1 and num_tile_rows_minus1 are inferred, and, for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus3[ i ] is inferred, by invoking the CTB raster and brick scanning conversion process as specified in clause 6.5.1:

–	the list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs,

–	the list CtbAddrRsToBs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the brick scan,

–	the list CtbAddrBsToRs[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the brick scan to a CTB address in the CTB raster scan of a picture,

–	the list BrickId[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in brick scan to a brick ID,

–	the list NumCtusInBrick[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick index to the number of CTUs in the brick,

–	the list FirstCtbAddrBs[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick ID to the CTB address in brick scan of the first CTB in the brick.

## 6.5 Scanning processes

### 6.5.1 CTB raster scanning, tile scanning, and brick scanning process

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag ) {
	remainingWidthInCtbsY = PicWidthInCtbsY
	i = 0
	while( remainingWidthInCtbsY > ( tile_cols_width_minus1 + 1 ) ) {
		colWidth[ i++ ] = tile_cols_width_minus1 + 1
		remainingWidthInCtbsY  −=  ( tile_cols_width_minus1 + 1 )
	}
	colWidth[ i ] = remainingWidthInCtbsY
	num_tile_columns_minus1 = i
} else {
	colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		colWidth[ i ] = tile_column_width_minus1[ i ] + 1
		colWidth[ num_tile_columns_minus1 ]  −=  colWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag ) {
	remainingHeightInCtbsY = PicHeightInCtbsY
	i = 0
	while( remainingHeightInCtbsY > ( tile_rows_height_minus1 + 1 ) ) {
		RowHeight[ i++ ] = tile_rows_height_minus1 + 1
		remainingHeightInCtbsY  −=  ( tile_rows_height_minus1 + 1 )
	}
	RowHeight[ i ] = remainingHeightInCtbsY
	num_tile_rows_minus1 = i
} else {
	RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
		RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

The list tileColBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, is derived as follows:

for( tileColBd[ 0 ] = 0, i = 0; i  <=  num_tile_columns_minus1; i++ )
		tileColBd[ i + 1 ] = tileColBd[ i ] + colWidth[ i ]	(6-3)

The list tileRowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs, is derived as follows:

for( tileRowBd[ 0 ] = 0, j = 0; j  <=  num_tile_rows_minus1; j++ )
	tileRowBd[ j + 1 ] = tileRowBd[ j ] + RowHeight[ j ]	(6-4)

The variable NumBricksInPic, specifying the number of bricks in a picture referring to the PPS, and the lists BrickColBd[ i ], BrickRowBd[ i ], BrickWidth[ i ], and BrickHeight[ i ] for i ranging from 0 to NumBricksInPic − 1, inclusive, specifying the locations of the brick column boundaries in units of CTBs, the locations of the brick boundaries in units of CTBs, the widths of the brick columns in units of CTBs, and the heights of brick columns in units of CTBs, are derived, and for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1minus3[ i ] is inferred, as follows:

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )
	if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]	(6-5)
		brickIdx++
	} else {
		if( uniform_brick_spacing_flag[ i ] ) {
			remainingHeightInCtbsY = RowHeight[ tileY ]
			j = 0
			while( remainingHeightInCtbsY > ( brick_height_minus1[ i ] + 1 ) ) {
				rowHeight2[ j++ ] = brick_height_minus1[ i ] + 1
				remainingHeightInCtbsY  −=  ( brick_height_minus1[ i ] + 1 )
			}
			rowHeight2[ j ] = remainingHeightInCtbsY
			num_brick_rows_minus1minus3 [ i ] = j − 2
		} else {
			rowHeight2[ num_brick_rows_minus1minus3[ i ] +2] = RowHeight[ tileY ]
			for( j = 0; j <= num_brick_rows_minus1minus3[ i ]+1; j++ ) {
				rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
				rowHeight2[ num_brick_rows_minus1minus3[ i ] +2 ]  −=  rowHeight2[ j ]
			}
		}
		for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus1minus3[ i ]+2; j++ )
			rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
		for( j = 0; j <= num_brick_rows_minus1minus3[ i ] + 12; j++ ) {
			BrickColBd[ brickIdx ] = tileColBd[ tileX ]
			BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
			BrickWidth[ brickIdx ] = colWidth[ tileX ]
			BrickHeight[ brickIdx ] = rowHeight2[ j ]
			brickIdx++
		}
	}
}
NumBricksInPic = brickIdx

## Proposal 3

It is proposed to move the signalling location of single_brick_per_slice_flag and when signalled rect_slice_flag before signalling of brick splitting related syntax elements and to conditionally signal the brick related syntax elements based on rect_slice_flag. This ensures that the information about brick splitting related syntax elements is not signalled for raster scan slices. The appropriate information is inferred in this case. This is more efficient.

Alternatively, a bitstream conformance constraint is proposed which requires brick related syntax elements to not be signalled for raster scan slices.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if(rect_slice_flag) { |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j <num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### Picture parameter set RBSP semantics

…

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs.

single_brick_per_slice_flag equal to 1 specifies that each slice that refers to this PPS includes one brick. single_brick_per_slice_flag equal to 0 specifies that a slice that refers to this PPS may include more than one brick. When not present, the value of single_brick_per_slice_flag is inferred to be equal to 1.

rect_slice_flag equal to 0 specifies that bricks within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that bricks within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When single_brick_per_slice_flag is equal to 1 rect_slice_flag is inferred to be equal to 1.

brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks. When not present brick_splitting_present_flag is inferred to be equal to 0.

brick_split_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more bricks. brick_split_flag[ i ] equal to 0 specifies that the i-th tile is not divided into two or more bricks. When not present, the value of brick_split_flag[ i ] is inferred to be equal to 0.

uniform_brick_spacing_flag[ i ] equal to 1 specifies that brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1.

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] – 1.

num_brick_rows_minus1[ i ] plus 1 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1[ i ] shall be in the range of 1 to RowHeight[ i ] − 1, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred as specified in 6.5.1.

brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0.

The following variables are derived, and, when uniform_tile_spacing_flag is equal to 1, the values of num_tile_columns_minus1 and num_tile_rows_minus1 are inferred, and, for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred, by invoking the CTB raster and brick scanning conversion process as specified in clause 6.5.1:

–	the list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs,

–	the list CtbAddrRsToBs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the brick scan,

–	the list CtbAddrBsToRs[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the brick scan to a CTB address in the CTB raster scan of a picture,

–	the list BrickId[ ctbAddrBs ] for ctbAddrBs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in brick scan to a brick ID,

–	the list NumCtusInBrick[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick index to the number of CTUs in the brick,

–	the list FirstCtbAddrBs[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the conversion from a brick ID to the CTB address in brick scan of the first CTB in the brick.

single_brick_per_slice_flag equal to 1 specifies that each slice that refers to this PPS includes one brick. single_brick_per_slice_flag equal to 0 specifies that a slice that refers to this PPS may include more than one brick. When not present, the value of single_brick_per_slice_flag is inferred to be equal to 1.

rect_slice_flag equal to 0 specifies that bricks within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that bricks within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When single_brick_per_slice_flag is equal to 1 rect_slice_flag is inferred to be equal to 1.

num_slices_in_pic_minus1 plus 1 specifies the number of slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_brick_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumBricksInPic − 1.

### …

### Alternative proposal 3:

Alternatively, a bitstream conformance constraint is proposed which requires brick related syntax elements to not be signalled for raster scan slices.

brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks. When rect_slice_flag is equal to 0, brick_splitting_present_flag shall be equal to 0.

## Proposal 4

In VVC two modes of slices are supported, namely the raster-scan slice mode and the rectangular slice mode. In the raster-scan slice mode, a slice contains a sequence of tiles in a tile raster scan of a picture. In the rectangular slice mode, a slice contains a number of bricks of a picture that collectively form a rectangular region of the picture. In rectangular slice mode an explicit signalling of slice_id values is supported. In this case the entire map of slices in the picture and the explicitly signalled slice_id values which are used as slice_address are known in the PPS. Ability to explicitly signal slice ID values in this manner is useful for bitstream merging use cases including those for 360 applications.

To fully enable those use cases, it is proposed to relax the restriction regarding the order of slice_address values when the slice_id values are explicitly signalled in PPS (i.e. when signalled_slice_id_flag is equal to 1).

The proposed changes are shown highlighted below compared to VVC WD:

### 7.4.6 Slice header semantics

#### 7.4.6.1 General slice header semantics

When present, the value of each of the slice header syntax elements slice_pic_parameter_set_id, slice_pic_order_cnt_lsb, and slice_temporal_mvp_enabled_flag shall be the same in all slice headers of a coded picture.

slice_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of slice_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

It is a requirement of bitstream conformance that the value of TemporalId of the current picture shall be greater than or equal to the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation 6-8.

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- If signalled_slice_id_flag is equal to 1, Tthe slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each brick, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded brick(s).

## Proposal 5

It is proposed to signal slice_pic_order_cnt_lsb before the conditional signaling of recovery_poc_cnt.

It is asserted that the Picture order count information is important to be known from a slice header as early as possible and it would be better to be able to parse this information without having to parse recovery_poc_cnt information which is conditionally signalled only for certain NAL unit type.

Thus it is proposed to signal slice_pic_order_cnt_lsb syntax element before the conditional signaling of recover_poc_cnt as well as signaling of slice_type syntax element.

The proposed changes are shown highlighted below compared to VVC WD:

### Slice header syntax

### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_pic_order_cnt_lsb | u(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( ( NalUnitType != IDR_W_RADL && NalUnitType != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |

## Proposal 6

It is proposed to signal if a rectangular slice is a motion constrained tile set. It is asserted that knowing this information can be helpful for 360 video bitstream rewriting scenarios as it allows knowing if rectangular slices from different representations can be easily rewritten together to create a merged bitstream.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag) { |  |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| is_mcts_flag[ i ] | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

is_mcts_flag[ i ] equal to 1 specifies that i-th slice is a motion constrained tile set. is_mcts_flag[ i ]  equal to 0 specifies that i-th slice is not a motion constrained tile set.

# Conclusion

This document proposes modifications to brick and slice signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019