| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0178-r1 |
| --- | --- |

| Title: | On DPB Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to DPB parameters signalling.

Proposal 1:

- It is proposed to code sps_max_dec_pic_buffering_minus1[ i ] with a delta coding.

- It is proposed to code sps_max_num_reorder_pics[ i ] with a delta coding.

Proposal 2:

- It is proposed to conditionally signal sps_sub_layer_ordering_info_present_flag based on the value of sps_max_sub_layers_minus1.

# Introduction

In proposal1: It is proposed to code the sps_max_dec_pic_buffering[ i ]and sps_max_num_reorder_pics[ i ]syntax elements with a delta coding. Each of these parameters are coded in VVC WD5  [1] using ue(v) coding and there is a strictly monotonic non-decreasing relationship between values for the parameters as a function of index i.

Thus it is the case that when i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. And when i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ].

It is asserted that in this case it is more natural to code the syntax elements sps_max_dec_pic_buffering[i] and sps_max_num_reorder_pics[ i ] with delta coding compared to the values for sps_max_dec_pic_buffering[ i − 1] and sps_max_num_reorder_pics[ i − 1] as it saves bits.

In proposal 2: It is proposed to conditionally signal sps_sub_layer_ordering_info_present_flag based on the value of sps_max_sub_layers_minus1.

# Proposals

Proposal 1:

It is proposed to code sps_max_dec_pic_buffering_minus1[ i ] with a delta coding.

It is proposed to code sps_max_num_reorder_pics[ i ] with a delta coding.

The proposed changes for proposal 1 are shown highlighted compared with VVC WD.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_deltaminus1[ i ] | ue(v) |
| sps_max_num_reorder_pics_delta[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| long_term_ref_pics_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| … |  |
| } |  |

#### Sequence parameter set RBSP semantics

…

sps_sub_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers. sps_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ], and sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ] apply to all sub-layers.

sps_max_dec_pic_buffering_deltaminus1[ i ] plus 1 is used to compute the value of SpsMaxDecPicBuffering[ i ]  which specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of SpsMaxDecPicBufferingminus1[ i ] shall be in the range of 0 to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified somewhere else. [Ed. (YK): Change to reference to the clause specifying MaxDpbSize when available.] When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. When sps_max_dec_pic_buffering_ deltaminus1 [ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ]0.

The variable SpsMaxDecPicBuffering[ i ] is calculated as follows:

SpsMaxDecPicBuffering[ i ] = sps_max_dec_pic_buffering_delta[ i ] + (i==0)? 1: SpsMaxDecPicBuffering[ i−1 ]

sps_max_num_reorder_pics_delta[ i ] is used to compute the value of SpsMaxNumReorderPics[ i ] which specifies the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i. The value of SpsMaxNumReorderPics [ i ] shall be in the range of 0 to (SpsMaxDecPicBuffering[ i ]-1) sps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. When sps_max_num_reorder_pics_delta[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ]0.

The variable SpsMaxNumReorderPics [ i ] is calculated as follows:

SpsMaxNumReorderPics [ i ] = sps_max_num_reorder_pics_delta [ i ] + (i==0)? 0: SpsMaxNumReorderPics [ i − 1 ]

sps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of SpsMaxLatencyPictures[ i ], which specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i.

When sps_max_latency_increase_plus1[ i ] is not equal to 0, the value of SpsMaxLatencyPictures[ i ] is specified as follows:

SpsMaxLatencyPictures[ i ] = sps_max_num_reorder_pics SpsMaxNumReorderPics[ i ] + sps_max_latency_increase_plus1[ i ] − 1	(7-8)

When sps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of sps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ].

Also in the specification, for proposal 1:

- Replace all occurrence of sps_max_dec_pic_buffering_minus1[ i ] with SpsMaxDecPicBuffering[ i ] − 1

- Replace all occurrence of (sps_max_dec_pic_buffering_minus1[ i ]+1) with SpsMaxDecPicBuffering[ i ]

- Replace all occurrence of sps_max_num_reorder_pics[ i ] with SpsMaxNumReorderPics [ i ]

Proposal 2:

It is proposed to conditionally signal sps_sub_layer_ordering_info_present_flag based on the value of sps_max_sub_layers_minus1.

The proposed changes for proposal 2 are shown highlighted compared with VVC WD.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if(!sps_max_sub_layers_minus1) |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics [ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| long_term_ref_pics_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| … |  |
| } |  |

#### Sequence parameter set RBSP semantics

…

sps_sub_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers. sps_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ], and sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ] apply to all sub-layers. When not present sps_sub_layer_ordering_info_present_flag is inferred to be equal to 0.

# Proposal 1 and Proposal 2 can work together with no additional change.

# Conclusion

This document proposes modifications to DPB parameters signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019