| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O180 |
| --- | --- |

| Title: | On Buffering Period |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to buffering period signalling.

Following is proposed:

- Proposal 1:

  - It is proposed to signal initial CPB removal delay values for different highest temporal sub-layers supported by the bitstream.

  - Operation of coded picture buffer is modified to use the appropriate initial buffering delay parameters based on the HighestTid.

- Proposal 2: It is proposed to keep track of the BitRate and CpbSize values for each temporal sub-layer representation separately and use the appropriate variable values during the operation.

# Introduction

VVC WD [1] includes a design of HRD including a buffering period SEI message.

The buffering period SEI message signals initial CPB removal delay for NAL HRD and VCL HRD. This is accomplished by syntax elements nal_initial_cpb_removal_delay[ i ], nal_initial_cpb_removal_offset[ i ] for NAL HRD and vcl_initial_cpb_removal_delay[ i ], vcl_initial_cpb_removal_offset[ i ] for VCL HRD. A number (signalled via syntax bp_cpb_cnt_minus1) of these NAL and VCL HRD initial CPB removal delays are signalled for different HRD schedules and it is allowed to select the appropriate initial CPB removal delay parameters based on the selected HRD schedule index (SchedSelIdx). However one buffering period SEI message only signals initial CPB removal delay parameters for a particular HighestTid value. Since the bitstream may operate at different highest TemporalID values and since initial CPB removal delay values are different for sub-bitstream with different number of temporal sub-layers, it is proposed to allow signaling of initial CPB removal delays for all supported different HighestTid values for the bitstream in the buffering period SEI message. Then the appropriate selection of initial CPB removal delay parameters can be done based on the selected HRD schedule and selected highest number of temporal sub-layers. It is asserted that this allows low overhead parameter signaling for initial CPB removal delay parameters.

# Proposals

## Proposal 1

Following is proposed:

- It is proposed to signal initial CPB removal delay values for different highest temporal sub-layers supported by the bitstream.

- Operation of coded picture buffer is modified to use the appropriate initial buffering delay parameters based on the HighestTid.

The proposed changes are shown highlighted compared with VVC WD.

D2.2 Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_seq_parameter_set_id | ue(v) |
| bp_nal_hrd_parameters_present_flag | u(1) |
| bp_vcl_hrd_parameters_present_flag | u(1) |
| if( bp_nal_hrd_parameters_present_flag ¦ ¦ bp_ vcl_hrd_parameters_present_flag ) { |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| } |  |
| concatenation_flag | u(1) |
| cpb_removal_delay_delta_minus1 | u(v) |
| cpb_removal_delay_deltas_present_flag | u(1) |
| if( cpb_removal_delay_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| for( j = 0; j <= sps_max_sub_layers_minus1; j++ ) { |  |
| bp_cpb_cnt_minus1[ j ] | ue(v) |
| if( bp_nal_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1[ j ] + 1; i++ ) { |  |
| nal_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| nal_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| if( bp_vcl_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1[ j ] + 1; i++ ) { |  |
| vcl_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| vcl_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

D3.2 Buffering period SEI message semantics

A buffering period SEI message provides initial CPB removal delay and initial CPB removal delay offset information for initialization of the HRD at the position of the associated access unit in decoding order.

When the buffering period SEI message is present, a picture is said to be a notDiscardablePic picture when the picture has TemporalId equal to 0 and is not a RASL or RADL picture.

When the current picture is not the first picture in the bitstream in decoding order, let prevNonDiscardablePic be the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL or RADL picture.

The presence of buffering period SEI messages is specified as follows:

–	If NalHrdBpPresentFlag is equal to 1 or VclHrdBpPresentFlag is equal to 1, the following applies for each access unit in the CVS:

- If the access unit is an IRAP access unit, a buffering period SEI message applicable to the operation point shall be associated with the access unit.

- Otherwise, if the access unit contains a notDiscardablePic, a buffering period SEI message applicable to the operation point may or may not be associated with the access unit.

- Otherwise, the access unit shall not be associated with a buffering period SEI message applicable to the operation point.

–	Otherwise (NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0), no access unit in the CVS shall be associated with a buffering period SEI message.

NOTE 1 – For some applications, frequent presence of buffering period SEI messages may be desirable (e.g., for random access at an IRAP picture or a non-IRAP picture or for bitstream splicing).

bp_seq_parameter_set_id indicates and shall be equal to the sps_seq_parameter_set_id for the SPS that is active for the coded picture associated with the buffering period SEI message. The value of bp_seq_parameter_set_id shall be equal to the value of pps_seq_parameter_set_id in the PPS referenced by the slice_pic_parameter_set_id of the slice headers of the coded picture associated with the buffering period SEI message. The value of bp_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

bp_nal_hrd_parameters_present_flag equal to 1 specifies that a list of syntax element pairs nal_initial_cpb_removal_delay[ i ][ j ]  and nal_initial_cpb_removal_offset[ i ][ j ]   are present in the buffering period SEI message. bp_nal_hrd_parameters_present_flag equal to 0 specifies that no syntax element pairs nal_initial_cpb_removal_delay[ i ][ j ]   and nal_initial_cpb_removal_offset[ i ][ j ]   are present in the buffering period SEI message.

It is a requirement of bitstream conformance that the value of bp_nal_hrd_parameters_present_flag in the buffering period SEI message assocaited with an access unit is equal to the value of vui_nal_hrd_parameters_present_flag in the VUI parameters of the active SPS.

bp_vcl_hrd_parameters_present_flag equal to 1 specifies that a list of syntax element pairs vcl_initial_cpb_removal_delay[ i ][ j ]   and vcl_initial_cpb_removal_offset[ i ][ j ]   are present in the buffering period SEI message. bp_vcl_hrd_parameters_present_flag equal to 0 specifies that no syntax element pairs vcl_initial_cpb_removal_delay[ i ][ j ]   and vcl_initial_cpb_removal_offset[ i ][ j ]  are present in the buffering period SEI message.

It is a requirement of bitstream conformance that the value of bp_vcl_hrd_parameters_present_flag in the buffering period SEI message associated with an access unit is equal to the value of vui_vcl_hrd_parameters_present_flag in the VUI parameters of the active SPS.

initial_cpb_removal_delay_length_minus1 plus 1 specifies the length, in bits, of the syntax elements nal_initial_cpb_removal_delay[ i ][ j ]  , nal_initial_cpb_removal_offset[ i ][ j ]  , vcl_initial_cpb_removal_delay[ i ][ j ]  , and vcl_initial_cpb_removal_offset[ i ][ j ]  of the buffering period SEI message. When not present, the value of initial_cpb_removal_delay_length_minus1 is inferred to be equal to 23.

cpb_removal_delay_length_minus1 plus 1 specifies the length, in bits, of the syntax elements cpb_removal_delay_delta_minus1 and cpb_removal_delay_delta[ i ] in the buffering period SEI message and the syntax element cpb_removal_delay_minus1[ i ] in the picture timing SEI message. When not present, the value of cpb_removal_delay_length_minus1 is inferred to be equal to 23.

dpb_output_delay_length_minus1 plus 1 specifies the length, in bits, of the syntax element dpb_output_delay in the picture timing SEI message. When not present, the value of dpb_output_delay_length_minus1 is inferred to be equal to 23.

concatenation_flag indicates, when the current picture is not the first picture in the bitstream in decoding order, whether the nominal CPB removal time of the current picture is determined relative to the nominal CPB removal time of the preceding picture with a buffering period SEI message or relative to the nominal CPB removal time of the picture prevNonDiscardablePic.

cpb_removal_delay_delta_minus1 plus 1, when the current picture is not the first picture in the bitstream in decoding order, specifies a CPB removal delay increment value relative to the nominal CPB removal time of the picture prevNonDiscardablePic. The lenght of this syntax element is cpb_removal_delay_length_minus1 + 1 bits.

When the current picture contains a buffering period SEI message and concatenation_flag is equal to 0 and the current picture is not the first picture in the bitstream in decoding order, it is a requirement of bitstream conformance that the following constraint applies:

- If the picture prevNonDiscardablePic is not associated with a buffering period SEI message, the cpb_removal_delay_minus1 of the current picture shall be equal to the cpb_removal_delay_minus1 of prevNonDiscardablePic plus cpb_removal_delay_delta_minus1 + 1.

- Otherwise, cpb_removal_delay_minus1 shall be equal to cpb_removal_delay_delta_minus1.

NOTE 2 – When the current picture contains a buffering period SEI message and concatenation_flag is equal to 1, the cpb_removal_delay_minus1 for the current picture is not used. The above-specified constraint can, under some circumstances, make it possible to splice bitstreams (that use suitably-designed referencing structures) by simply changing the value of concatenation_flag from 0 to 1 in the buffering period SEI message for an IRAP picture at the splicing point. When concatenation_flag is equal to 0, the above-specified constraint enables the decoder to check whether the constraint is satisfied as a way to detect the loss of the picture prevNonDiscardablePic.

cpb_removal_delay_deltas_present_flag equal to 1 specifies that the buffering period SEI message contains CPB removal delay deltas. cpb_removal_delay_deltas_present_flag equal to 0 specifies that no CPB removal delay deltas are present in the buffering period SEI message.

num_cpb_removal_delay_deltas_minus1 plus 1 specifies the number of syntax elements cpb_removal_delay_delta[ i ] in the buffering period SEI message. The value of num_cpb_removal_offsets_minus1 shall be in the range of 0 to 15, inclusive.

cpb_removal_delay_delta[ i ] specifies the i-th CPB removal delay delta. The lenght of this syntax element is cpb_removal_delay_length_minus1 + 1 bits.

bp_cpb_cnt_minus1[ j ]  plus 1 specifies the number of syntax element pairs nal_initial_cpb_removal_delay[ i ][ j ] and nal_initial_cpb_removal_offset[ i ][ j ]  for the j-th temporal sub-layer representation when bp_nal_hrd_parameters_present_flag is equal to 1, and the number of syntax element pairs vcl_initial_cpb_removal_delay[ i ][ j ]  and vcl_initial_cpb_removal_offset[ i ][ j ] for the j-th temporal sub-layer representation when bp_vcl_hrd_parameters_present_flag is equal to 1. The value of bp_cpb_cnt_minus1[ j ]  shall be in the range of 0 to 31, inclusive.

nal_initial_cpb_removal_delay[ i ][ j ]  specify the i-th initial CPB removal delay for the NAL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of nal_initial_cpb_removal_delay[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of nal_initial_cpb_removal_delay[ i ][ j ]  shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ]  BitRate[ i ] ), the time-equivalent of the CPB size in 90 kHz clock units, where BitRate[ i ], CpbSize[ i ] are obtained respectively from equation (E-66) and (E-67) by setting HighestTid value equal to j.

nal_initial_cpb_removal_offset[ i ][ j ]  specify the i-th initial CPB removal offset for the NAL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of nal_initial_cpb_removal_offset[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of nal_initial_cpb_removal_delay[ i ][ j ]  and nal_initial_cpb_removal_offset[ i ][ j ] shall be constant for each value of i and for each value of j.

vcl_initial_cpb_removal_delay[ i ][ j ]  specify the i-th initial CPB removal delay for the VCL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of vcl_initial_cpb_removal_delay[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of vcl_initial_cpb_removal_delay[ i ][ j ]  shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ]  BitRate[ i ] ), the time-equivalent of the CPB size in 90 kHz clock units, where BitRate[ i ], CpbSize[ i ] are obtained respectively from equation (E-66) and (E-67) by setting HighestTid value equal to j.

vcl_initial_cpb_removal_offset[ i ][ j ]  specify the i-th initial CPB removal offset for the VCL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of vcl_initial_cpb_removal_offset[ i ][ j ]   is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of vcl_initial_cpb_removal_delay[ i ][ j ]  and vcl_initial_cpb_removal_offset[ i ][ j ] shall be constant for each value of i and for each value of j.

C.2 Operation of coded picture buffer

C.2.1 General

The specifications in this clause apply independently to each set of coded picture buffer (CPB) parameters that is present and to both the Type I and Type II conformance points shown in Figure C.1 and the set of CPB parameters is selected as specified in clause C.1.

C.2.2 Timing of decoding unit arrival

The process specified in the remainder of this clause is invoked for derivation of the initial and final CPB arrival times for access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ][ HighestTid ] and nal_initial_cpb_removal_offset[ SchedSelIdx ][ HighestTid ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ][ HighestTid ]  and vcl_initial_cpb_removal_offset[ SchedSelIdx ][ HighestTid ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

The time at which the first bit of access unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

The initial arrival time of access unit m is derived as follows:

–	If the access unit is access unit 0 (i.e., when m is equal to 0), initArrivalTime[ 0 ] is set equal to 0.

–	Otherwise (the decoding unit is access unit m with m > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit m is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

initArrivalTime[ m ] = FinalArrivalTime[ m − 1 ]	(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit m is derived as follows:

initArrivalTime[ m ] = Max( FinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(C-3)

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

tmpNominalRemovalTime = NominalRemovalTime[ m ]	(C-4)

where NominalRemovalTime[ m ] is the nominal CPB removal time of access unit m, as specified in clause C.2.3.

–	If access unit m is not the first access unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] ) ÷ 90000	(C-5)

–	Otherwise (access unit m is the first access unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 )	(C-6)

The final arrival time for access unit m is derived as follows:

FinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷ BitRate[ SchedSelIdx ]	(C-7)

where sizeInbits[ m ] is the size in bits of access unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C.1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the selected hrd_parameters( ) syntax structures for the access unit containing access unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the selected hrd_parameters( ) syntax structures for the access unit containing access unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing access unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the current access unit.

C.2.3 Timing of access unit removal and decoding of access unit

The variables InitCpbRemovalDelay[ SchedSelIdx ], and InitCpbRemovalDelayOffset[ SchedSelIdx ], are derived as follows:

–	InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ][ HighestTid ]  and nal_initial_cpb_removal_offset[ SchedSelIdx ][ HighestTid ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ][ HighestTid ]  and vcl_initial_cpb_removal_offset[ SchedSelIdx ][ HighestTid ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause C.1.

The nominal removal time of the access unit n from the CPB is specified as follows:

–	If access unit n is the access unit with n equal to 0 (the access unit that initializes the HRD), the nominal removal time of the access unit from the CPB is specified by:

NominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000	(C-8)

–	Otherwise, the following applies:

–	When access unit n is the first access unit of a buffering period that does not initialize the HRD, the following applies:

The nominal removal time of the access unit n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = NominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = CpbRemovalDelayVal
} else {
	baseTime = NominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay =
		Max( ( CpbRemovalDelayDeltaMinus1 + 1 ),	(C-9)
			Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
				FinalArrivalTime[ n − 1 ] − NominalRemovalTime[ n − 1 ] ) ÷ ClockTick ) )
}
NominalRemovalTime[ n ] = baseTime + ClockTick * tmpCpbRemovalDelay

where NominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first access unit of the previous buffering period, NominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL or RADL picture, CpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ] and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n and concatenationFlag and CpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and cpb_removal_delay_delta_minus1, respectively, in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

NominalRemovalTime[ n ] = NominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * CpbRemovalDelayVal	(C-10)

where NominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period and CpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ] and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message, selected as specified in clause C.1, associated with access unit n.

The removal time of access unit n from the CPB is specified as follows, where FinalArrivalTime[ n ] and NominalRemovalTime[ n ] are the final CPB arrival time and nominal CPB removal time, respectively, of access unit n:

if( !low_delay_hrd_flag[ HighestTid ]  | |  NominalRemovalTime[ n ]  >=  FinalArrivalTime[ n ] )
	CpbRemovalTime[ n ] = NominalRemovalTime[ n ]
else				(C-11)
	CpbRemovalTime[ n ] = NominalRemovalTime[ n ] + ClockTick *
		Ceil( ( FinalArrivalTime[ n ] − NominalRemovalTime[ n ] ) ÷ ClockTick )

NOTE 1 – When low_delay_hrd_flag[ HighestTid ] is equal to 1 and NominalRemovalTime[ n ] is less than FinalArrivalTime[ n ], the size of access unit n is so large that it prevents removal at the nominal removal time.

At the CPB removal time of access unit n, the access unit is instantaneously decoded.

## Proposal 2

It is proposed to keep track of the BitRate and CpbSize values for each temporal sub-layer representation separately and use the appropriate variable values during the operation. Currently without this change there is some unclarity when BitRate and CpbSize values are used for different HighestTid values. It is asserted that the proposed changes below makes the specification text unambiguous.

The proposed changes are shown highlighted compared with VVC WD.

E.2.3 Sub-layer HRD parameters syntax

| sub_layer_hrd_parameters( subLayerId ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= CpbCnt; i++ ) { |  |
| bit_rate_value_minus1[ i ][ subLayerId ] | ue(v) |
| cpb_size_value_minus1[ i ][ subLayerId ] | ue(v) |
| cbr_flag[ i ][ subLayerId ] | u(1) |
| } |  |
| } |  |

E.3.2. HRD parameters semantics

bit_rate_scale (together with bit_rate_value_minus1[ i ][ subLayerId ]) specifies the maximum input bit rate of the i-th CPB.

cpb_size_scale (together with cpb_size_value_minus1[ i ][ subLayerId ]) specifies the CPB size of the i-th CPB when the CPB operates at the access unit level.

E.3.3. Sub-layer HRD parameters semantics

The variable CpbCnt is set equal to vui_cpb_cnt_minus1[ subLayerId ].

bit_rate_value_minus1[ i ][ subLayerId ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB with HighestTid equal to subLayerId when the CPB operates at the access unit level. bit_rate_value_minus1[ i ] [ subLayerId ]  shall be in the range of 0 to 232 − 2, inclusive. For any i > 0, bit_rate_value_minus1[ i ][ subLayerId ]  shall be greater than bit_rate_value_minus1[ i − 1 ][ subLayerId ].

The bit rate in bits per second is given by:

BitRate[ i ][ subLayerId ]  = ( bit_rate_value_minus1[ i ][ subLayerId ] + 1 ) * 2( 6 + bit_rate_scale )	(E-66)

When the bit_rate_value_minus1[ i ][ subLayerId ]  syntax element is not present, the value of BitRate[ i ][ subLayerId ]  is inferred to be equal to CpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, CpbBrVclFactor and CpbBrNalFactor are specified in subclause A.4.]

cpb_size_value_minus1[ i ][ subLayerId ]  is used together with cpb_size_scale to specify the i-th CPB size with HighestTid equal to subLayerId  when the CPB operates at the access unit level. cpb_size_value_minus1[ i ][ subLayerId ]  shall be in the range of 0 to 232 − 2, inclusive. For any i greater than 0, cpb_size_value_minus1[ i ][ subLayerId ]  shall be less than or equal to cpb_size_value_minus1[ i − 1 ][ subLayerId ].

The CPB size in bits is given by:

CpbSize[ i ][ subLayerId ]  = ( cpb_size_value_minus1[ i ][ subLayerId ] + 1 ) * 2( 4 + cpb_size_scale )	(E-67)

When the cpb_size_value_minus1[ i ][ subLayerId ]  syntax element is not present, the value of CpbSize[ i ][ subLayerId ] is inferred to be equal to CpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, CpbBrVclFactor and CpbBrNalFactor are specified in subclause A.4.]

cbr_flag[ i ][ subLayerId ]  equal to 0 specifies that to decode this bitstream by the HRD using the i-th CPB specification with HighestTid equal to subLayerId , the hypothetical stream scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ i ][ subLayerId ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode with HighestTid equal to subLayerId. When not present, the value of cbr_flag[ i ][ subLayerId ]  is inferred to be equal to 0.

D.3.2 Buffering period SEI message semantics

<When used without proposal 1>

bp_cpb_cnt_minus1 plus 1 specifies the number of syntax element pairs nal_initial_cpb_removal_delay[ i ] and nal_initial_cpb_removal_offset[ i ] when bp_nal_hrd_parameters_present_flag is equal to 1, and the number of syntax element pairs vcl_initial_cpb_removal_delay[ i ] and vcl_initial_cpb_removal_offset[ i ] when bp_vcl_hrd_parameters_present_flag is equal to 1. The value of bp_cpb_cnt_minus1 shall be in the range of 0 to 31, inclusive.

nal_initial_cpb_removal_delay[ i ] specify the i-th initial CPB removal delay for the NAL HRD in units of a 90 kHz clock. The length of nal_initial_cpb_removal_delay[ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of nal_initial_cpb_removal_delay[ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ][ HighestTid]   BitRate[ i ][ HighestTid]  ), the time-equivalent of the CPB size in 90 kHz clock units. [Ed. (YK): Add a clarification of CpbSize[ i ]  BitRate[ i ] at the beginning of this clause.]

nal_initial_cpb_removal_offset[ i ] specify the i-th initial CPB removal offset for the NAL HRD in units of a 90 kHz clock. The length of nal_initial_cpb_removal_offset[ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of nal_initial_cpb_removal_delay[ i ] and nal_initial_cpb_removal_offset[ i ] shall be constant for each value of i.

vcl_initial_cpb_removal_delay[ i ] specify the i-th initial CPB removal delay for the VCL HRD in units of a 90 kHz clock. The length of vcl_initial_cpb_removal_delay[ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of vcl_initial_cpb_removal_delay[ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ][ HighestTid]    BitRate[ i ][ HighestTid]   ), the time-equivalent of the CPB size in 90 kHz clock units.

vcl_initial_cpb_removal_offset[ i ] specify the i-th initial CPB removal offset for the VCL HRD in units of a 90 kHz clock. The length of vcl_initial_cpb_removal_offset[ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of vcl_initial_cpb_removal_delay[ i ] and vcl_initial_cpb_removal_offset[ i ] shall be constant for each value of i.

… <When used with proposal 1>

nal_initial_cpb_removal_delay[ i ][ j ]  specify the i-th initial CPB removal delay for the NAL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of nal_initial_cpb_removal_delay[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of nal_initial_cpb_removal_delay[ i ][ j ]  shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ][ j ]    BitRate[ i ][ j ] ), the time-equivalent of the CPB size in 90 kHz clock units. [Ed. (YK): Add a clarification of CpbSize[ i ]  BitRate[ i ] at the beginning of this clause.]

nal_initial_cpb_removal_offset[ i ][ j ]  specify the i-th initial CPB removal offset for the NAL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of nal_initial_cpb_removal_offset[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of nal_initial_cpb_removal_delay[ i ][ j ]  and nal_initial_cpb_removal_offset[ i ][ j ] shall be constant for each value of i and for each value of j.

vcl_initial_cpb_removal_delay[ i ][ j ]  specify the i-th initial CPB removal delay for the VCL HRD in units of a 90 kHz clock for the j-th temporal sub-layer representation. The length of vcl_initial_cpb_removal_delay[ i ][ j ]  is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of vcl_initial_cpb_removal_delay[ i ][ j ]  shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ][ j ]  BitRate[ i ][ j ] ), the time-equivalent of the CPB size in 90 kHz clock units.

  - Operation of coded picture buffer

    - General

The specifications in this clause apply independently to each set of coded picture buffer (CPB) parameters that is present and to both the Type I and Type II conformance points shown in Figure C.1 and the set of CPB parameters is selected as specified in clause C.1.

    - Timing of decoding unit arrival

The process specified in the remainder of this clause is invoked for derivation of the initial and final CPB arrival times for access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

The time at which the first bit of access unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

The initial arrival time of access unit m is derived as follows:

–	If the access unit is access unit 0 (i.e., when m is equal to 0), initArrivalTime[ 0 ] is set equal to 0.

–	Otherwise (the decoding unit is access unit m with m > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit m is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

initArrivalTime[ m ] = FinalArrivalTime[ m − 1 ]	(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit m is derived as follows:

initArrivalTime[ m ] = Max( FinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(C-3)

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

tmpNominalRemovalTime = NominalRemovalTime[ m ]	(C-4)

where NominalRemovalTime[ m ] is the nominal CPB removal time of access unit m, as specified in clause C.2.3.

–	If access unit m is not the first access unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] ) ÷ 90000	(C-5)

–	Otherwise (access unit m is the first access unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 )	(C-6)

The final arrival time for access unit m is derived as follows:

FinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷ BitRate[ SchedSelIdx ][ HighestTid ]  	(C-7)

where sizeInbits[ m ] is the size in bits of access unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C.1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ][ HighestTid ]   and CpbSize[ SchedSelIdx ][ HighestTid ]    are constrained as follows:

–	If the content of the selected hrd_parameters( ) syntax structures for the access unit containing access unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the selected hrd_parameters( ) syntax structures for the access unit containing access unit m that results in a BitRate[ SchedSelIdx1 ][ HighestTid ] or CpbSize[ SchedSelIdx1 ][ HighestTid ] for the access unit containing access unit m. The value of BitRate[ SchedSelIdx1 ][ HighestTid ] or CpbSize[ SchedSelIdx1 ][ HighestTid ] may differ from the value of BitRate[ SchedSelIdx0 ][ HighestTid ] or CpbSize[ SchedSelIdx0 ][ HighestTid ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ][ HighestTid ] and CpbSize[ SchedSelIdx ][ HighestTid ].

When the HSS selects values of BitRate[ SchedSelIdx ][ HighestTid ] or CpbSize[ SchedSelIdx ][ HighestTid ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ][ HighestTid ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ][ HighestTid ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ][ HighestTid ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ][ HighestTid ] comes into effect at the CPB removal time of the current access unit.

C.5 Decoder conformance

C.5.1 General

A decoder conforming to this Specification shall fulfil all requirements specified in this clause.

A decoder claiming conformance to a specific profile, tier and level shall be able to successfully decode all bitstreams that conform to the bitstream conformance requirements specified in clause C.4, in the manner specified in Annex A, provided that all VPSs, SPSs, PPSs and APSs referred to in the VCL NAL units and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified in this Specification.

When a bitstream contains syntax elements that have values that are specified as reserved and it is specified that decoders shall ignore values of the syntax elements or NAL units containing the syntax elements having the reserved values, and the bitstream is otherwise conforming to this Specification, a conforming decoder shall decode the bitstream in the same manner as it would decode a conforming bitstream and shall ignore the syntax elements or the NAL units containing the syntax elements having the reserved values as specified.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile, tier and level, as specified in clause C.4 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All cropped decoded pictures output by the HRD shall also be output by the DUT, each cropped decoded picture output by the DUT shall be a picture with PictureOutputFlag equal to 1, and, for each such cropped decoded picture output by the DUT, the values of all samples that are output shall be equal to the values of the samples produced by the specified decoding process.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex A for the specified profile, tier and level or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex A. The same delivery schedule is used for both the HRD and the DUT.

When the HRD parameters and the buffering period SEI messages are present with vui_cpb_cnt_minus1[ HighestTid ] greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ) and initial CPB removal delay ( f( r ) ÷ r ) as follows:

α = ( r − BitRate[ SchedSelIdx − 1 ][ HighestTid ] ) ÷ ( BitRate[ SchedSelIdx ][ HighestTid ]  − BitRate[ SchedSelIdx − 1 ][ HighestTid ] ),	(C-17)

c( r ) = α * CpbSize[ SchedSelIdx ][ HighestTid ]  + ( 1 − α ) * CpbSize[ SchedSelIdx − 1 ][ HighestTid ],	(C-18)

f( r ) = α * InitCpbRemovalDelay[ SchedSelIdx ] * BitRate[ SchedSelIdx ][ HighestTid ]  + 
		( 1 − α ) * InitCpbRemovalDelay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ][ HighestTid ]	(C-19)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ][ HighestTid ]   <=  r  <=  BitRate[ SchedSelIdx ][ HighestTid ]  such that r and c( r ) are within the limits as specified in Annex A for the maximum bit rate and buffer size for the specified profile, tier and level.

NOTE 1 – InitCpbRemovalDelay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both the HRD and the DUT up to a fixed delay.

For output order decoder conformance, the following applies:

–	The HSS delivers the bitstream BitstreamToDecode to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest decoding unit.

–	A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream BitstreamToDecode such that the bit rate and CPB size are restricted as specified in Annex A. The order of pictures output shall be the same for both the HRD and the DUT.

–	The HRD CPB size is given by CpbSize[ SchedSelIdx ] as specified in clause E.3.3, where SchedSelIdx and the HRD parameters are selected as specified in clause C.1. The DPB size is given by sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1. Removal time from the CPB for the HRD is the final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is as described in clauses C.5.2 through C.5.2.3.

C.1 General

…

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure C.1, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( nal_hrd_parameters_present_flag )" is selected), the test is conducted at the Type II conformance point shown in Figure C.1, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 2 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C.1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C.1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ][ HighestTid ]   and CpbSize[ SchedSelIdx ][ HighestTid ]   for the variable bit rate (VBR) case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive.

# Conclusion

This document proposes modifications to buffering period initial delay parameters signaling in VVC. Additionally it is proposed to keep track of the BitRate and CpbSize values for each temporal sub-layer representation separately and use the appropriate values during the operation. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019