| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0181 |
| --- | --- |

| Title: | Comments on High-Level Syntax of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes comments on high-level syntax of VVC WD.

Following is proposed:

- Proposal 1: Parameter sets activation/ referencing specification text is proposed

- Proposal2: It is proposed to signal information regarding sub-layer reference/ non-reference properties for a picture.

- Proposal 3: A bug-fix is proposed for semantics of num_bricks_in_slice_minus1.

- Proposal 4: A semantics restriction is proposed on the DPS referred to by SPS, for each CVS in the bitstream.

# Introduction

This document proposes comments on high-level syntax of VVC WD [1].  The following is proposed:

- Proposal 1: Parameter sets activation/ referencing specification text is proposed

- Proposal 2: It is proposed to signal information regarding sub-layer reference/ non-reference properties for a picture.

- Proposal 3: A bug-fix is proposed for semantics of num_bricks_in_slice_minus1.

- Proposal 4: A semantics restriction is proposed on the DPS referred to by SPS, for each CVS in the bitstream.

# Proposals

## Proposal 1

Currently VVC WD has missing text for parameter set activation. Specification text is proposed for this. It is asserted that even if a formal activation process is not needed and instead parameter sets are simply referred, several of the aspects in the text below, in particular those related to 1. various referred parameter sets having the same content, 2. availability of parameter sets before they are activated/ referred and 3. how many parameter sets can be referred by a layer are necessary to be specified.

The proposed specification text is shown highlighted below (compared to HEVC specification text).

##### Order of DPS, VPS, SPS, PPS and APS RBSPs and their activation

TBD.

This clause specifies the activation process of video parameter sets (VPSs), sequence parameter sets (SPSs) and PPSs.

NOTE 1 – The VPS, SPS and PPS mechanism decouples the transmission of infrequently changing information from the transmission of coded block data. VPSs, SPSs and PPSs may, in some applications, be conveyed "out-of-band".

A PPS RBSP includes parameters that can be referred to by the coded slice segment NAL units of one or more coded pictures. Each PPS RBSP is initially considered not active for the base layer at the start of the operation of the decoding process. At most one PPS RBSP is considered active for the base layer at any given moment during the operation of the decoding process, and the activation of any particular PPS RBSP for the base layer results in the deactivation of the previously-active PPS RBSP for the base layer (if any).

When a PPS RBSP (with a particular value of pps_pic_parameter_set_id) is not active for the base layer and it is referred to by a coded slice segment NAL unit with nuh_layer_id equal to 0 the nuh_layer_id of the coded slice NAL unit which referrs it (using a value of slice_pic_parameter_set_id equal to the pps_pic_parameter_set_id value), it is activated for the base layer. This PPS RBSP is called the active PPS RBSP for the base layer until it is deactivated by the activation of another PPS RBSP for the base layer. A PPS RBSP, with that particular value of pps_pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means, and the PPS NAL unit containing the PPS RBSP shall have nuh_layer_id equal to 0 the nuh_layer_id of the coded slice NAL unit which refers it.

Any PPS NAL unit containing the value of pps_pic_parameter_set_id for the active PPS RBSP for a coded picture (and consequently for the layer containing the coded picture) shall have the same content as that of the active PPS RBSP for the coded picture, unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

A LMCS APS RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures. Each LMCS APS RBSP is initially considered not active for the layer at the start of the operation of the decoding process. If slice_lmcs_enabled_flag is equal to 1, at most one LMCS APS RBSP is considered active for the layer at any given moment during the operation of the decoding process, and the activation of any particular LMCS APS RBSP for the layer results in the deactivation of the previously active LMCS APS RBSP for the layer (if any).

When a LMCS APS RBSP  is not active for the layer and it is referred to by a coded slice NAL unit of a coded picture which belongs to the same layer (using a value of slice_lmcs_aps_id equal to the adaptation_parameter_set_id value), it is activated for the layer. This LMCS APS RBSP is called the active LMCS APS RBSP for the layer until it is deactivated by the activation of another LMCS APS RBSP for the layer. A LMCS APS RBSP, with that particular value of adaptation_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the coded slice NAL unit which refers it  or provided through external means, and the LMCS APS NAL unit containing the LMCS APS RBSP shall have nuh_layer_id equal to the nuh_layer_id of the coded slice NAL unit which referrs it.

Any LMCS APS NAL unit containing the value of adaptation_parameter_set_id  for the active LMCS APS RBSP for a coded picture (and consequently for the layer containing the coded picture) shall have the same content as that of the active LMCS APS RBSP for the coded picture, unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

An ALF APS RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures. Each ALF APS RBSP is initially considered not active for the layer at the start of the operation of the decoding process. If slice_alf_enabled_flag is equal to 1, at most (num_alf_aps_ids +1) ALF APS RBSPs are considered active for the layer at any given moment during the operation of the decoding process, and the activation of new (num_alf_aps_ids+1) ALF APS RBSP  for the layer results in the deactivation of all the previously-active ALF APS RBSPs for the layer (if any).

When a ALF APS RBSP is not active for the layer and it is referred to by a coded slice NAL unit (using a value of slice_alf_aps_id_luma[ i ]  or slice_alf_aps_id_chroma equal to the adaptation_parameter_set_id value), it is activated for the layer. Each of the  (num_alf_aps_ids_ +1) ALF APS RBSPs are called the active ALF APS RBSPs for the layer until they are deactivated by the activation of another ALF APS RBSPs for the layer. Each ALF APS RBSP, with a particular value of adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ], for i in the range of 0 to num_alf_aps_ids, inclusive or with a particular value of adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be available to the decoding process prior to their activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the coded slice NAL unit which refers it or provided through external means, and the ALF APS NAL unit containing the ALF APS RBSP shall have nuh_layer_id equal to the nuh_layer_id of the coded slice NAL unit which refers it.

Any ALF APS NAL unit containing the value of adaptation_parameter_set_id  for the active ALF APS RBSP for a coded picture (and consequently for the layer containing the coded picture) shall have the same content as that of the active ALF APS RBSP with the same value for adaptation_parameter_set_id for the coded picture, unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

An SPS RBSP includes parameters that can be referred to by one or more PPS RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each SPS RBSP is initially considered not active for the base layer at the start of the operation of the decoding process. At most one SPS RBSP is considered active for the base layer at any given moment during the operation of the decoding process, and the activation of any particular SPS RBSP for the base layer results in the deactivation of the previously-active SPS RBSP for the base layer (if any).

When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for the base layer and it is referred to by activation of a PPS RBSP (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value) for the base layer or, when vps_base_layer_internal_flag is equal to 1 and vps_base_layer_available_flag is equal to 1, is referred to by an SEI NAL unit containing an active parameter sets SEI message (in which active_seq_parameter_set_id[ 0 ] is equal to the sps_seq_parameter_set_id value), it is activated for the base layer. This SPS RBSP is called the active SPS RBSP for the base layer until it is deactivated by the activation of another SPS RBSP for the base layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the SPS NAL unit containing the SPS RBSP shall have nuh_layer_id equal to 0 the nuh_layer_id of PPS NAL unit or nuh_layer_id of  SEI NAL unit containing an active parameter sets SEI message which refers it. An activated SPS RBSP for the base layer shall remain active for the entire coded video sequence (CVS).

NOTE 2 – Because an IRAP access unit with NoRaslOutputFlagNoIncorrectOutputFlag equal to 1 or a GRA access unit with NoIncorrectOutputFlag equal to 1 begins a new CVS and an activated SPS RBSP for the base layer must remain active for the entire CVS, an SPS RBSP can only be activated for the base layer by an active parameter sets SEI message when the active parameter sets SEI message is part of an IRAP access unit with NoRaslOutputFlag NoIncorrectOutputFlag equal to 1 or a GRA access unit with NoIncorrectOutputFlag equal to 1.

Any SPS NAL unit with nuh_layer_id equal to 0 containing the value of sps_seq_parameter_set_id for the active SPS RBSP for the base layer for a CVS shall have the same content as that of the active SPS RBSP for the base layer for the CVS, unless it follows the last access unit of the CVS and precedes the first VCL NAL unit and the first SEI NAL unit containing an active parameter sets SEI message (when present) of another CVS.

A VPS RBSP includes parameters that can be referred to by one or more SPS RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each VPS RBSP is initially considered not active at the start of the operation of the decoding process. At most one VPS RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular VPS RBSP results in the deactivation of the previously-active VPS RBSP (if any).

When a VPS RBSP (with a particular value of vps_video_parameter_set_id) is not already active and it is referred to by activation of an SPS RBSP (in which sps_video_parameter_set_id is equal to the vps_video_parameter_set_id value) for the base layer, or is referred to by an SEI NAL unit containing an active parameter sets SEI message (in which active_video_parameter_set_id is equal to the vps_video_parameter_set_id value), it is activated. This VPS RBSP is called the active VPS RBSP until it is deactivated by the activation of another VPS RBSP. A VPS RBSP, with that particular value of vps_video_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the VPS NAL unit containing the VPS RBSP shall have nuh_layer_id equal to 0 vps_included_layer_id[ 0 ]. An activated VPS RBSP shall remain active for the entire CVS.

NOTE 3 – Because an IRAP access unit with NoRaslOutputFlagNoIncorrectOutputFlag equal to 1 or a GRA access unit with NoIncorrectOutputFlag equal to 1 begins a new CVS and an activated VPS RBSP must remain active for the entire CVS, a VPS RBSP can only be activated by an active parameter sets SEI message when the active parameter sets SEI message is part of an IRAP access unit with NoRaslOutputFlagNoIncorrectOutputFlag equal to 1 or a GRA access unit with NoIncorrectOutputFlag equal to 1.

Any VPS NAL unit containing the value of vps_video_parameter_set_id for the active VPS RBSP for a CVS shall have the same content as that of the active VPS RBSP for the CVS, unless it follows the last access unit of the CVS and precedes the first VCL NAL unit, and the first SPS NAL unit and the first SEI NAL unit containing an active parameter sets SEI message (when present) of another CVS.

NOTE 4 – If VPS RBSP, SPS RBSP,PPS RBSP, ALF APS RBSP, LMCS APS RBSP, APS RBSP, or DPS RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the VPS RBSP, SPS RBSP, PPS RBSP, ALF APS RBSP, LMCS APS RBSP, APS RBSP, or DPS RBSP respectively. Otherwise (VPS RBSP, SPS RBSP, PPS RBSP, ALF APS RBSP, LMCS APS RBSP, APS RBSP, or DPS RBSP are conveyed by other means not specified in this Specification), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

All constraints that are expressed on the relationship between the values of the syntax elements and the values of variables derived from those syntax elements in VPSs, SPSs, PPSs, ALF APS RBSP, LMCS APS RBSP, APS RBSP, and DPS RBSP and other syntax elements are expressions of constraints that apply only to the active VPS RBSP, the active SPS RBSP for the base layer and the active PPS RBSP for the base layer, the active LMCS APS for the layer, the active ALF APSes for the layer, and the active DPS for the layer. If any VPS RBSP, SPS RBSP,PPS RBSP, ALF APS RBSP, LMCS APS RBSP, APS RBSP, or DPS RBSP is present that is never activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it was activated by reference in an otherwise conforming bitstream.

During operation of the decoding process (see clause 8), the values of parameters of the active VPS RBSP, the active SPS RBSP for the base layer, and the active PPS RBSP for the base layer, the active LMCS APS for the layer (if any) and the active ALF APSes for the layer (if any), the active APSes for the layer (if any) are considered in effect. For interpretation of SEI messages, the values of the active VPS RBSP, the active SPS RBSP for the base layer, and the active PPS RBSP for the base layer, the active LMCS APS for the layer (if any) and the active ALF APSes for the layer (if any)  for the operation of the decoding process for the VCL NAL units of the coded picture with nuh_layer_id equal to 0 in the same access unit are considered in effect unless otherwise specified in the SEI message semantics.

A DPS RBSP includes parameters that can be referred to by one or more SPS RBSPs. Each DPS RBSP is initially considered not active at the start of the operation of the decoding process. At most one DPS RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular DPS RBSP results in the deactivation of the previously active DPS RBSP (if any). When a SPS RBSP is activated and has a sps_decoding_parameter_set_id equal to 0, no DPS is active when decoding each CVS referring to the active SPS. When a DPS RBSP (with a particular value of dps_decoding_parameter_set_id) is not already active and it is referred to by activation of an SPS RBSP in which sps_decoding_parameter_set_id is equal to the dps_decoding_parameter_set_id value and is not equal to 0it is activated. This DPS RBSP is called the active DPS RBSP until it is deactivated by the activation of another DPS RBSP. A DPS RBSP, with that particular value of dps_decoding_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the DPS NAL unit containing the DPS RBSP shall have nuh_layer_id equal to vps_included_layer_id[ 0 ]. An activated DPS RBSP shall remain active for the entire bitstream (and thus the entire CVS).

## Proposal 2

It is proposed to signal information regarding sub-layer reference/ non-reference properties for a picture.

We propose signaling information for following properties for a picture:

- Sub-layer reference/ sub-layer non-reference property

- Global non-reference property

Knowing this information allows pictures to be discarded under certain situations (e.g. playback is lagging real-time). We propose signalling these picture properties information at a fixed location in the slice header. Preferably the picture properties information may be signalled as the first information in the slice header.

The proposed specification text is shown highlighted below compared with VVC WD:

Definitions:

  - non-reference picture: A picture that is not used for inter prediction of any other picture.

3.125 sub-layer non-reference (SLNR) picture: A picture that contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures of the same sub-layer in decoding order.

NOTE – Samples of an SLNR picture may be used for inter prediction in the decoding process of subsequent pictures of higher sub-layers in decoding order.

3.126 sub-layer reference picture: A picture that contains samples that may be used for inter prediction in the decoding process of subsequent pictures of the same sub-layer in decoding order.

NOTE – Samples of a sub-layer reference picture may also be used for inter prediction in the decoding process of subsequent pictures of higher sub-layers in decoding order.

### Slice header syntax

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| if(!IRAPFlag) { |  |
| slnr_flag | u(1) |
| nonref_flag | u(1) |
| } |  |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |

Where

IRAPFlag = ((nal_unit_type == IDR_W_RADL) || ((nal_unit_type == IDR_N_LP) || (nal_unit_type == CRA_NUT))

slnr_flag equal to 1 specifies that the picture this slice belongs to is a sub-layer non-reference picture. slnr_flag equal to 0 indicates that the picture this slice belongs to is a sub-layer reference picture.

nonref_flag equal to 1 specifies that the picture that this slice belongs to is not used for inter prediction. nonref_flag equal to 0 specifies that the picture this slice belongs to may be used for inter prediction.

When TemporalId is equal to 6 and slnr_flag is equal to 1, nonref_flag shall be equal to 1.

It is a requirement of bitstream conformance that:

When present, the value of each of the slice header syntax elements slnr_flag, and nonref_flag shall be the same in all slice headers of a coded picture.

The above proposed picture property information may be optionally signalled controlled by a gating flag bit which may be signalled in a parameter set (e.g. SPS or PPS).

## Proposal 3

A bug-fix is proposed for inference rules for num_bricks_in_slice_minus1 syntax element.

In VVC WD [1] the syntax element num_bricks_in_slice_minus1 is conditionally signalled in slice header – only when syntax elements rect_tile_group_flag and single_tile_per_tile_group_flag are each equal to 0. The bug-fix below is necessary since when single_brick_per_slice_flag is equal to 1 rect_tile_group_flag is inferred to be equal to 1.

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0.

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  slice_id[ sliceIdx ] )
		sliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = top_left_brick_idx[ sliceIdx ]
	botBrickIdx = brickIdx + bottom_right_brick_idx_delta[ sliceIdx ]
	for( bIdx = 0; brickIdx  <=  botBrickIdx; brickIdx++ )	(7-40)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

## Proposal 4

A semantics restriction is proposed on the DPS referred to by SPS, for each CVS in the bitstream. Without this restriction different DPS with possibly different values for syntax elements in the DPS may be activated/ referred by different CVSes in a bitstream, which is undesirable.

#### Sequence parameter set RBSP semantics

sps_decoding_parameter_set_id, when greater than 0, specifies the value of dps_decoding_parameter_set_id for the DPS referred to by the SPS. When sps_decoding_parameter_set_id is equal to 0, the SPS does not refer to a DPS and no DPS is active when decoding each CVS referring to the SPS.

Each DPS RBSP referred by each SPS RBSP referred (or activated by) by each CVS in the bitstream shall have the same content.

# Conclusion

This document proposes comments and bug-fixes for VVC WD. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019