| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0182r1 |
| --- | --- |

| Title: | AHG12: On picture and sub-picture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a layer structure where each layer may consist of one or multiple sub-pictures forming a layer picture. HLS design for such layer structure can be summarized as follows:

- Propose to remove pic_width_in_luma_samples and pic_height_in_luma_samples in SPS

- Propose to signal maximum decoded picture width and height in DPS

- Propose to signal decoded picture width and height in PPS

- Propose to signal layer picture size, sub-picture indication and layout information associated with each layer in VPS

- Propose to override default layer picture size, sub-picture indication and layout information in SPS for the associated layer

# Introduction

JVET-N0044 compared several approaches for independently coded picture regions including MCTSs, sub-pictures and independent sub-picture layers. JVET-N0826 proposed a harmonized sub-picture-based coding design for VVC, a picture may be divided into sub-pictures, and each sub-picture may refer to its own PPS. The sub-picture indication and layout information such as location and size are signaled in SPS. JVET-N0045 proposed to specify independent sub-picture layer for VVC where each sub-picture is an independent layer and output picture layout is signaled in VPS. For sub-picture-based approach, a single SPS is activated for the picture consisting of multiple sub-pictures as shown in Figure 1. while for layer approach, a separate SPS may be activated per each layer as shown in Figure 2. In both approaches, each sub-picture refers to its own PPS.

Figure 1 sub-picture-based parameter sets

Figure 2 layer-based sub-picture parameter sets

A layer concept without inter-layer prediction (JVET-N0278) is adopted in VVC, SPS and PPS could be reused across layers. JVET-N0045 also proposed PPSs and SPSs associated with layer_id equal to 0 may be activated in multiple layers like in HEVC.

Activating a single SPS across layers may be beneficial to align sequence level coding parameters among multiple encoded version of the same content for sub-bitstream extraction and reposition process as shown in Figure 3. However, layer specific syntax elements such as pic_width_in_luma_samples and pic_height_in_luma_samples , sub-picture indication and layout information in SPS may prevent SPS to be activated across multiple layers. In addition, as specified in HEVC, the value space of active SPSs shall be the same within a CVS, and a CVS is bounded by an IRAP access unit. As a result, sub-picture change within a CVS is not possible which may impact low-latency viewport switching and ARC/RPR.

Figure 3 Parameter set activation example for sub-picture extraction and reposition

Several contributions proposed resolution signaling change to support ARC. JVET-M0135 proposed to signal a list of resolutions in SPS and use index to the SPS resolution list in PPS to specify the resolution of the picture referring to the PPS. JVET-N0048 and JVET-N0279 proposed to signal maximum resolution in SPS and signal either the resolution other than the maximum resolution or the specific resolution in PPS.

In this contribution, a decoded picture represents a picture or sub-picture that can be independently decoded, and a layer picture represents a picture consisting of one or multiple decoded sub-pictures at a specific layer. This contribution proposes to signal the size of decoded picture in PPS, and signal the default layer picture size, sub-picture indication and layout information for each layer in VPS. PPS may refer to the index of VPS layer list or override the default information as needed.

# Proposal

A layer-based sub-picture structure is illustrated in Figure 4 where each layer consists of a single or multiple sub-picture. Layer 0 contains a low-resolution layer picture consisting of 6 sub-pictures, layer 1 contains a high-resolution layer picture consisting of 6 sub-pictures, and layer 2 picture is a sub-picture corresponding to sub-picture #2 at layer 0 and sub-picture #8 at layer 1. An output picture may be formed from merged sub-pictures from layer 0 and layer 1, or from merged sub-picture from layer 0 and layer 1.

Figure 4 Example of a layer sub-picture structure

## Proposed coding picture resolution signaling

Table 1 is proposed SPS syntax with syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples removed.

Table 1 Proposed sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| …. |  |

Table 2 is proposed DPS syntax with maximum width and height of decoded pictures referring to DPS.

Table 2 Proposed decoding parameter set

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_max_sub_layers_minus1 | u(3) |
| dps_reserved_zero_bit | u(1) |
| max_pic_width_in_luma_samples | ue(v) |
| max_pic_height_in_luma_samples | ue(v) |
| profile_tier_level( dps_max_sub_layers_minus1 ) |  |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

max_pic_width_in_luma_samples specifies the maximum width of decoded pictures referring to the DPS in units of luma samples. max_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

max_pic_height_in_luma_samples specifies the maximum height of decoded pictures referring to the DPS in units of luma samples. max_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

Table 3 is the proposed PPS syntax with decoding width and height of pictures referring to PPS.

Table 3 Proposed picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| pps_pic_width_in_luma_samples | ue(v) |
| pps_pic_height_in_luma_samples | ue(v) |
| …. |  |

pps_pic_width_in_luma_samples specifies the width of decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pps_pic_height_in_luma_samples specifies the height of decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

## Proposed sub-picture information signaling

Table 4 is the proposed VPS with layer picture size, sub-picture indication and layout information specified for each layer. Some syntax elements were proposed in JVET-N0826.

Table 4 Proposed Video Parameter Set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| vps_layer_pic_width_in_luma_samples[ i ] | ue(v) |
| vps_layer_pic_height_in_luma_samples[ i ] | ue(v) |
| vps_num_sub_pics_minus1[ i ] | ue(v) |
| vps_sub_pic_id_len_minus1[ i ] | ue(v) |
| for ( j = 0; j <= vps_num_sub_pics_minus1; j++ ) { |  |
| vps_sub_pic_id[ i ][ j ] | u(v) |
| vps_sub_pic_treated_as_pic_flag[ i ][ j ] | u(1) |
| if( vps_num_sub_pics_minus1[ i ] ) { |  |
| vps_sub_pic_x_offset[ i ][ j ] | ue(v) |
| vps_sub_pic_y_offset[ i ][ j ] | ue(v) |
| vps_sub_pic_width_in_luma_samples[ i ][ j ] | ue(v) |
| vps_sub_pic_height_in_luma_samples[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_layer_pic_width_in_luma_samples[ i ] specifies the deafult width of picture associated with i-th layer in units of luma samples. A layer picture may consist of a single or multiple sub-pictures.

vps_layer_pic_height_in_luma_samples [ i ] specifies the default height of pictures associated with i-th layer in units of luma samples. A layer picture may consist of a single or multiple sub-pictures.

vps_num_sub_pics_minus1[ i ] plus 1 specifies the number of sub-pictures of pictures in the i-th layer. The value of num_sub_pics_minus1[ i ]  shall be in the range of 0 to 1024, inclusive.

vps_sub_pic_id_len_minus1[ i ] plus 1 specifies the number of bits used to represent the syntax element sub_pic_id[ i ][ j ] in the VPS. The value of vps_sub_pic_id_len_minus1 shall be in the range of Ceil( Log2( vps_num_sub_pic_minus1 + 1 ) − 1 to 9, inclusive.

vps_sub_pic_id[ i ][ j ] specifies the sub-picture ID of the j-th sub-picture in each picture of i-th layer. The length of vps_sub_pic_id[ i ][ j ]  is vps_sub_pic_id_len_minus1 + 1 bits.

vps_sub_pic_treated_as_pic_flag[ i ][ j ] equal to 1 specifies that the j-th sub-picture of each picture in the i-th layer is treated as a picture in the decoding process excluding in-loop filtering operations. sub_pic_treated_as_pic_flag[ i ][ j ] equal to 0 specifies that the j-th sub-picture of each picture in the i-th layer is not treated as a picture in the decoding process excluding in-loop filtering operations.

vps_sub_pic_x_offset[ i ][ j ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture relative to the top-left corner luma sample of each picture in the i-th layer CVS. When not present, the value of sub_pic_x_offset[ i ][ j ] is inferred to be equal to 0. sub_pic_x_offset[ i ][ j ] shall be an integer multiple of CtbSizeY.

vps_sub_pic_y_offset[ i ][ j ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture relative to the top-left corner luma sample of each picture in the i-th layer CVS. When not present, the value of sub_pic_y_offset[ i ][ j ] is inferred to be equal to 0. sub_pic_y_offset[ i ][ j ] shall be an integer multiple of CtbSizeY.

vps_sub_pic_width_in_luma_samples[ i ][ j ] specifies the width, in units of luma samples, of the j-th sub-picture of each picture in the i-th layer. When the sum of sub_pic_x_offset[ i ][ j ] and sub_pic_width_in_luma_samples[ i ][ j ] is less than pic_width_in_luma_samples, the value of sub_pic_width_in_luma_samples[ i ][ j ] shall be an integer multiple of CtbSizeY. When not present, the value of sub_pic_width_in_luma_samples[ i ][ j ] is inferred to be equal to pic_width_in_luma_samples.

vps_sub_pic_height_in_luma_samples[ i ] [ j ] specifies the height, in units of luma samples, of the j-th sub-picture for each picture in the i-th layer. When the sum of sub_pic_y_offset[ i ][ j ] and sub_pic_height_in_luma_samples[ i ][ j ] is less than pic_height_in_luma_samples, the value of sub_pic_height_in_luma_samples[ i ][ j ] shall be an integer multiple of CtbSizeY. When not present, the value of sub_pic_height_in_luma_samples[ i ][ j ] is inferred to be equal to pic_height_in_luma_samples.

An override flag is proposed to be signaled in SPS to override the default sub-picture information in VPS. Table 5 shows the proposed syntax elements.

Table 5 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_subpic_override_flag | u(1) |
| if ( sps_subpic_override_flag ) { |  |
| sps_layer_pic_width_in_luma_samples | ue(v) |
| sps_layer_pic_height_in_luma_samples | ue(v) |
| sps_num_sub_pics_minus1 | ue(v) |
| sps_sub_pic_id_len_minus1 | ue(v) |
| for ( j = 0; j <= sps_num_sub_pics_minus1; j++ ) { |  |
| sps_sub_pic_id[ i ] | u(v) |
| sps_sub_pic_treated_as_pic_flag[ i ] | u(1) |
| if( sps_num_sub_pics_minus1[ i ] ) { |  |
| sps_sub_pic_x_offset[ i ] | ue(v) |
| sps_sub_pic_y_offset[ i ] | ue(v) |
| sps_sub_pic_width_in_luma_samples[ i ] | ue(v) |
| sps_sub_pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_subpic_override_flag equals to 1 specifies the syntax elements sps_layer_pic_width_in_luma_samples, sps_layer_pic_height_in_luma_samples, sps_num_sub_pics_minus1, sps_sub_pic_id_len_minus1, sps_sub_pic_id[ i ], sps_sub_pic_treated_as_pic_flag[ i ], sps_sub_pic_x_offset[ i ], sps_sub_pic_y_offset[ i ], sps_sub_pic_width_in_luma_samples[ i ], sps_sub_pic_height_in_luma_samples[ i ] are present in the SPS. sps_subpic_override_flag equal to 0 indicates these syntax elements are not present in the SPS and their value may be inferred from the corresponding

The semantics of syntax elements sps_layer_pic_width_in_luma_samples, sps_layer_pic_height_in_luma_samples, sps_num_sub_pics_minus1, sps_sub_pic_id_len_minus1, sps_sub_pic_id[ i ], sps_sub_pic_treated_as_pic_flag[ i ], sps_sub_pic_x_offset[ i ], sps_sub_pic_y_offset[ i ], sps_sub_pic_width_in_luma_samples[ i ], sps_sub_pic_height_in_luma_samples[ i ] are similar to the semantics of the corresponding VPS syntax elements.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).