| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0183 |
| --- | --- |

| Title: | AHG12: On layer-based sub-picture extraction and reposition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to indicate Output Sub-Picture Sets (OSPS) in VPS for sub-picture extraction and reposition across multiple layers. The proposal can be summarized as follows:

- A bitstream may consist of multiple layers and a layer picture may be divided into sub-pictures.

- Output Sub-picture Sets (OSPS) are indicated in VPS to specify normative sub-bitstream extraction, reposition and conformance points for sub-pictures and sets.

- Sub-bitstream extraction process update to support sub-picture extraction and reposition across multiple layers.

# Introduction

JVET-N0278 was adopted in 14th meeting to support layer concept without inter-layer referencing in VVC. nuh_layer_id is specified in NAL unit header as identifier of the layer associated to a NAL unit, and VPS is included in VVC bitstream to specify maximum allowed number of layers and corresponding layer id. Sub-bitstream extraction process is also specified to extract a sub-bitstream based on a target nuh_layer_id value lIdTarget, and a target highest TemporalId value tIdTarget.

A harmonized design of sub-picture-based coding for VVC was proposed in 14th meeting, it proposed a picture may be divided into sub-pictures, and each sub-picture may refer to its own PPS. The sub-picture position and size is proposed to be signaled in SPS. Output Sub-Picture Sets (OSPS) are proposed to specify normative extraction and conformance points.

Several sub-picture-based viewport dependent omnidirectional video processing approaches have been recommended in OMAF to support sub-picture bitstreams with several resolutions merged to one bitstream. Figure 1 illustrates one example to achieve 5K effective ERP resolution.

Figure 1 Example of extractor track combining sub-pictures from bitstreams of different resolution

This contribution proposes a sub-picture-based design to support merging of sub-picture tracks of different resolutions and produce a repositioned single output bitstream based on the VVC layer concept.

# Proposal

The proposed sub-picture-based design is summarized as follows,

- A bitstream may consist of one or multiple layers.

- A layer picture may be divided into one or multiple sub-pictures.

- Output Sub-picture Sets (OSPS) is indicated in VPS to specify normative extraction, reposition conformance points for sub-pictures and sets

Figure 2 is a layer-based sub-picture example, where layer  0 is a low-resolution picture consisting of 6 sub-pictures, and layer 1 is a high-resolution picture consisting of 5 sub-pictures, and layer 2 is a sub-picture corresponding to sub-picture #2 at layer 0 and #8 at layer 1. Each layer is an independent layer.

Figure 2 Example of a layer-based sub-picture structure

Output sub-picture set (OSPS) is set of sub-pictures accompanied by profile, tier, level and HRD information for means of extraction of a bitstream that is created from an input bitstream by operation of the sub-bitstream extraction and reposition process with the input bitstream, a target nuh_layer_id value lIdTarget, a target highest TemporalId value, and a target sub-picture list including the identifier, location and size of the sub-pictures as inputs. In Figure 2 example, an output picture can be formed from merged sub-pictures from layer 0 and layer 1, or from merged sub-picture from layer 0 and layer 1.

## Proposed changes of video parameter set

In Table 1, the syntax elements highlighted in yellow were proposed in JVET-N0826 (signaled in SPS), and new proposed syntax elements are highlighted in cyan.

Table 1 Proposed Video Parameter Set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| output_sub_pic_sets_present_flag | u(1) |
| if( output_sub_pic_sets_present_flag ) { |  |
| num_osps_format_sets | ue(v) |
| for( i = 0; i < num_osps_format_sets – 1 ; i++ ){ |  |
| osps_width_in_luma_samples[ i ] | ue(v) |
| osps_height_in_luma_samples[ i ] | ue(v) |
| profile_tier_level(sps_max_sub_layers_minus1)[ i ] |  |
| } |  |
| num_output_sub_pic_sets | ue(v) |
| for( i = 0; i < num_output_sub_pic_sets – 1 ; i++ ) { |  |
| num_sub_pics_in_osps[ i ] | ue(v) |
| osps_format_set_idx[ i ] | ue(v) |
| for( j = 0; j < num_sub_pics_in_osps[ i ] – 1 ; j++ ) { |  |
| if( vps_max_layers_minus1 != 0) |  |
| sub_pic_layer_id_in_osps[ i ][ j ] | u(7) |
| sub_pic_id _in_osps[ i ][ j ] | ue(v) |
| sub_pic_temporal_id_in_osps[ i ][ j ] | ue(v) |
| sub_pic_x_offset_in_osps[ i ][ j ] | ue(v) |
| sub_pic_y_offset_in_osps[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantic of new proposed syntax elements is defined as follows.

sub_pic_layer_id_in_osps[ i ][ j ] specifies the layer identifier lIdTarget of j-th sub-picture belonging to the i-th output sub-picture set.

sub_pic_temporal_id_in_osps[ i ][ j ] specifies the highest TemporalId value tIdTarget of j-th sub-picture belonging to the i-th output sub-picture set.

sub_pic_x_offset_in_osps[ i ][ j ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture belonging to the i-th output sub-picture set relative to the top-left corner luma sample of output picture in the CVS. When not present, the value of sub_pic_x_offset_in_osps[ i ][ j ] is inferred to be equal to 0. sub_pic_x_offset_in_osps[ i ][ j ] shall be an integer multiple of CtbSizeY.

sub_pic_y_offset_in_osps[ i ][ j ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture belonging to the i-th output sub-picture set relative to the top-left corner luma sample of each picture in the CVS. When not present, the value of sub_pic_y_offset_in_osps[ i ][ j ] is inferred to be equal to 0. sub_pic_y_offset_in_osps[ i ][ j ] shall be an integer multiple of CtbSizeY.

## Proposed sub-bitstream extraction and reposition process for clause 10

Inputs to this process are a bitstream, and a target sub-picture set subPicSetTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, every lIdTarget associated with subPicSetTarget specified in the active VPS equal to any value in the range of 0 to 126, inclusive, and every tIdTarget associated with subPicSetTarget specified in the active VPS equal to any value in the range of 0 to 6, inclusive, and sIdtarget equal to sub_pic_id associated with subPicSetTarget specified in the active VPS as inputs, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with sub_pic_id equal to sIdTarget, TemporalId equal to tIdTarget and nuh_layer_id equal to lIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

The extracted sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	Remove all NAL units with nuh_layer_id not equal to lIdTarget.

The reposition bitstream is derived as follows:

–	Merge the NAL units of extracted sub-bitstreams in the order as specified in subPicSetTarget.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).