| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0185 |
| --- | --- |

| Title: | AHG8/AHG17: On layer dependency signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes HLS design to support scalability in VVC v1. The proposals can be summarized as follows:

- Support spatial and SNR scalability in VVC v1 without a separate scalability profile

- Propose to signal vps_all_layers_independent_flag, layer ID and layer dependency in VPS

- In case a separate scalability profile is under consideration

  - The layer ID of all independent layers are signaled in VPS

  - The layer ID of all layers are signaled in VPS extension

  - The dependency among layers are signaled in VPS extension

# Introduction

MPEG document N17074, “Requirements for a future video coding standard v5”, clause 5.14 has the following statement about scalability requirement for VVC:

Scalability modalities (such as temporal, spatial, and SNR scalability) shall be supported.

VCEG document VCEG-BD03, “Requirements for Future Video Coding”, clause 11 has the following statement about hierarchical/layered coding for VVC:

Requirement: - FVC should support extraction of decodable subsets of a bitstream. Remark: Scalability modalities (such as temporal, spatial, and SNR scalability) can be considered in a first version of a future codec specification.

ATSC3.0 supports HEVC Scalable Main and Scalable Main 10 profiles to enable HDR and Multiview applications. However, the overall deployment of scalable video codec is not comparable to layer codec. One of the main concerns is the separate codec profiles and versions. Given the lessens learned from past standard development and demands of emerging immersive media applications (e.g. 360-degree video), it is desirable to have scalability support in VVC v1 main profile.

JVET-N0278 was adopted in 14th meeting to support layer concept without inter-layer referencing in VVC, layer id and basic layer-based HLS are available in VVC, the consideration of basic scalable capabilities would further enhance VVC support for broader scalable applications.

For SHVC, the reference layer picture, resampled if necessary, is used as additional reference picture for enhancement layer prediction, which enables interlayer texture and motion parameter prediction. Most SHVC relevant syntax elements are specified in the parameter set extension so that a decoder designed for a single layer application may not have to parse those SHVC relevant syntax elements.

This contribution proposes HLS design supporting basic spatial and SNR scalability.

# VPS/VPS extension syntax and semantics

## Single profile for scalability and non-scalability

When a single VVC profile would support both scalable and non-scalable capability, the scalable relevant HLS can be consolidated as follows:

Table 1 Proposed VPS syntax II

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_all_layers_independent_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if (!vps_all_layers_independent_flag) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_all_layers_independent_flag equal to 1 specifies all layers that are specified by the VPS are independent layer. vps_all_layers_independent_flag equal to 0 specifies the layer that are specified by the VPS may not be a independent layer.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i − 1 ]. For any value of i in the range of 0 to MaxLayersMinus1, inclusive, when not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, it is inferred to be equal to 0.

The variable DependencyFlag[ i ][ j ] is derived as follows:

for( i = 0; i <= MaxLayersMinus1; i++ )
         for( j = 0; j <= MaxLayersMinus1; j++ ) {
                   DependencyFlag[ i ][ j ] = direct_dependency_flag[ i ][ j ]
                   for( k = 0; k < i; k++ )
                          if( direct_dependency_flag[ i ][ k ] && DependencyFlag[ k ][ j ] ) 
                                DependencyFlag[ i ][ j ] = 1
         }

## A separate scalability profile

The following HLS design assumes separate scalable profile to be specified in VVC.

Table 2 proposed VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_all_layers_independent_flag | u(1) |
| if (!vps_all_layers_independent_flag) { |  |
| vps_num_independent_layers_minus1 | ue(v) |
| for( i = 0; i <= vps_num_independent_layers_minus1; i++ ) |  |
| vps_independent_layer_id[ i ] | ue(v) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_all_layers_independent_flag equal to 1 specifies all layers that are specified by the VPS are independent layer. vps_all_layers_independent_flag equal to 0 specifies the layer that are specified by the VPS may not be a independent layer.

vps_num_independent_layers_minus1 plus 1 specifies the number of independent layers that are specified by the VPS. The value of vps_num_indepedent_layers_minus1 shall be in the range of 0 to vps_max_layers_minus1, inclusive.

vps_independent_layer_id[ i ] specifies the value of nuh_layer_id of i-th independent layer.

Table 3 proposed VPS extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i − 1 ]. For any value of i in the range of 0 to MaxLayersMinus1, inclusive, when not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, it is inferred to be equal to 0.

The variable DependencyFlag[ i ][ j ] is derived as follows:

for( i = 0; i <= MaxLayersMinus1; i++ )
         for( j = 0; j <= MaxLayersMinus1; j++ ) {
                   DependencyFlag[ i ][ j ] = direct_dependency_flag[ i ][ j ]
                   for( k = 0; k < i; k++ )
                          if( direct_dependency_flag[ i ][ k ] && DependencyFlag[ k ][ j ] ) 
                                DependencyFlag[ i ][ j ] = 1
         }

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).