| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0187-v1 |
| --- | --- |

| Title: | Non-CE3: MIP simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Thibaud Biatek Luong Pham Van Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This document proposes modifications to matrix intra prediction (MIP) in VVC. The proposed modifications are as follows: (1) treating intra mode of neighbouring MIP-coded block as PL mode for non-MIP MPM derivation and for chroma DM derivation, (2) disabling MIP for 4x4 coding blocks, and (3) aligning the reference sample derivation for MIP with the non-MIP reference sample derivation. It is asserted that these modifications simplify the MIP tool, provide simpler specification text, reduce the memory requirement for storing the MIP matrices and result in negligible BD-rate loss.

# Introduction

Matrix intra prediction (MIP) was adopted in the 14th JVET meeting. Under MIP, prediction samples for a block are derived from a subset of the reference samples of the block using a matrix multiplication. The block sizes are classified into three categories, and under each category several MIP modes are defined. The encoder signals the mode from the category associated with the current block: the signalling mechanism may either using an MPM index (three MPMs are defined for MIP) or a remaining mode index. Table 1 shows the different categories of blocks and number of MIP modes defined for each category.

Table 1: Number of MIP modes and parameters for various block sizes

| Category | Block sizes | Number of MIP modes | Number of parameters |
| --- | --- | --- | --- |
| 1 | 4x4 | 35 | 1440 |
| 2 | 4x8, 8x4, 8x8 | 19 | 1440 |
| 3 | Other block sizes | 11 | 3456 |

For derivation of MPM of non-MIP blocks, neighbouring MIP-coded blocks are considered as follows: a mapping table is used to derive an “equivalent” regular mode from the MIP mode used by the neighbouring MIP-coded block. This equivalent regular mode is used in the MPM derivation of non-MIP blocks. Similarly, in chroma DM derivation, a collocated luma block may be coded with MIP; an equivalent regular mode is derived using the mapping table and used as the chroma DM mode.

MIP also defines a new reference sample derivation process for deriving the reference samples. Non-MIP blocks already have a reference sample derivation process; the derivation process used by MIP and non-MIP modes are non-identical.

# Proposal

This document proposes three simplifications for the MIP tool:

- Proposal 1: When a neighbouring block is MIP-coded, the MPM derivation for non-MIP modes considers the neighbouring block as unavailable and uses PL mode as the neighbouring mode. Similarly, when a collocated luma block used in chroma DM derivation is MIP-coded, the chroma DM mode is chosen as PL mode. These simplifications result in removal of the mapping between MIP and intra prediction modes (Table 8-4 in specification).

- Proposal 2: For 4x4 blocks, 35 matrices are defined, and the parameters defined for the 4x4 blocks account for nearly 23% of the total number of parameters specified for MIP. It is proposed that MIP be disallowed for 4x4 blocks. The resultant BD-rate loss is minimal.

- Proposal 3: It is proposed to align the reference sample derivation process for MIP and non-MIP modes. The proposal is to re-use the reference sample derivation that is used currently for the non-MIP modes also for the MIP modes.

# Simulation results

Simulation results for the proposed changes are provided in the following Tables.

## Proposal 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% |  |  |
| Class A2 | 0.03% | 0.02% | -0.02% |  |  |
| Class B | 0.01% | -0.04% | 0.09% |  |  |
| Class C | 0.00% | -0.12% | 0.05% |  |  |
| Class E | 0.03% | 0.01% | 0.03% |  |  |
| Overall | 0.01% | -0.02% | 0.04% |  |  |
| Class D | -0.03% | 0.19% | 0.00% |  |  |
| Class F | -0.02% | -0.02% | 0.08% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% |  |  |
| Class A2 | 0.07% | 0.22% | 0.06% |  |  |
| Class B | 0.01% | 0.04% | 0.01% |  |  |
| Class C | -0.02% | 0.15% | 0.01% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% |  |  |
| Class D | 0.00% | 0.07% | 0.00% |  |  |
| Class F | 0.00% | 0.13% | 0.17% |  |  |

## Proposal 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.01% |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.04% |  |  |
| Class C | 0.06% | 0.02% | -0.01% |  |  |
| Class E | 0.03% | -0.10% | -0.08% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.11% | 0.25% | 0.00% |  |  |
| Class F | 0.04% | 0.04% | 0.21% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.20% |  |  |
| Class C | 0.00% | -0.24% | 0.01% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.06% | 0.11% | 0.03% |  |  |
| Class F | 0.04% | -0.04% | -0.04% |  |  |

## Proposal 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.01% | -0.02% | 0.01% |  |  |
| Class E | 0.01% | -0.06% | -0.03% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.31% | -0.01% |  |  |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.05% | -0.20% | -0.20% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | 0.08% | 0.26% |  |  |
| Class F |  |  |  |  |  |

# 8-bit implementation of MIP

JVET-O0084 proposes an 8-bit version of MIP, where the matrix coefficients are represented by eight bits. The proposal results in less memory requirement, and also provides some coding efficiency.

The proposed simplifications in this document are also tested on top of JVET-O0084. As JVET-O0084 already includes an alignment of the reference sample derivation of MIP and non-MIP modes, Proposal 3 is not tested here. Proposals 1 and 2 are tested on the software provided with JVET-O0084. The simulation results obtained are given in the following tables.

## Proposal 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.03% | 0.04% |  |  |
| Class C | 0.01% | 0.07% | 0.03% |  |  |
| Class E | 0.01% | 0.08% | 0.04% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | 0.15% | -0.01% |  |  |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.03% | 0.16% |  |  |
| Class C | 0.01% | 0.08% | 0.01% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | 0.28% | -0.33% |  |  |
| Class F | 0.05% | 0.12% | 0.04% |  |  |

## Proposal 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | -0.03% |  |  |
| Class C | 0.08% | 0.00% | 0.03% |  |  |
| Class E | 0.01% | -0.03% | 0.05% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.16% | 0.29% | 0.02% |  |  |
| Class F | 0.03% | 0.05% | 0.04% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.09% | -0.23% |  |  |
| Class C | 0.06% | -0.08% | -0.26% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | 0.20% | 0.16% |  |  |
| Class F | 0.05% | -0.02% | 0.28% |  |  |

# Conclusions

Three simplifications to matrix intra prediction are proposed. The benefits of these simplifications include simpler MIP design, less memory requirement, cleaner specification text and a more unified design for MIP and non-MIP reference sample derivation. The proposed changes result in negligible performance loss. It is proposed to adopt these simplifications to VVC.

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).