| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0190-v1 |
| --- | --- |

| Title: | Non-CE4: LIC model parameters using forward mapped luma samples in LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jay N Shingala Ashwin Natesan | Tel: Email: | +91 8066601000 jay.shingala@ittiam.com ashwin.natesan@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd |  |  |

_____________________________

# Abstract

This proposal relates to simplifying the normative method of deriving the linear model parameters for Local illumination compensation (LIC) in the mapped (reshaped) domain by using the neighbour reconstruction samples and forward mapped neighbour inter predicted samples when Luma mapping with chroma scaling (LMCS) is enabled. The proposed method computes the LIC scaling parameter ‘a’ in the forward mapped (reshaped) domain and the offset parameter ‘b’ in the original (unmapped) domain so that the decode pipeline order of Motion Compensation (which includes LIC) is followed by forward mapping of inter prediction as per the existing scheme of LMCS. The method of deriving LIC model parameters for chroma is unchanged as chroma samples are not reshaped.

The proposed method to derive the LIC parameters in the mapped (reshaped) domain was implemented on top of CE 4-3.1a [2] core LIC experiment in VTM-5.0 [3]. Test results reportedly show BD-rate gain of 0.47% (Y), 0.35% (U), 0.46% (V) with encoding time of 118% and decoding time of 100% for RA and  BD-rate  gain of 0.34% (Y), 0.22% (U), 0.50% (V) with encoding time of 116% and decoding time of 100% for LDB configuration under CTC test conditions [4].

# Introduction

In the base LIC of JVET-N0343 [1], when LMCS (luma reshaping) is used, i.e. when slice_lmcs_enabled_flag is enabled, inverse mapping (InvLUT) is applied to the neighbour reconstructed samples of the current CU prior to LIC parameter derivation, since the current CU neighbours are in the mapped (reshaped) domain while the reference picture samples are in the original (non-reshaped) domain. The process of LIC parameter derivation using neighbour reconstruction (C) and reference inter predicted samples (L) and linear transformation of current prediction in original domain followed by forward mapping (FwdLUT) is pictorially represented in Figure 1. This method is applicable in all subsequent LIC core experiments including CE-4.3.1 [1] of JVET-O meeting.

The equations for deriving the LIC scale (a) and offset (b) in the original domain using inverse mapping of reconstruction ‘C’  and reference inter prediction ‘L’ of ‘N’ neighbour samples are shown below

a= N·∑Ln·InvLUT(Cn)-∑Ln·∑InvLUT(Cn)N·∑Ln·Ln-∑Ln·∑Ln

b= ∑InvLUT(Cn)-a·∑LnN

As inverse mapping (InvLUT) is computationally more expensive than the equi-spaced piecewise linear forward mapping (FwdLUT) process, it is desirable to compute the LIC parameters in the mapped (reshaped) domain to reduce LIC complexity when LMCS is enabled.

Figure 1: LIC model in original domain                    Figure 2: Proposed LIC model in mapped domain

# Proposed method

The proposed method in this contribution uses current CU neighbour reconstructed samples as is (in reshaped domain) and the forward mapped neighbour inter reference samples for derivation of LIC parameters as depicted in Figure 2.

Simplest way to eliminate the inverse mapping in LIC could be to compute the LIC parameters in mapped (reshaped) domain by using the neighbour recon samples (C) as is and the forward mapped inter prediction (L) samples. However, this would mean the linear transformation for LIC needs to be applied in the mapped (reshaped) domain after completing the forward mapping (FwdLUT) of current inter prediction data, which can affect the decode pipeline order in current LMCS scheme.

The proposed method addresses this issue by computing only the offset parameter (b) in the original domain. This is accomplished by using single inverse mapping of the average of neighbour reconstructed samples and using sum of neighbour inter prediction samples in original domain. The scale parameter ‘a’ is expected to be nearly same in the mapped (reshaped) domain and original domain. The derivation of ‘a’ and ‘b’ as per proposed method are depicted in equation below

a= N·∑FwdLUT(Ln)·Cn-∑FwdLUT(Ln)·∑CnN·∑FwdLUT(Ln)·FwdLUT(Ln)-∑FwdLUT(Ln)·∑FwdLUT(Ln)

b= N.InvLUT∑CnN-a·∑LnN

Note that the modified equation for ‘a’ and ‘b’ is applicable only for luma as chroma samples are not reshaped.

# Test results

The proposed method is applied on top of unidirectional LIC core experiment of CE-4.3.1a [1] in VTM5.0. The simulations were performed under JVET common test conditions (CTC) [4]. Table-1 shows the test results for the proposed method against VTM-5.0. The BD-rate gains of the proposed method are comparable to the reported core LIC gains of CE-4.3.1a [1]

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.43% | -0.45% | 121% | 103% |
| Class A2 | -0.39% | -0.32% | -0.43% | 108% | 95% |
| Class B | -0.48% | -0.23% | -0.36% | 121% | 101% |
| Class C | -0.37% | -0.47% | -0.62% | 122% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.35% | -0.46% | 118% | 101% |
| Class D | -0.13% | -0.02% | -0.04% | 117% | 100% |
| Class F | -0.67% | -0.39% | -0.47% | 117% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.04% | -0.59% | 128% | 100% |
| Class C | -0.42% | -0.64% | -0.47% | 123% | 98% |
| Class E | 0.00% | 0.03% | -0.40% | 90% | 103% |
| Overall | -0.34% | -0.22% | -0.50% | 116% | 100% |
| Class D | -0.17% | -0.17% | -0.04% | 115% | 100% |
| Class F | -1.75% | -1.58% | -1.62% | 114% | 103% |

Table 1. Results of proposed LIC model parameter derivation in CE-4.3.1a against VTM-5.0

# Conclusion

The proposed modifications in LIC model derivation process retains the gains of LIC whilst replacing the inverse mapping (invLUT) of neighbour reconstruction samples with forward mapping (fwdLUT) of reference inter prediction samples.

# References

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, and M. Karczewicz, "CE1: Unidirectional illumination compensation (CE1-5.1)," Document of Joint Video Experts Team, JVET-N0343

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, H. Chen, X. Ma and H. Yang, "CE4-3.1a and CE4-3.1b: Unidirectional local illumination compensation with affine prediction," Document of Joint Video Experts Team, JVET-O0066.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).