| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0191-v4 |
| --- | --- |

| Title: | CE1-related: Simplified CABAC initialization method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd |  |  |

_____________________________

# Abstract

Several CABAC initialization methods are studied in CE1. An alternative is proposed, which is asserted to have lower complexity. Indeed 25% faster code can be realized in software compared to the fastest CE1 method. From a coding efficiency point of view, the proposed method is as good if not better than the methods studied in CE1 under the conditions defined in CE1: considering averages, in the worst case, it is 0.01% worse than the best CE1 result (RA: –0.01% vs –0.02%); in the best case it is 0.11% better than the best CE1 result (RA, high QP: 0.11% vs 0.22%).

Version 2 of this document includes updated results for the low QP range.

Version 3 of this document includes additional source code and results related to software SIMD implementations.

Version 4 of this document includes additional results obtained with encoding each CTU independently.

# Introduction

The CABAC initialization methods studied in CE1 have a common trait: they remove the need for a lookup table that is currently present in the draft VVC specification. Additionally, an initial value in computed as a linear combination of QP and 2 constants, which is then clipped to a valid range. A simplified method that does not require such clipping is proposed.

# Proposal

The quantization parameter QP is first mapped into a variable q which may take values between 0 and 16:

q = Clip3(8, 24, qp >> 1) – 8

Note that there is already a corresponding clipping operation in the draft VVC specification. Here the range is tightened such as to enable operations with lower bit counts. Additionally, the least significant bit of QP is dropped.

Two variables a and b are derived from a 6-bit initialization value initValue as follows:

a = (initValue >> 3) * 9

b = (initValue & 7) * 9

Note that a and b are both 6-bit values where the 3 most significant bits are identical to the 3 least significant bits.

A 10-bit state variable s is then derived as:

s = 16 * a + (b – a) * q + 8

A second 14-bit state variable is derived by simply scaling s by 16. These two state variables represent the initial state of the CABAC probability estimator.

# Results

## Compression efficiency

Compression efficiency is measured according to the criteria defined in CE1. At a high level the method compares as follows to CE1 methods as follows:

| Method | AI | RA | LB | LP | RA (low) | RA (high) |
| --- | --- | --- | --- | --- | --- | --- |
| This proposal | -0.02% | -0.01% | -0.02% | -0.09% | -0.01% | 0.11% |
| Best CE1 | -0.02% | -0.02% | -0.01% | -0.09% | -0.02% | 0.22% |
| Best 6-bit CE1 | -0.02% | -0.01% | 0.00% | -0.09% | -0.01% | 0.22% |

Details with per-class averages are provided below.

Regular QP range:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 0.00% | 102% | 102% |
| Class A2 | -0.01% | 0.00% | -0.05% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.06% | -0.06% | -0.02% | 101% | 101% |
| Class C | -0.03% | -0.03% | 0.03% | -0.03% | 103% | 101% |
| Class E | -0.03% | -0.20% | -0.22% | -0.05% | 99% | 99% |
| Overall | -0.02% | -0.06% | -0.06% | -0.02% | 101% | 101% |
| Class D | -0.03% | -0.02% | -0.28% | -0.04% | 100% | 100% |
| Class F | -0.03% | 0.03% | -0.05% | 0.00% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.14% | 0.13% | 0.00% | 99% | 101% |
| Class A2 | -0.01% | 0.26% | 0.13% | 0.02% | 101% | 102% |
| Class B | 0.01% | 0.09% | -0.20% | 0.01% | 101% | 101% |
| Class C | -0.05% | -0.08% | -0.05% | -0.06% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.09% | -0.03% | -0.01% | 101% | 101% |
| Class D | -0.02% | 0.17% | -0.02% | -0.02% | 101% | 101% |
| Class F | -0.02% | -0.04% | -0.06% | -0.01% | 102% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.22% | -0.15% | 0.00% | 101% | 102% |
| Class C | -0.03% | 0.42% | -0.40% | -0.01% | 99% | 101% |
| Class E | -0.01% | 0.01% | 0.63% | 0.02% | 101% | 101% |
| Overall | -0.02% | 0.24% | -0.04% | 0.00% | 100% | 102% |
| Class D | -0.04% | 0.30% | -0.84% | -0.05% | 102% | 101% |
| Class F | -0.12% | 0.24% | 0.51% | -0.14% | 102% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | -0.07% | -0.29% | -0.06% | 104% | 102% |
| Class C | -0.13% | 0.05% | -0.36% | -0.13% | 103% | 101% |
| Class E | -0.12% | 0.12% | 0.17% | -0.10% | 101% | 99% |
| Overall | -0.09% | 0.02% | -0.20% | -0.09% | 103% | 101% |
| Class D | 0.01% | -0.02% | -1.56% | -0.03% | 100% | 100% |
| Class F | -0.12% | 0.07% | -0.15% | -0.06% | 100% | 101% |

Low QP range:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.04% | 0.02% | 97% | 98% |
| Class A2 | -0.01% | 0.01% | 0.02% | 0.00% | 97% | 99% |
| Class B | -0.01% | 0.03% | 0.03% | 0.00% | 100% | 99% |
| Class C | -0.02% | 0.04% | 0.00% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.02% | 0.00% | 99% | 100% |
| Class D | -0.03% | 0.06% | -0.10% | -0.04% | 100% | 101% |
| Class F | 0.14% | 0.03% | 0.04% | -0.34% | 100% | 101% |

High QP range:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.17% | 1.36% | 0.54% | 0.18% | 104% | 101% |
| Class A2 | 0.10% | 0.80% | 0.03% | 0.13% | 103% | 102% |
| Class B | -0.06% | 0.88% | -0.06% | -0.02% | 98% | 101% |
| Class C | 0.29% | 0.93% | 0.83% | 0.31% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.11% | 0.97% | 0.31% | 0.14% | 101% | 101% |
| Class D | 0.42% | 2.02% | 1.83% | 0.44% | 97% | 100% |
| Class F | 0.12% | 0.99% | 0.72% | 0.17% | 102% | 101% |

Additional tests were conducted using the standard QP range where the VTM was configured to use 1 CTU per slice. Configuration parameters were set as follows:

SliceMode : 1

SliceArgument :1

| Configuration | This proposal | CE1-1.2 |
| --- | --- | --- |
| AI | -0.03 / -0.35 / -0.39 | -0.06 / 0.19 / -0.14 |
| RA | -0.12 / -0.22 / -0.24 | 0.02 / 0.00 / 0.28 |
| LB |  |  |
| LP | -0.08 / 0.97 / -0.99 | -0.08 / 0.95 / -1.32 |

## Complexity considerations

Several initialization methods from CE1 were implemented in an optimized and vectorized way in software to measure execution speed, as summarized in the table below.

| Experiment | CE1-1.1 | CE1-1.2 | CE1-3.1.b | Proposed |
| --- | --- | --- | --- | --- |
| Cycles/context | 1.18 | 1.03 | 1.57 | 0.75 |

Within CE1 experiment 1.2 was measured to be the fastest. In particular it is faster than 1.1 given that it requires fewer clipping operations.

The proposed initialization method was also implemented in SIMD. Note that in all cases the format output by the initialization function is such as to match the format used in highly efficient implementations of the core engine, as previously evaluated in a CE5.

The proposed method is 25% faster than the next fastest method (see experiment 1.2 of CE1). Indeed, the fastest CE1 method uses 1.03 cycles per context, and the proposed method 0.75 cycles per context.

The code below shows the SIMD-coded initialization loop. A set of 400 contexts is assumed (which is close to the current number in VVC). It should be noted that initialization values are stored in an optimized format in the “states” array.

void init(int qp, uint32_t *probs, uint8_t *states, uint8_t *rates)

{

qp -= 16;

if (qp < 0) qp = 0;

if (qp > 32) qp = 32;

qp >>= 1;

for (int i = 0; i < 400; i += 8)

{

__m128i x = _mm_loadl_epi64((__m128i *)&states[i]);

__m128i y = _mm_loadl_epi64((__m128i *)&rates[i]);

__m128i y0 = _mm_unpacklo_epi8(y, _mm_setzero_si128());

x = _mm_unpacklo_epi8(x, x);

x = _mm_and_si128(x, _mm_set1_epi16(0xf00f));

x = _mm_maddubs_epi16(_mm_set1_epi16(256 * 9 + qp * 9), x);

x = _mm_add_epi16(x, _mm_set1_epi16(8 - 8 * 9 * qp));

__m128i hi = _mm_add_epi16(_mm_slli_epi16(x, 6), _mm_srli_epi16(x, 9));

__m128i lo = _mm_add_epi16(_mm_slli_epi16(x, 7), y0);

__m128i y00 = _mm_unpacklo_epi16(lo, hi);

__m128i y01 = _mm_unpackhi_epi16(lo, hi);

_mm_store_si128((__m128i *)&probs[i], y00);

_mm_store_si128((__m128i *)&probs[i+4], y01);

}

}

The code below shows the SIMD-coded initialization loop for method CE1-1.2 that was used to produce the results above. It has been measured to be about 20% faster than the code provided in contribution JVET-O0065-v2.

void init(int qp, uint32_t *probs, uint8_t *states, uint8_t *rates)

{

for (int i = 0; i < 400; i += 8)

{

__m128i x = _mm_loadl_epi64((__m128i *)&states[i]);

__m128i y = _mm_loadl_epi64((__m128i *)&rates[i]);

__m128i y0 = _mm_unpacklo_epi8(y, _mm_setzero_si128());

__m128i m = _mm_and_si128(x, _mm_set1_epi8(0x38));

__m128i n = _mm_shuffle_epi8(_mm_setr_epi8(0*18+1, 1*18+1, 2*18+1, 3*18+1, 4*18+1, 5*18+1, 6*18+1, 7*18+1,

0*18+1, 1*18+1, 2*18+1, 3*18+1, 4*18+1, 5*18+1, 6*18+1, 7*18+1), x);

__m128i x0 = _mm_maddubs_epi16(_mm_unpacklo_epi8(n, m), _mm_set1_epi16(16+256*(qp-16)));

x0 = _mm_and_si128(x0, _mm_set1_epi16(~15));

x0 = _mm_sub_epi16(x0, _mm_set1_epi16(32*(qp-16)));

x0 = _mm_max_epi16(_mm_set1_epi16(1*16), _mm_min_epi16(_mm_set1_epi16(127*16), x0));

__m128i hi = _mm_add_epi16(_mm_slli_epi16(x0, 5), _mm_srli_epi16(x0, 10));

__m128i lo = _mm_add_epi16(_mm_slli_epi16(x0, 6), y0);

__m128i y00 = _mm_unpacklo_epi16(lo, hi);

__m128i y01 = _mm_unpackhi_epi16(lo, hi);

_mm_store_si128((__m128i *)&probs[i], y00);

_mm_store_si128((__m128i *)&probs[i+4], y01);

}

}

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text

Modify section 9.5.2.2 as follows:

For each context variable, the two variables pStateIdx0 and pStateIdx1 are initialized as follows:

- Table 9-6 to Table X-X contain the values of the 86 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses 7.3.7.1 through 7.3.7.11, except end_of_brick_one_bit and pcm_flag.

- From the 86 bit table entry initValue, the two 43 bit variables slopeIdxlowIdx and offsetIdxhighIdx are derived as follows:

slopeIdxlowIdx = initValue  >>  43
offsetIdxhighIdx = initValue & 157	(9-6)

- The variables m and n, used in the initialization of context variables, are derived from slopeIdxlowIdx and offsetIdxhighIdx as follows:

m = slopeIdx * 5 – 45(highIdx – lowIdx) * 9
n = ( offsetIdx  <<  3 ) – 16lowIdx * 9 * 16 + 8	(9-7)

- The two values assigned to pStateIdx0 and pStateIdx1 for the initialization are derived from SliceQpY, which is derived in Equation 7-60. Given the variables m and n, the initialization is specified as follows:

preCtxState = Clip3( 0, 127, ( ( m * Clip3( 0, 5116, ( ( SliceQpY >> 1 ) – 8 )  >>  4 ) + n )	(9-8)

- The two values assigned to pStateIdx0 and pStateIdx1 for the initialization are derived as follows from initial state mapping table initStateIdxToState[ preCtxState ] specified in Table 9-4:

pStateIdx0 = initStateIdxToState[preCtxState] >> 4
pStateIdx1 = initStateIdxToState[preCtxState] << 4	(9-9)

NOTE 1 – The variables pStateIdx0 and pStateIdx1 correspond to probability state indices as further described in subclause 9.5.4.3.

In Table 9-5, the ctxIdxOffset for which initialization is needed for each of the three initialization types, specified by the variable initType, are listed. Also listed is the table number that includes the values of initValue needed for the initialization for each value of ctxIdxInc. For P and B slice types, the derivation of initType depends on the value of the cabac_init_flag syntax element. The variable initType is derived as follows:

if( slice_type  = =  I )
	initType = 0
else if( slice_type  = =  P )
	initType = cabac_init_flag ? 2 : 1	(9-10)
else
	initType = cabac_init_flag ? 1 : 2

Table 9-4 – Initial state mapping table initStateIdxToState[ preCtxState ]

| preCtxState | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| initStateIdxToState | 307 | 323 | 340 | 359 | 378 | 398 | 419 | 442 | 466 | 491 | 517 | 544 | 574 | 604 | 637 | 671 |
| preCtxState | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initStateIdxToState | 707 | 745 | 784 | 826 | 871 | 917 | 966 | 1018 | 1073 | 1130 | 1191 | 1254 | 1321 | 1392 | 1467 | 1545 |
| preCtxState | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initStateIdxToState | 1628 | 1715 | 1807 | 1903 | 2005 | 2112 | 2226 | 2345 | 2470 | 2602 | 2741 | 2888 | 3043 | 3206 | 3377 | 3558 |
| preCtxState | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initStateIdxToState | 3748 | 3949 | 4160 | 4383 | 4617 | 4864 | 5124 | 5399 | 5687 | 5992 | 6312 | 6650 | 7006 | 7381 | 7775 | 8192 |
| preCtxState | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initStateIdxToState | 8192 | 8608 | 9002 | 9377 | 9733 | 10071 | 10391 | 10696 | 10984 | 11259 | 11519 | 11766 | 12000 | 12223 | 12434 | 12635 |
| preCtxState | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initStateIdxToState | 12825 | 13006 | 13177 | 13340 | 13495 | 13642 | 13781 | 13913 | 14038 | 14157 | 14271 | 14378 | 14480 | 14576 | 14668 | 14755 |
| preCtxState | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initStateIdxToState | 14838 | 14916 | 14991 | 15062 | 15129 | 15192 | 15253 | 15310 | 15365 | 15417 | 15466 | 15512 | 15557 | 15599 | 15638 | 15676 |
| preCtxState | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initStateIdxToState | 15712 | 15746 | 15779 | 15809 | 15839 | 15866 | 15892 | 15917 | 15941 | 15964 | 15985 | 16005 | 16024 | 16043 | 16060 | 16076 |