| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0192 |
| --- | --- |

| Title: | Non-CE3: MPM flag first signaling for intra luma mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Yao, J.-Q. Zhu, W.-T. Cai, K. Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com zhujianqing@cn.fujitsu.com caiwenting@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

In the current VVC draft [1], for intra luma mode coding, MRL (multiple reference line intra prediction) index and ISP (intra sub-partitions) flag are first signaled even if the block mode is remainder mode (non MPM mode). Since MRL and ISP can only be enabled on MPM mode, this document propose to clean-up the draft syntax by first signal MPM flag. The results reportedly show an average luma BD rate performance of (0.01%, −0.04%, 0.00%) for (Y, Cb, Cr) AI, (0.00%, −0.13%, −0.03%) for (Y, Cb, Cr) RA without increase of encoding running time over VTM-5.0.

Introduction

In the current VVC signaling of intra luma mode coding, as illustrated in Figure 1, MRL index and ISP flag are first signaled (ISP flag is inferred not be used if MRL is used). Then MPM flag is signaled when both MRL index and ISP flag are zero. If MPM flag is non-zero, not planar flag and MPM index are signaled indicating the mode to be used from MPM list. Otherwise if MPM flag is zero, MPM remainder is signaled.

Considering a block is in non MPM mode, MRL index and ISP flag are still signaled, although they are not allowed. For the purpose of draft syntax clean-up. This document propose to first signal MPM flag before MRL index.

Figure 1. VVC signaling of intra luma mode coding

Proposed syntax clean-up

The proposed signaling of luma intra mode coding is shown in Figure 2. MPM flag is signaled at first before MRL index. The rest syntax signaling are not changed. MRL index and ISP flag are only signaled when MPM flag is non zero. In other words, if a block has non MPM mode (MPM flag is zero), signaling of MRL index and ISP flag will be skipped.

The draft syntax changes of the proposed signaling is shown in Table 1 as highlighted in yellow. The proposed draft syntax is not only shorter but also logically simpler than that of the current VVC.

Figure 2. Proposed signaling of intra luma mode coding

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| … |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |
| … |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| … |  |
| } |  |
| … |  |
| } |  |

Table 1. Proposed draft syntax of luma intra mode coding

Results

The proposed signaling is implemented upon VTM-5.0 and is evaluated for all-intra and random-access configuration according to the common test conditions in JVET-N1010 [2].

All syntax context modelling and initial values are identical to VTM-5.0. The result of the proposed syntax can get better if redo initial CABAC value training. Encoding time is not reliable according to CPU resources.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 104% | 99% |
| Class A2 | 0.00% | -0.07% | -0.03% | 107% | 99% |
| Class B | 0.01% | -0.06% | 0.00% | 103% | 100% |
| Class C | 0.01% | -0.09% | 0.00% | 103% | 101% |
| Class E | 0.01% | 0.06% | 0.01% | 100% | 99% |
| Overall | 0.01% | -0.04% | 0.00% | 103% | 100% |
| Class D | 0.00% | 0.24% | 0.01% | 109% | 100% |
| Class F | 0.02% | 0.04% | 0.06% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.06% | 102% | 100% |
| Class A2 | 0.01% | 0.00% | 0.07% | 103% | 100% |
| Class B | 0.00% | -0.14% | -0.06% | 105% | 101% |
| Class C | 0.00% | -0.20% | -0.14% | 119% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.03% | 107% | 100% |
| Class D | 0.06% | -0.02% | 0.14% | 124% | 101% |
| Class F | 0.00% | 0.00% | 0.07% | 117% | 100% |

Table 2. Proposed method over VTM-5.0

Conclusions

This contribution presents to clean-up draft syntax of luma intra mode coding by first signaling MPM flag. It shorten the draft syntax of luma intra mode coding with no performance loss and complexity increase. It is proposed to adopt this contribution.

Acknowledgments

The contribution has been kindly cross-checked by Qualcomm. The cross-check results are available in the contribution JVET-O0768.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, May 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).