| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0193-v2 |
| --- | --- |

| Title: | CE7-related: Remove transform depth in cbf coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes to remove transform depth in the context modeling of tu_cbf_luma and tu_cbf_cb. It is reported that this change results in 0.00%/0.00%/0.00%, 0.00%/-0.03%/-0.03% and 0.03%/0.17%/0.02% Y/Cb/Cr BD-rates in AI, RA and LB configuration, respectively. Together with CE7-4.1, the simplification on cbf coding results in 0.04%/-0.04%/0.00%, 0.03%/-0.19%/-0.09% and -0.01%/0.07%/0.32% Y/Cb/Cr BD-rates in AI, RA and LB configuration, respectively

Introduction

In HEVC, Residual Quad-tree (RQT) creates transform units (TU) with multiple transform depths, and the cbf context modeling is based on transform depth of TUs.

In VVC draft 5, the transform depth can be only 0 or 1. For tu_cbf_luma of coding unit not using ISP or BDPCM, the context modeling is as follows

ctxInc = trDepth = = 0  ?  1  :  0                           (1)

For tu_cbf_cb, the context modeling is as follows:

ctxInc = trDepth = = 0  ?  0  :  1                           (2)

They depend on trDepth in a similar way, but it could be better if they are in a unified expression.

Proposed method

As the first aspect of the proposal, it is suggested to unify the context modeling, as follows

ctxInc = trDepth                                            (3)

This does not influence coding performance, if the CABAC initialization table is changed accordingly.

As the second aspect of the proposal, since RQT is not included in VVC, the transform-depth based context modeling might be obsolete in VVC. Therefore, the coding performance of context modeling without using transform depth is tested. In this test, only one context (corresponding to trDepth 0) is used for non-ISP and non-BDPCM case, i.e.,

ctxInc = 0                                                 (4)

Experiment results

The proposed methods have been implemented on top of VTM5.0 [1]. The common test conditions [2] are used for evaluation.

Removing transform depth in cbf coding leads to 0.00%, 0.00% and 0.03% luma BD-rate in AI, RA and LB, as listed in Table 1.

Since CE7-4.1 simplifies the tu_cbf_luma context modeling for ISP CU, a combined test with CE7-4.1 was made. Together with CE7-4.1, only one context is used for cbf coding of non-BDPCM CU, and the coding performance is 0.04%, 0.03% and -0.01% luma BD-rate in AI, RA and LB, as listed in Table 2.

Table 1 Use one context model for tu_cbf_luma and tu_cbf_cb for non-ISP/BDPCM CU

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 106% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.11% | 101% | 101% |
| Class A2 | 0.06% | -0.08% | 0.04% | 101% | 101% |
| Class B | 0.00% | 0.03% | -0.04% | 101% | 101% |
| Class C | -0.04% | 0.00% | -0.15% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.03% | 101% | 102% |
| Class D | 0.02% | 0.04% | 0.00% | 101% | 104% |
| Class F | -0.01% | -0.08% | -0.06% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.35% | 0.29% | 100% | 104% |
| Class C | 0.01% | -0.22% | 0.00% | 101% | 103% |
| Class E | 0.08% | 0.40% | -0.43% | 100% | 102% |
| Overall | 0.03% | 0.17% | 0.02% | 101% | 103% |
| Class D | 0.04% | 0.56% | -0.17% | 100% | 101% |
| Class F | -0.02% | -0.40% | 0.20% | 100% | 102% |

Note: CTC AI results are identical, because transform depth is always 0 for non-ISP/BDPCM CU in AI. Decoding time may not be accurate.

Table 2 Proposed method + CE7-4.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.02% | 100% | 100% |
| Class A2 | 0.04% | -0.01% | -0.03% | 100% | 100% |
| Class B | 0.04% | -0.03% | -0.04% | 100% | 101% |
| Class C | 0.03% | -0.10% | 0.01% | 101% | 105% |
| Class E | 0.06% | -0.03% | 0.05% | 100% | 103% |
| Overall | 0.04% | -0.04% | 0.00% | 100% | 102% |
| Class D | 0.00% | -0.08% | 0.00% | 101% | 106% |
| Class F | 0.06% | -0.03% | 0.03% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.39% | -0.01% | 101% | 101% |
| Class A2 | 0.08% | -0.25% | -0.02% | 100% | 101% |
| Class B | 0.02% | -0.05% | -0.14% | 100% | 101% |
| Class C | -0.01% | -0.15% | -0.13% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.19% | -0.09% | 100% | 102% |
| Class D | 0.03% | 0.05% | -0.02% | 100% | 103% |
| Class F | 0.02% | -0.02% | 0.17% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.60% | 0.44% | 101% | 104% |
| Class C | 0.02% | -0.15% | -0.17% | 100% | 101% |
| Class E | -0.08% | -0.54% | 0.76% | 100% | 102% |
| Overall | -0.01% | 0.07% | 0.32% | 100% | 103% |
| Class D | -0.01% | 0.15% | 0.92% | 100% | 103% |
| Class F | 0.08% | -0.12% | 0.48% | 101% | 104% |

Conclusions

In this proposal, it is propose to unify and clean up the context modeling of tu_cbf_luma and tu_cbf_cb.

It is suggested to adopt the proposed simplification into VVC.

Draft text for Test 1

- 9.5.4.2.5 Derivation process of ctxInc for the syntax element tu_cbf_luma

Inputs to this process are the variable binIdx, the colour component index cIdx, the binary logarithm of the transform block width log2TbWidth and the transform block height log2TbHeight and the current transform tree depth trDepth.

Output of this process is the variable ctxInc.

The variable ctxInc is derived as follows:

- If IntraSubpartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

ctxInc = trDepth = = 0  ?  1  :  0		(9-34)

ctxInc =  0		(9-34)

Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_cbf_cb[ ][ ][ ] | trDepth = = 0 ? 0 : 1 0 | na | na | na | na | na |

References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).