| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0195-v1 |
| --- | --- |

| Title: | Non-CE6: Simplification on LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes to simplify LFNST from three aspects: 1) for 4xN and Nx4 TU (N>8), LFNST is applied to only the top-left 4x4 sub-block; 2) limit the sum of non-zero coefficient check in LFNST index parsing to only the top-left 2x2 coefficients; 3) reduce four transform sets to three transform sets, which saves 25% memory for storing LFNST matrices, i.e., 2KB.

It is reported that aspect 1 results in 0.02%/0.01%/0.03% Y/Cb/Cr BD-rates in AI configuration. Aspect 1 and 2 together result in 0.03%/-0.20%/-0.12% and 0.03%/-0.12%/-0.07% Y/Cb/Cr BD-rates in AI and RA configuration, respectively. The three aspects together result in 0.10%/-0.15%/-0.05% and 0.04%/-0.25%/-0.17% Y/Cb/Cr BD-rates in AI and RA configuration, respectively.

Proposed method

This proposal studies the LFNST algorithm in VTM5.0 and simplifies it from three aspects.

Aspect 1: unification on the number of LFNST blocks in TBs

In VTM5, LFNST is applied to two 4x4 blocks in a transform block of size 4xN and Nx4 (N>8), and is applied to one 4x4 block for the rest of the transform block sizes.

It is propose to always transform one block (the top left) for any size of TB. There is 0.02% AI coding loss from this unification, and the spec can be simplified.

Aspect 2: simplify the LFNST index parsing by checking top-left 2x2 coefficients in a TB

In VTM5, the parsing of LFNST index is based on the sum of non-zero coefficients of TBs in a CU. If the sum is not greater than a threshold, the LFNST is not allowed for the CU.

It is studied that checking only the top-left 2x2 coefficients (instead of all the coefficients) brings no coding loss.

Aspect 3: remove LFNST transform matrices related to transform set 2

In VTM5, there are 4 LFNST transform sets, and each of them has two 16x48 and two 16x16 transform matrices. In total, the memory for the transform matrices is 8 x 16 x (48+16) = 8KB

It is proposed to use 3 transform sets. Transform set 2 (thus the matrices related to this set) is removed. Accordingly, the mapping table from predModeIntra to lfnstTrSetIdx is changed to Table 1. LFNST transform matrices are not modified. Because of the reduction of transform matrices from 4 sets to 3 sets, 25% LFNST matrix memory (i.e., 2KB ) is saved.

Table 1 Mapping table with three transform sets

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 18 | 1 |
| 19 <= predModeIntra <= 49 | 3 |
| 50 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

According to the new mapping table, directional prediction modes with reference samples from both left and top sides uses transform set 3, and those with reference samples from only one side uses transform set 1, as illustrated in Fig. 1. Re-training the transform matrices in transform set 1 and 3 may improve the coding performance, but has not been done yet.

Fig. 1 Transform set mapping with 4 sets in VTM5 (left) and the proposed method with 3 sets (right)

Experiment results

The proposed methods have been implemented on top of VTM5.0 [1]. The common test conditions [2] are used for evaluation. The coding performance of different aspects are summarized in Table 2. Detailed results are provided in Table 3 to Table 5.

Table 2 Summary of coding results

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Test 1: aspect 1 | 0.02% | 0.01% | 0.03% | 100% | 101% |
| Test 2: aspect 1+2 | 0.03% | -0.20% | -0.12% | 101% | 102% |
| Test 3: aspect 1+2+3 | 0.10% | -0.15% | -0.05% | 100% | 101% |
| RA | Y | U | V | EncT | DecT |
| Test 1: aspect 1 |  |  |  |  |  |
| Test 2: aspect 1+2 | 0.03% | -0.12% | -0.07% | 101% | 101% |
| Test 3: aspect 1+2+3 | 0.04% | -0.25% | -0.17% | 101% | 101% |

Table 3 Coding performance of Aspect 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | -0.03% | 0.01% | 100% | 102% |
| Class B | 0.02% | -0.03% | 0.05% | 100% | 100% |
| Class C | 0.04% | 0.02% | 0.05% | 101% | 102% |
| Class E | 0.02% | 0.06% | -0.03% | 100% | 102% |
| Overall | 0.02% | 0.01% | 0.03% | 100% | 101% |
| Class D | 0.03% | 0.01% | 0.00% | 100% | 103% |
| Class F | -0.01% | 0.03% | 0.13% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 3 Coding performance of Aspect 1 + 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.22% | -0.23% | 101% | 100% |
| Class A2 | 0.04% | -0.25% | -0.18% | 100% | 101% |
| Class B | 0.03% | -0.23% | -0.09% | 101% | 100% |
| Class C | 0.02% | -0.11% | -0.06% | 101% | 104% |
| Class E | 0.04% | -0.20% | -0.06% | 101% | 104% |
| Overall | 0.03% | -0.20% | -0.12% | 101% | 102% |
| Class D | 0.01% | 0.08% | -0.04% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.03% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.35% | 0.09% | 101% | 101% |
| Class A2 | 0.07% | 0.10% | -0.01% | 101% | 101% |
| Class B | 0.03% | -0.12% | -0.12% | 101% | 101% |
| Class C | 0.00% | -0.11% | -0.16% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.12% | -0.07% | 101% | 101% |
| Class D | -0.01% | -0.04% | 0.17% | 101% | 102% |
| Class F | 0.03% | 0.09% | 0.01% | 101% | 102% |

Table 3 Coding performance of Aspect 1 + 2 + 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.19% | -0.25% | 100% | 100% |
| Class A2 | 0.10% | -0.20% | -0.11% | 100% | 100% |
| Class B | 0.08% | -0.29% | -0.22% | 100% | 101% |
| Class C | 0.08% | -0.06% | 0.16% | 100% | 103% |
| Class E | 0.12% | 0.06% | 0.20% | 101% | 104% |
| Overall | 0.10% | -0.15% | -0.05% | 100% | 101% |
| Class D | 0.02% | 0.12% | -0.03% | 101% | 104% |
| Class F | 0.03% | 0.11% | -0.01% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.22% | 0.12% | 101% | 101% |
| Class A2 | 0.04% | -0.14% | -0.09% | 101% | 101% |
| Class B | 0.06% | -0.33% | -0.39% | 101% | 101% |
| Class C | 0.00% | -0.28% | -0.19% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.25% | -0.17% | 101% | 101% |
| Class D | 0.05% | -0.14% | -0.21% | 100% | 102% |
| Class F | 0.04% | 0.07% | 0.20% | 101% | 102% |

Conclusions

In this proposal, it is propose to unify and simplify LFNST from three aspects. Aspect 1 unifies the number of 4x4 LFNST blocks as one, with 0.02% loss in AI. Aspect 2 reduces operations in LFNST index parsing, with no loss in AI. Aspect 3 reduces 2KB matrix memory, with 0.07% loss in AI. The coding loss of three aspects together is 0.10%/-0.15%/-0.05% in AI and 0.04%/-0.25%/-0.17% in RA.

It is suggested to adopt the proposed method into VVC.

Draft text

For aspect 1:

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-953)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-954)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-955)

nLfnstSize = 1  <<  log2LfnstSize		(8-956)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-957)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-958)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-959)

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-960)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-961)

u[ x ] = d[ xC ][ yC ] 			(8-962)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(8-963)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(8-964)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

For aspect 2:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |  |
| numSigCoeff++ |  |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| } |  |  |
| … |  |  |
| } |  |
| } |  |
| numSigCoeff += sig_coeff_flag[ 0 ][ 0 ] + sig_coeff_flag[ 0 ][ 1 ] + sig_coeff_flag[ 1 ][ 0 ] + sig_coeff_flag[ 1 ][ 1 ] |  |
| } |  |  |

For aspect 3:

Table 8-18 – Specification of lfnstTrSetIdx

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 18 | 1 |
| 19 <= predModeIntra <= 49 | 3 |
| 50 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

Accordingly, lowFreqTransMatrix[ m ][ n ] tables related to lfnstTrSetIdx 2, approximately 3 pages, can be removed from the spec.

References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).