| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0196-v1 |
| --- | --- |

| Title: | CE2-related: CCLM for dual tree with 32x32 latency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes to conditionally allow CCLM for chroma CUs in dual tree, by which the luma-chroma latency is reduced from 64x64 to 32x32. The CCLM allowing condition is based on the split mode of 64x64 luma coding tree node and the split mode of 32x32 chroma tree node and that of 32x16 chroma tree node, which is of low complexity and can be determined in parsing of coding_tree(). It is reported that the coding performance is 0.07%/1.06%/1.07% and 0.02%/0.39%/0.35% Y/Cb/Cr BD-rates in AI and RA configurations, respectively.

Introduction

Reducing luma-chroma latency for separate tree is under study in CE2. This proposal gives another solution that guarantees CCLM of a 16x16 chroma block can be started after reconstruction of a 32x32 luma block. The CCLM allowing condition is based on the split mode of 64x64 luma coding tree node and the split mode of 32x32 chroma tree node and that of 32x16 chroma tree node, and thus is of low complexity. The proposed method introduces less coding loss than CE2-3.

Proposed method

The proposed method allows CCLM for a chroma CU when the corresponding 64x64 luma region can be reconstructed in 32x32 pipeline with Quad-tree processing order (QT order) and the corresponding 32x32 chroma region can also be reconstructed in 16x16 pipeline with QT order.

There are two cases that a 64x64 luma region can be reconstructed in 32x32 pipeline with QT order, as shown in Fig. 1.

- If a 64x64 luma coding tree node does not split and the corresponding 64x64 luma CU does not use ISP, the 64x64 luma CU can be tiled into four 32x32 luma sub-blocks, and the reconstruction of the luma sub-blocks can be arranged in the QT order.

- If a 64x64 luma coding tree node is partitioned with a QT split, the four 32x32 child nodes are reconstructed in the QT order.

Similarly, there are four cases that a 32x32 / 32x16 chroma region can be reconstructed in 16x16 pipeline with QT order.

- If a 32x32 chroma coding tree node does not split, it can be tiled into four 16x16 chroma sub-blocks, and processed in QT order.

- If a 32x32 chroma coding tree node is partitioned with a QT split, the four 16x16 child nodes are reconstructed in the QT order.

- If the 32x32 chroma coding tree node is partitioned with a horizontal BT (HBT) split, and the 32x16 node does not split, the 32x16 CU can be tiled into two 16x16 chroma blocks and reconstructed in QT order.

- If the 32x32 chroma coding tree node is partitioned with a HBT split, and the 32x16 node is partitioned with a vertical BT (VBT), the 16x16 child node are reconstructed in the QT order.

Fig. 1 Illustration of CCLM allowing condition

Therefore, when the 64x64 luma coding tree node is partitioned with Not Split (and ISP is not used for the 64x64 CU) or QT, the chroma CUs in 32x32 / 32x16 chroma coding tree node are allowed to use CCLM in the following way:

- if the 32x32 chroma node is partitioned with Not Split or QT split, all chroma CUs in the 32x32 node can use CCLM

- if the 32x32 chroma node is partitioned with HBT, and the 32x16 child node does not split or uses VBT split, all chroma CUs in the 32x16 chroma node can use CCLM.

In other luma and chroma coding tree split conditions, CCLM is not allowed for chroma CU.

Note: when CTU size is equal to or smaller than 32x32, CCLM is allowed for each chroma CU.

As the CCLM allowing conditions are based on coding tree split mode, it can be determined in parsing coding_tree(). Drafted spec text is provided below.

Draft text

- 7.3.7.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 1 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 2 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else { |  |
| CclmAllowLumaCond = 1 |  |
| CclmAllowChromaCond = 1 |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 0, 0, 0, 0, 0, SINGLE_TREE ) |  |
| } |  |
| } |  |

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| dual_tree_implicit_qt_split( x0, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x0, y1, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y1, cbSize / 2, cqtDepth + 1 ) |  |
| } else { |  |
| CclmAllowLumaCond = cbSize = = 64? 0 : 1 |  |
| CclmAllowChromaCond = cbSize = = 64? 0 : 1 |  |
| coding_tree( x0, y0, cbSize, cbSize, 1, cbSubdiv, cqtDepth, 0, 0, 0, DUAL_TREE_LUMA ) |  |
| coding_tree( x0, y0, cbSize, cbSize, 0, cbSubdiv, cqtDepth, 0, 0, 0, DUAL_TREE_CHROMA ) |  |
| } |  |
| } |  |

- 7.3.7.4 Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOn, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| … |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |
| split_qt_flag | ae(v) |
| if( !split_qt_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  |
| … |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  |
| … |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  |
| … |  |
| } else { /* SPLIT_TT_HOR */ |  |
| … |  |
| } |  |
| if( cbWidth = = 64 && cbHeight = = 32 && treeType = = DUAL_TREE_CHROMA && MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER && CclmAllowLumaCond ) |  |
| CclmAllowChromaCond = 1 |  |
| } else { |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| coding_tree( x0, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 0, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_tree( x0, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 2, treeType ) |  |
| if( y1 < pic_height_in_luma_samples && x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 3, treeType ) |  |
| if( cbWidth = = 64 && cbHeight = = 64 && treeType = = DUAL_TREE_LUMA ) |  |
| CclmAllowLumaCond = 1 |  |
| if( cbWidth = = 64 && cbHeight = = 64 && treeType = = DUAL_TREE_CHROMA && CclmAllowLumaCond ) |  |
| CclmAllowChromaCond = 1 |  |
| } else |  |
| if( cbWidth = = 64 && (cbHeight = = 64 ¦¦ cbHeight = = 32) && treeType = = DUAL_TREE_CHROMA && CclmAllowLumaCond ) |  |
| CclmAllowChromaCond = 1 |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| if( cbWidth = = 64 && cbHeight = = 64 && treeType = = DUAL_TREE_LUMA && !intra_subpartitions_mode_flag[ x0 ][ y0 ] ) |  |
| CclmAllowLumaCond = 1 |  |
| } |  |

- 8.4.4 Derivation process for chroma intra prediction mode

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1 and CclmAllowChromaCond is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Experiment results

The proposed methods have been implemented on top of VTM5.0 [1]. The common test conditions [2] are used for evaluation.

The proposed CCLM allowing method leads to 0.07% / 1.06% / 1.07% and 0.02% / 0.39% / 0.35% luma BD-rate in AI and RA, as listed in Table 1.

Table 1 Coding performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 2.18% | 2.33% | 98% | 101% |
| Class A2 | 0.08% | 1.36% | 0.56% | 98% | 100% |
| Class B | 0.06% | 1.07% | 1.40% | 99% | 100% |
| Class C | 0.03% | 0.30% | 0.32% | 100% | 103% |
| Class E | 0.02% | 0.67% | 0.80% | 100% | 102% |
| Overall | 0.07% | 1.06% | 1.07% | 99% | 101% |
| Class D | 0.01% | 0.34% | 0.15% | 100% | 103% |
| Class F | 0.06% | 0.57% | 0.54% | 99% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.49% | 0.58% | 100% | 101% |
| Class A2 | 0.05% | 0.68% | 0.27% | 100% | 101% |
| Class B | 0.02% | 0.49% | 0.52% | 100% | 101% |
| Class C | -0.02% | -0.01% | 0.02% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.39% | 0.35% | 100% | 102% |
| Class D | 0.02% | 0.10% | -0.06% | 100% | 103% |
| Class F | 0.02% | 0.29% | 0.41% | 100% | 102% |

Conclusions

CCLM allowing condition that guarantees 32x32 luma-chroma latency is proposed. The condition check can be simply done in coding_tree() parsing. Compared with CE2-3. the BD-rate loss is reduced to 0.07% / 1.06% / 1.07%.

It is suggested to adopt the proposed method into VVC.

References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).