| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0197 |
| --- | --- |

| Title: | Non-CE3: Cleanup for ISP flag and MRL index signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Na Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, Intra Subblock Partitioning and Multiple Reference Line are disallowed for the non-Most Probable Modes. However, the on/off control flag of Intra Subblock Partitioning and the reference line index are still signalled for such intra prediction methods, which is unnecessary. To tackle this problem, this contribution proposes to signal the on/off control flag of Intra Subblock Partitioning and the reference line index only when Most Probable Mode is used. It is reported that the proposed method can obtain 0.01% luma BD-rate gain for the AI and RA configurations on top of VTM-5.0 without encoding and decoding run-time changes.

# Introduction

Intra Subblock Partitioning (ISP) and Multiple Reference Line (MRL) have been adopted into VVC [1], and the ISP flag and the reference index in MRL are further signalled to indicate whether ISP and non-adjacent reference line are employed by a Coding Unit (CU), respectively. For the intra prediction modes that are not included in the Most Probable Mode (MPM) list, ISP and MRL are disallowed, and the ISP flag and the MRL index are always equal to zero. However, the ISP flag and the MRL index are still signalled for such intra prediction modes as shown in Table 1, which is unreasonable.

Table 1: Coding unit syntax.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_mip_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |

# Proposed method

To tackle the abovementioned problem, this contribution proposes to signal the MPM flag before the ISP flag and the MRL index and signal the ISP flag and the MRL index only for the intra prediction modes that are from the MPM list (i.e., when MPM flag is true). The draft change is shown in Table 2.

The deleted parts are highlighted in grey strikethrough, and the newly added parts are highlighted in yellow.

Table 2: Proposed change of coding unit syntax.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_mip_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |

# Experimental results

The proposed method was implemented on top of the reference software VTM-5.0 [2], and simulations were performed following the common test conditions [3]. The results are shown in Table 3~Table 5. More detailed results can be found in the attached spreadsheets.

Table 3: Coding performance of proposed method in all intra configuration.

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | -0.02% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | -0.03% | 100% | 101% |
| Class B | 0.00% | -0.07% | -0.03% | 99% | 101% |
| Class C | -0.01% | -0.02% | 0.10% | 101% | 98% |
| Class E | -0.01% | 0.07% | 0.03% | 100% | 101% |
| Overall | -0.01% | 0.00% | 0.01% | 100% | 100% |
| Class D | -0.01% | 0.17% | 0.00% | 99% | 100% |
| Class F | -0.01% | -0.02% | 0.01% | 101% | 101% |

Table 4: Coding performance of proposed method in random access configuration.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.18% | 0.09% | 100% | 100% |
| Class A2 | 0.01% | 0.13% | -0.03% | 100% | 100% |
| Class B | 0.01% | 0.00% | -0.04% | 100% | 99% |
| Class C | -0.05% | -0.27% | -0.24% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.08% | -0.07% | 100% | 100% |
| Class D | 0.02% | 0.16% | 0.03% | 100% | 100% |
| Class F | 0.02% | -0.02% | 0.19% | 100% | 100% |

Table 5: Coding performance of proposed method in low delay B configuration.

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.21% | 0.18% | 102% | 100% |
| Class C | 0.03% | 0.12% | 0.10% | 100% | 100% |
| Class E | -0.11% | -1.02% | 0.71% | 99% | 99% |
| Overall | -0.02% | -0.13% | 0.29% | 101% | 100% |
| Class D | -0.02% | 0.04% | 0.37% | 99% | 98% |
| Class F | -0.01% | 0.21% | -0.15% | 100% | 101% |

# Conclusions

This contribution proposes cleanup for signalling of the ISP flag and the MRL index. It is recommended to adopt the proposed method into VVC.

# References

- “Versatile Video Coding (Draft 5)”, B. Bross, J. Chen, S. Liu, Doc. JVET-N1001, Mar. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).