| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0198 |
| --- | --- |

| Title: | AHG12: On brick partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Adarsh Ramasubramonian Yao-Jen Chang Vadim Seregin Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add vertical brick partitioning support to tile brick partitioning. A brick partitioning direction flag is added at tile level to signal horizontal or vertical brick partitioning.

# Problem Statement

The current picture partitioning scheme does not allow certain 360 video region wise packing (RWP) schemes such as shown below [2].

Figure 1: RWP for viewport based cubemap.

The packing scheme on the right side of Figure 1 can be achieved by introducing vertical brick partitioning scheme to VVC. This is illustrated below:

Figure 2: Partitioning scheme using horizontal and vertical bricks for RWP for cubemap.

Similar to horizontal bricks that span across the entire tile horizontally, vertical bricks span the tile vertically.

# Proposal

Proposed brick split direction signalling is shown below [1].

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| brick_split_dir_flag | u(1) |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( !brick_split_dir_flag ) |  |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| else |  |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_width_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_columns_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_columns_minus1[ i ]; j++ ) |  |
| brick_column_width_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| …. |  |

brick_split_dir_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more vertical bricks. brick_split_dir_flag[ i ] equal to 0 specifies that the i-th tile is divided into two or more horizontal bricks. When not present, the value of brick_split_dir_flag[ i ] is inferred to be equal to 0.

uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal or vertical brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ] or brick_width_minus1[ i ] for horizontal and vertical bricks, respectively. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal or vertical brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ] or num_brick_columns_minus1[ i ] and a list of syntax elements brick_column_width_minus1[ i ][ j ] for horizontal and vertical bricks, respectively. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1.

brick_width_minus1[ i ] plus 1 specifies the width of the brick columns excluding the right-most brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_width_minus1 shall be in the range of 0 to colWidth[ i ] − 2, inclusive. When not present, the value of brick_width_minus1[ i ] is inferred to be equal to colWidth[ i ] – 1.

num_brick_columns_minus1[ i ] plus 1 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_columns_minus1[ i ] shall be in the range of 1 to colWidth[ i ] − 1, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_columns_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_columns_minus1[ i ] is inferred as specified in 6.5.1.

brick_column_width_minus1[ i ][ j ] plus 1 specifies the width of the j-th brick in the i-th tile in units of CTBs when uniform_tile_spacing_flag is equal to 0.

### 6.5.1 CTB raster scanning, tile scanning, and brick scanning process

………

The variable NumBricksInPic, specifying the number of bricks in a picture referring to the PPS, and the lists BrickColBd[ brickIdx ], BrickRowBd[ brickIdx ], BrickWidth[ brickIdx ], and BrickHeight[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the locations of the vertical brick boundaries in units of CTBs, the locations of the horizontal brick boundaries in units of CTBs, the widths of the bricks in units of CTBs, and the heights of bricks in units of CTBs, are derived, and for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred, as follows:

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )
	if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]	(6-5)
		brickIdx++
	} else {
		if( !brick_split_dir_flag ) {
			if( uniform_brick_spacing_flag[ i ] ) {
				remainingHeightInCtbsY = RowHeight[ tileY ]
				j = 0
				while( remainingHeightInCtbsY > ( brick_height_minus1[ i ] + 1 ) ) {
					rowHeight2[ j++ ] = brick_height_minus1[ i ] + 1
					remainingHeightInCtbsY  −=  ( brick_height_minus1[ i ] + 1 )
				}
				rowHeight2[ j ] = remainingHeightInCtbsY
				num_brick_rows_minus1[ i ] = j
			} else {
				rowHeight2[ num_brick_rows_minus1[ i ] ] = RowHeight[ tileY ]
				for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) {
					rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
					rowHeight2[ num_brick_rows_minus1[ i ] ]  −=  rowHeight2[ j ]
				}
			}
			for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus1[ i ]; j++ )
				rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
			for( j = 0; j < num_brick_rows_minus1[ i ] + 1; j++ ) {
				BrickColBd[ brickIdx ] = tileColBd[ tileX ]
				BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
				BrickWidth[ brickIdx ] = colWidth[ tileX ]
				BrickHeight[ brickIdx ] = rowHeight2[ j ]
				brickIdx++
			}
		} else {
			if( uniform_brick_spacing_flag[ i ] ) {
				remainingWidthInCtbsX = colWidth[ tileX ]
				j = 0
				while( remainingWidthInCtbsX > ( brick_width_minus1[ i ] + 1 ) ) {
					colWidth2[ j++ ] = brick_width_minus1[ i ] + 1
					remainingWidthInCtbsX  −=  ( brick_width_minus1[ i ] + 1 )
				}
				colWidth2[ j ] = remainingWidthInCtbsX
				num_brick_columns_minus1[ i ] = j
			} else {
				colWidth2[ num_brick_columns_minus1[ i ] ] = colWidth[ tileX ]
				for( j = 0; j < num_brick_columns_minus1[ i ]; j++ ) {
					colWidth2[ j ] = brick_column_width_minus1[ i ][ j ]+ 1
					colWidth2[ num_brick_columns_minus1[ i ] ]  −=  colWidth2[ j ]
				}
			}
			for( colBd2[ 0 ] = 0, j = 0; j  <=  num_brick_columns_minus1[ i ]; j++ )
				colBd2[ j + 1 ] = colBd2[ j ] + colWidth2[ j ]
			for( j = 0; j < num_brick_columns_minus1[ i ] + 1; j++ ) {
				BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
				BrickColBd[ brickIdx ] = tileColBd[ tileX ] + colBd2[ j ]
				BrickWidth[ brickIdx ] = colWidth2[ tileX ]
				BrickHeight[ brickIdx ] = rowHeight[ j ]
				brickIdx++
			}
		}
	}
}
NumBricksInPic = brickIdx

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- R. Skupin, Y. Sanchez, Y.-K. Wang, M. M. Hannuksela, J. Boyce, M. Wien, “Standardization Status of 360 degree Video Coding and Delivery”, VCIP, 2017.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).