| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0200 |
| --- | --- |

| Title: | CE8-related: RDPCM intra mode mapping, maximum RDPCM block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Yu Han Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com yuhan@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to assign horizontal/vertical RDPCM directions to horizontal/vertical intra prediction modes instead of planar intra mode. The simulations results show Class F: -0.12% AI, -0.09% RA, 0.02% LB BD-Rate, and Class TGM: -0.18% AI, -0.18% RA, -0.04% LB BD-Rate, versus the VTM-5.0 anchor under common test conditions. The maximum coding block width/height that supports RDPCM is aligned to maximum transform skip size signalled in PPS instead of fixed 32 maximum dimension.

# Problem Statement

In current VVC [1], the intra mode assigned to RDPCM coded blocks is planar mode. Assigning horizontal and vertical intra prediction mode for horizontal and vertical RDPCM direction coded blocks, respectively, aligns it with HEVC RDPCM intra mode directions and provides slight improvement in coding efficiency. The mapping used can be expressed as:

intraDir = intra_bdpcm_dir_flag == 1 ? 50 : 18.

In current VVC spec., RDPCM mode uses TS coding and signalled for block sizes less than equal to 32. The maximum TS size is signalled in PPS. It is proposed to align the maximum allowed RDPCM size to maximum TS size. The proposed change is highlighted in syntax table below.

#### 7.3.7.5 Coding unit syntax

| …. |  |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 MaxTsSize && cbHeight <= 32 MaxTsSize ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag [ x0 ][ y0 ] | ae(v) |
| else { |  |

# Results

The proposed scheme is implemented on top of VTM-5.0 software [2]. The simulations were carried out under the JVET common test conditions [3].

The test results for proposed BDPCM scheme (1) under JVET common test conditions with TGM class is tabulated in Table 1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | 0.04% | -0.08% | 100% | 92% |
| Class TGM | -0.18% | -0.06% | -0.07% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | 0.08% | 0.10% | 101% | 101% |
| Class TGM | -0.18% | -0.02% | -0.13% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.21% | -0.44% | 100% | 98% |
| Class TGM | -0.04% | 0.01% | 0.04% | 100% | 101% |

Table 1: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-5.0.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

- “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).