| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0201 |
| --- | --- |

| Title: | AHG17: On Gradual Random Access (GRA) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Adarsh Ramasubramonian Vadim Seregin Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution describes an update process to recovery point picture when IRAP or GRA_NUT is decoded in between a GRA picture with NoIncorrectPicOutputFlag equal to 1 and its associated recovery point picture in decoding order. In addition, it is proposed to allow signalling of non-negative recovery_poc_cnt values.

# Proposal (Part 1)

An encoder can decide to encode an IRAP picture or start another GRA process after signalling a GRA picture with NoIncorrectPicOutputFlag equal to 1 before its associated recovery point picture in decoding order. In this situation, it is proposed to replace the RpPicOrderCntVal of the decoding process with PicOrderCntVal of the IRAP picture for IRAP case, and with PicOrderCntVal + recovery_poc_cnt, where PicOrderCntVal and recovery_poc_cnt are the PicOrderCntVal and recovery_poc_cnt of the new GRA picture and set NoIncorrectPicOutputFlag equal to 1. Basically, a GRA picture with NoIncorrectPicOutputFlag equal to 1’s RpPicOrderCntVal is valid until an IRAP or a new GRA_NUT or its associated recovery point picture (or a later poc value) is decoded.

In VVC spec. [1],

  - General decoding process

    - General

….

For each GRA picture in the bitstream, the following applies:

–	If the current picture is the first picture in the bitstream in decoding order or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

- If the current picture is before the RpPicOrderCntVal of previous GRA picture in decoding order with NoIncorrectPicOutputFlag equal to 1, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, NoIncorrectPicOutputFlag is set equal to 0.

NOTE – The above operations, for both IRAP pictures and GRA pictuures, are needed for identification of the CVSs in the bitstream.

# Proposal (Part 2)

In VVC spec. [1], the recovery_poc_cnt is a signed value. If the RpPicOrderCntVal of a GRA picture is before the GRA picture in output order, then there would be gaps in picture output. It is proposed to change the recover_poc_cnt value to be non-negative, so that there would be no gaps in output pictures from the recovery picture point and onwards.

#### 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v)ue(v) |
| slice_pic_order_cnt_lsb | u(v) |
| … |  |

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If there is a picture picA that follows the current GRA picture in decoding order in the CVS and that has PicOrderCntVal equal to the PicOrderCntVal of the current GRA picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GRA picture in decoding order. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2  0 to MaxPicOrderCntLsb / 2 − 1, inclusive.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).