| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0206_v1 |
| --- | --- |

| Title: | CE8-related : Unified condition between BDPCM and transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Jungah Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {sunmi.yoo, junghak.nam, jungah.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the current VVC, BDPCM enabling condition depends on only block size (i.e., width <= 32 && height <= 32). Considering that the BDPCM is applied on only the transform-skipped residual blocks and the transform skip is able to be restricted less than 32, it is possible for the current spec text to allow the BDPCM when the transform skip is not enabled. Therefore, in this contribution, a unified enabling condition between the BDPCM and the transform skip is suggested.

Introduction

Currently, the BDPCM is applied on the transform-skipped residual blocks. In other words, when a block is coded with the BDPCM mode, it is also inferred as a TS block so that the TS flag is not transmitted, and the TS residual coding is applied. However, in the current design, the BDPCM enabling condition depends only on block sizes (i.e. width <32 && height <32), which is different from the TS enabling condition. Because of this inconsistency, the TS size restriction syntax element, log2_transform_skip_max_size_minus2 in the picture parameter set, does not work properly – it is allowed for larger blocks to be transform skipped (those blocks are coded as the BDPCM) than the size defined by this syntax element. For example, even if log2_transform_skip_max_size_minus2 is set to 2, which indicates that the transform skip is allowed from 4x4 to 16x16, the TS block larger than 16x16 can be coded since the BDPCM is inferred as the TS block.

Proposed method

In this proposal, a unified enabling condition between the BDPCM and the TS is suggested by adjusting the current BDPCM enabling condition to be dependent on the TS enable flag and maximum allowed TS size. The proposed syntax is shown below:

#### 7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( transform_skip_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| (…) |  |
| } |  |
| } |  |
| } |  |

Conclusion

In conclusion, the unified BDPCM condition, which is aligned with transform skip condition, is proposed. It is recommended for the proposed BDPCM condition to be included in the next version of VVC.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1001, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).