| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0208-v1 |
| --- | --- |

| Title: | Non-CE9: An improvement of early termination for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

An early termination of BDOF (bidirectional optical flow) by calculating SAD (sum of absolute difference) based on 4x4 sample unit of two interpolated pictures and thresholding on two steps of BDOF block units and 4x4 sample unit has been introduced. Although the average of computational complexity can be reduced by it, the worst of computational complexity cannot be reduced. That is a problem from the viewpoint of hardware design. In this contribution, since there is a process of deriving difference by right-shifting two interpolated pictures to 10-bit in the derivation process of BDOF, a method to make the difference for calculating SAD the same as this difference is proposed. Also, SAD calculation for every other line the same as the early termination of DMVR is introduced. As a result, additional computation for early termination has been reduced and the worst of computational complexity has been improved. As an experimental result on CTC, the average of decoding time became 1% faster and the average gains of coding efficiency were -0.01%/-0.02%/-0.02% for Y/Cb/Cr respectively.

Introduction

It may be unnecessary to introduce any additional early termination from the view point of hardware design, because the worst complexity cannot be reduced by it. In this contribution, a method to reduce the worst complexity of BDOF is proposed. There is a following process of BDOF in subsection 8.5.6.4,

For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply. The variables diff[ x ][ y ] is derived as diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 ), where the variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

Since the difference between two prediction samples in 6x6 sample unit has been gotten in the above process, we can use this result for SAD of early termination. In addition, we introduce the SAD calculation for every other line the same as the early termination of DMVR.

Proposal changes of specification

The working draft [1] should be changed by the followings.

- 8.5.6  Decoding process for inter blocks

- 8.5.6.1	General

- If bdofFlag is equal to TRUE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ) Max( 4, bitDepth − 8 ).

- The variables sbDiffThres, bdofBlkDiffThres, and sbSumDiff are derived as follows:

sbDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) )2 * sbWidth*sbHeight 	(8-705)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )64	(8-706)

sbSumDiff =  0		(8-707)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff

= i=03j=01Abs( (predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + 2*j ]>>shift) − 
		(predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + 2*j ]>>shift) ) 	(8-708)

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-709)

sbSumDiff += bdofBlkSumDiff		(8-710)

- The variable sbBdofFlag is derived as follows:

- If sbSumDiff is less than sbDiffThres, sbBdofFlag is set equal to FALSE.

- Otherwise, sbBdofFlag set equal to TRUE.

Experimental results

The proposed method has been implemented on VTM-5.0 software. The simulation was based on VTM configuration JVET-N1010 [2]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.17% | -0.04% | 100% | 99% |
| Class A2 | 0.01% | 0.02% | 0.04% | 100% | 99% |
| Class B | 0.00% | 0.04% | 0.01% | 100% | 99% |
| Class C | -0.03% | -0.02% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.02% | 100% | 99% |
| Class D | 0.04% | -0.03% | -0.14% | 99% | 99% |
| Class F | 0.00% | 0.03% | 0.02% | 100% | 99% |

Conclusion

In this contribution, a speed up method of BDOF was proposed. Since the additional complexity for early termination of BDOF was reduced, the average of decoding times became 1% faster and there was a little gain or no loss of coding efficiency on average. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v8, Geneva, June 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-N1010, Geneva, April. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).