| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0209 |
| --- | --- |

| Title: | Non-CE6: LFNST simplification based on the methods proposed in CE6-2.1a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Mehdi Salehifar Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com junghak.nam@lge.com jaehyun.lim@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the 14th JVET meeting, Low Frequency Non-Separable Transform (LFNST) was adopted and also some LFNST simplification methods are being considered in CE6-2.1. The proposal introduces several configurations/variations of LFNST simplification based on the methods proposed in CE6-2.1 by considering 4 aspects as follows:

(a) Max. 16 LFNST coefficients

(b) Additional zero-out of primary transform coefficients when 4x4 LFNST is applied

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

(d) Change of the region where non-zero coefficients are counted

In CE6-2.1, all of (a), (b), (c), and (d) are incorporated on top of VTM 5.0 LFNST.

In this contribution, several combinations of the four features are tested, of which descriptions and experimental results are summarized as follows:

| Test | Description | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate | EncT | DecT | BD-rate | EncT | DecT |
| CE6-2.1a | (a)+(b)+(c)+(d) | 0.06% | 90% | 99% | 0.02% | 97% | 100% |
| Test 1 | (a) | 0.02% | 100% | 100% | 0.01% | 100% | 100% |
| Test 2 | (c)+(d) | 0.02% | 93% | 98% | 0.01% | 99% | 100% |
| Test 3 | (c) | 0.03% | 94% | 100% | 0.02% | 99% | 100% |
| Test 4 | (a)+(c)+(d) | 0.04% | 92% | 98% | 0.03% | 99% | 100% |
| Test 5 | (a)+(c) | 0.05% | 93% | 100% | 0.02% | 99% | 100% |

The results of CE6-2.1a in the above table are those in its contribution document. Specifically, Test 3 and Test 5 does not require any normative changes relative to VTM 5.0 and Test 1, respectively.

LFNST Simplification Method

(a) Max. 16 LFNST coefficients

In VTM 5.0 LFNST, 4x4 LFNST can be applied to each of two top-left adjacent 4x4 blocks in 4xN or Nx4 TU (N≥16), which ends up generating 32 LFNST coefficients in forward direction. The feature (a) makes LFNST be applied to only one top-left sub-block, from which the number of LFNST coefficients cannot exceed 16.

(b) Additional zero-out of primary transform coefficients when 4x4 LFNST is applied

With the feature (b), output pattern of forward primary transform followed by forward 4x4 LFNST is modified from VTM 5.0 with the additional zero-out of primary transform coefficients that lie outside of top-left region where 4x4 LFNST can be applied, as depicted in Fig. 1.

Figure 1  The additional zero-out of primary transform coefficients when 4x4 LFNST is applied (non-zero coefficients can only reside in the shared region, and the positions with 0 marks indicates the associated zero-out.)

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

In a similar way with the feature (b), the feature (c) enables additional zero-out of primary transform coefficients when 8x8 LFNST is applied, as illustrated in Fig. 2.

Figure 2  The additional zero-out of primary transform coefficients when 8x8 LFNST is applied (non-zero coefficients can only reside in the shared region, and the positions with 0 marks indicates the associated zero-out.)

(d) Change of the region where non-zero coefficients are counted

The zero-out shown in Fig. 1 and Fig. 2 modifies the region where non-zero coefficients cannot exist if LFNST is applied. Therefore, condition for signaling an LFNST index by checking the existence of non-zero coefficients including further zeroed-out region can be changed accordingly. Moreover, with the features (b) or (c), all the region outside of where LFNST coefficients are located must be zeroed out, which entails that the existence checking can be performed easier by checking whether or not the scanning position of last non-zero coefficient exceeds maximum allowable position of LFNST coefficients.

Experimental results

The proposed method has been implemented based on VTM 5.0 reference software according to common test conditions defined in JVET-N1010 [4] and Core Experiment description in JVET-N1026 [5]. The below table shows the overall results briefly for comparison purpose, where the CE6-2.1a results are brought from [3].

| Test | Description | AI |  |  | RA |  |  | RA /w inter-MTS on |  |  | AI 100frs, low-QP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate | EncT | DecT | BD-rate | EncT | DecT | BD-rate | EncT | DecT | BD-rate | EncT | DecT |
| CE6-2.1a | (a)+(b)+(c)+(d) | 0.06% | 90% | 99% | 0.02% | 97% | 100% | 0.02% | 97% | 99% | 0.11% | 83% | 102% |
| Test 1 | (a) | 0.02% | 100% | 100% | 0.01% | 100% | 100% | 0.02% | 100% | 100% | 0.01% | 100% | 100% |
| Test 2 | (c)+(d) | 0.02% | 93% | 98% | 0.01% | 99% | 100% | 0.03% | 99% | 100% | 0.00% | 92% | 100% |
| Test 3 | (c) | 0.03% | 94% | 100% | 0.02% | 99% | 100% | 0.02% | 99% | 100% | 0.01% | 92% | 101% |
| Test 4 | (a)+(c)+(d) | 0.04% | 92% | 98% | 0.03% | 99% | 100% | 0.03% | 99% | 101% | 0.01% | 91% | 100% |
| Test 5 | (a)+(c) | 0.05% | 93% | 100% | 0.02% | 99% | 100% | 0.04% | 99% | 101% | 0.02% | 91% | 101% |

Test 1:  (a)

(a) Max. 16 LFNST coefficients

Results with inter MTS off (CTC)

Inter MTS on

Low QP (2, 7, 12, and 17)

Test 2:  (c) + (d)

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

(d) Change of the region where non-zero coefficients are counted

Results with inter MTS off  (CTC)

Results with inter MTS on

Low QP (2, 7, 12, and 17)

Test 3:  (c)

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

Results with inter MTS off  (CTC)

Results with inter MTS on

Low QP (2, 7, 12, and 17)

Test 4:  (a) + (c) + (d)

(a) Max. 16 LFNST coefficients

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

(d) Change of the region where non-zero coefficients are counted

Results with inter MTS off  (CTC)

Results with inter MTS on

Low QP (2, 7, 12, and 17)

Test 5:  (a) + (c)

(a) Max. 16 LFNST coefficients

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

Results with inter MTS off  (CTC)

Results with inter MTS on

Low QP (2, 7, 12, and 17)

Discussion

How the proposed zero-out scheme is configured

For example, CE6-2.1a in [3] employs zero-out for both 4x4 LFNST and 8x8 LFNST, and Test 2/3/4/5 in this contribution for only 8x8 LFNST.

As found in the experiment results, up-to 0.04% coding loss can be recovered if 8x8 LFNST zero-out is only enabled, which increases 3% encoding time relative to CE6-2.1a. Additionally, coding loss (0.11%) that occurs in low QP case for CE6-2.1a almost disappears in Test 2/3/4/5, because on low QP condition CUs of smaller size appear more often than CTC and the zero-out for 4x4 LFNST is turned off in Test 2/3/4/5.

Whether normative changes due to the zero-out is necessary or not

Comparing between Test 2 and Test 3 or between Test 4 and Test 5, it is turned out that normative changes in LFNST index coding (the feature (d)) due to the zero-out increase only marginal gain, which may not be strongly justified if coding performance benefit is majorly concerned.

But, if all the normative changes in LFNST index signaling are disabled in CE6-2.1a, 0.18% coding loss was observed on AI condition, which seems to be significant.

Complexity reduction due to the proposed features

It is clear that the zero-out features, (b) and (c), could reduce computational complexity incurred by two transforms, i.e. primary transform and LFNST. Also, they could also reduce summed processing latency in the two transforms, which may help particularly real-time encoder implementation. The high computing cases in the two transforms happen in TUs of large size, of which computational load could be significantly reduced and implementation burden in transform design be relieved, especially if the feature (c) is employed. According to the experimental results of Test 2/3/4/5, complexity reduction in encoder design could be realized in either normative or non-normative way. However, from the decoder perspective, the similar complexity reduction can be achieved mostly in a normative way, because the additional zero-out in primary transform is only effective when LFNST is applied. Therefore, it should be further discussed about necessity of the relevant normative changes for effective decoder complexity reduction. Of course, in general, design margin could be widened and power consumption be reduced if the additional zero-out is incorporated.

Conclusion

In this proposal, five combinations of simplification features introduced in CE6-2.1a were tested and the benefits regarding the features were discussed based on experimental results, considering trade-off between coding gain and complexity, normative and non-normative changes, reductions of both computation and latency in total transforms, etc. It is recommended that the most beneficial decision should be made based on test results in this proposal together with those of CE6-2.1a from multi-perspective views.

Spec text

The spec texts for all the tests are included together with this contribution document. It should be noted that spec texts of Test 3 and Test 5 are the same as those of VTM 5.0 and Test 1 respectively, because they only add non-normative changes relative to each base configuration.

Spec text difference between Test 2 and CE6-2.1a

One minor difference due to the feature (b) is presented with red bold letters as shown below.

Test 2:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 3 && log2TbHeight >= 3 ) ¦ ¦ |  |
| ( lastScanPos > 7 && ( log2TbWidth == 2 ¦ ¦ log2TbHeight == 3 ) && log2TbWidth == log2TbHeight ) ) { |  |
| lfnstZeroOutSigCoeffFlag = 1 |  |
| } |  |
| …… |  |
| } |  |

CE6-2.1a:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ |  |
| ( lastScanPos > 7 && ( log2TbWidth == 2 ¦ ¦ log2TbHeight == 3 ) && log2TbWidth == log2TbHeight ) ) { |  |
| lfnstZeroOutSigCoeffFlag = 1 |  |
| } |  |
| …… |  |
| } |  |

Reference

- M. Koo et al., “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0193, 14th meeting, Geneva, CH, 19–27 March 2019.

- M. Siekmann et al., “CE6-related: Simplification of the Reduced Secondary Transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0555, 14th meeting, Geneva, CH, 19–27 March 2019.

- M. Siekmann et al., “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0094, 15th meeting, Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19–27 March 2019.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-N1026, 14th meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).