| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0210-v2 |
| --- | --- |

| Title: | Non-CE9: BDOF for chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

BDOF (bi-directional optical flow) applies only to luma component. This contribution discusses how to apply BDOF to chroma components. Currently, WD and VTM software support not only 4:2:0 format, but also 4:2:2, 4:4:4 and RGB formats. The impact on chroma components of those formats should not be ignored and considering the consistency with the range extensions, BDOF applied to chroma components should be also introduced to 4:2:0 format. Therefore, this contribution proposes a method to apply BDOF to chroma components efficiently by using optical flow vector obtained by luma component. As an experimental result on CTC, the average of encoding and decoding times increased 1% and 5% respectively, the average of coding gains for 4:2:0, Y/Cb/Cr are 0.01%0.43%/0.61% respectively. Compared to BDOF applied to each component independently, the coding efficiency is almost the same or a little better and the encoding and decoding times are 2% and 5& improved. If two speed up methods which are proposed in JVET-O0208 and JVET-O0211, are introduced, the average of decoding times can additionally improve 2% without coding loss. As experimental results of 4:2:2, 4:4:4 and RGB formats, the average coding gain of YUV4:2:2 are 0.01%/0.46%/0.52% respectively, the average coding gain of YUV4:4:4 are 0.00%/0.56%/0.95% respectively and the average of coding gains of RGB4:4:4 are 0.15%/0.94%/1.51%. According to the number of chroma samples, the coding gains have been improved.

Introduction

WD and VTM software already has supported 4:2:2, 4:4:4 and RGB formats [1]. BDOF applies only to luma component currently. Considering the consistency with the range extensions, BDOF for chroma components should be introduced to not only 4:2:2, 4:4:4 and RGB formats, but also 4:2:0 format. In this contribution, we propose a method to apply BDOF to chroma components efficiently by using optical flow vector obtained by luma component. Figure 1 shows the proposed processing.

Figure 1	Proposed BDOF processing for chroma components

Proposal changes of specification

The working draft [2] should be changed by the followings.

#### 8.5.6.4	Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the variable cIdx specifying the colour component index,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 ).

- The variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift3 is set to equal to Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ).

- The variable scaleX is set equal to (cIdx == 0) ? 0 : ((chroma_format_idc == 1 | | chroma_format_idc == 2) ?  1 : 0).

- The variable scaleY is set equal to (cIdx == 0) ? 0 : ((chroma_format_idc == 1) ?  1 : 0).

For xIdx = 0..( nCbW >> (2-scaleX) ) − 1 and yIdx = 0..( nCbH >> (2-scaleY) ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << (2-scaleX)) + 1 and ySb is set equal to ( yIdx << (2-scaleY) ) + 1.

- If bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + (2-(1<<scaleX)), y = ySb − 1.. ySb + (2-(1<<scaleY)), the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(8-766)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift4 )

- Otherwise (bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + (4-(1<<scaleX)), y = ySb − 1..ySb + (4-(1<<scaleY)), the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-767)

vy = Clip3( 1, nCbH, y )		(8-768)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-771)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-772)

- If cIdx is equal to 0 for luma samples, the variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-773)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-774)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-775)

  - If cIdx is equal to 0 for luma samples, the variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-776)

sGy2 = ij( tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-777)

sGxGy = ij( tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-778)

sGxGym = sGxGy >> 12		(8-779)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-780)

sGxdI = ij( −tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-781)

sGydI = ij( −tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-782)

  - If cIdx is equal to 0 for luma samples, the horizontal and vertical motion offset of the current subblock are derived as:

vx [0] = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy [0] = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx [0] * sGxGym ) << 12 + vx [0] * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

  - Otherwise (cIdx is not equal to 0 for chroma samples), the horizontal and vertical motion offset of the current subblock are derived as vx [cIdx] = vx [0] and vy [cIdx] = vy [0].

  - For x =xSb − 1..xSb + (2-(1<<scaleX)), y = ySb − 1..ySb + (2-(1<<scaleY)), the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx [cIdx] * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-785)
	+ Round( ( vy [cIdx] * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-786)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

Experimental results

The proposed method has been implemented on VTM-5.0 software. The simulation was based on VTM configuration JVET-N1010 [3]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU. In addition, according to JCT-VC Range Extensions (RExt) CTC [4], the proposed has been tested by using YUV4:2:2, YUV4:4:4 and RGB4:4:4 sequences. Note that although some sequences are encoded half number of frames in JCTVC-P1006, all sequences were encoded full number of frames in this contribution.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.34% | -0.54% | 101% | 105% |
| Class A2 | -0.04% | -0.71% | -1.00% | 101% | 106% |
| Class B | 0.02% | -0.38% | -0.45% | 101% | 105% |
| Class C | -0.01% | -0.34% | -0.57% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.43% | -0.61% | 101% | 105% |
| Class D | -0.02% | -0.56% | -0.56% | 101% | 105% |
| Class F | 0.00% | -0.21% | -0.25% | 101% | 104% |

Table 2  Comparison between experimental results for 4:2:0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Proposed method | -0.01% | -0.43% | -0.61% | 101% | 105% |
| Proposed method with speed up techniques | -0.01% | -0.46% | -0.61% | 101% | 103% |
| BDOF applied to each component | -0.02% | -0.35% | -0.58% | 103% | 110% |

Table 3  Comparison between experimental results for RGB 4:4:4

|  | Random Access RGB 4:4:4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Proposed method | -0.15% | -0.94% | -1.51% | 101% | 106% |
| Proposed method with speed up techniques | -0.15% | -0.95% | -1.51% | 101% | 104% |
| BDOF applied to each component | -0.11% | -0.89% | -1.39% | 104% | 112% |

Table 4  Comparison between experimental results for YUV 4:4:4

|  | Random Access YUV 4:4:4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Proposed method | 0.00% | -0.56% | -0.95% | 102% | 106% |
| Proposed method with speed up techniques | -0.01% | -0.56% | -0.90% | 102% | 104% |
| BDOF applied to each component | 0.01% | -0.50% | -0.81% | 106% | 113% |

Table 5  Comparison between experimental results for YUV 4:2:2

|  | Random Access YUV 4:2:2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Proposed method | -0.01% | -0.46% | -0.52% | 101% | 105% |
| Proposed method with speed up techniques | 0.01% | -0.47% | -0.51% | 101% | 103% |
| BDOF applied to each component | -0.02% | -0.38% | -0.50% | 104% | 111% |

Conclusion

This contribution proposed a method to apply BDOF to chroma components efficiently by using optical flow vector obtained by luma component. As experimental results, this method has less computational complexity than independent BDOF for each component and the coding efficiency has been improved. Although we think BDOF for chroma components should be introduced to 4:2:2, 4:4:4 and RGB formats, considering the consistency with the range extensions, it should be also introduced to 4:2:0 format. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen, X. Zhao, X. Li, X. Xu, S. Liu, H. Wang, Y.-H. Chao, W.-J. Chien, V. Seregin and M. Karczewicz, Support of 4:4:4 and 4:2:2 chroma formats in VVC, Joint Video Experts Team Document, JVET-N0671-v4, Geneva, June 2019.

[2]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v8, Geneva, June 2019.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-N1010, Geneva, April 2019.

[4]	C. Rosewarne, K. Sharman, D. Flynn, Common test conditions and software reference configurations for HEVC range extensions, JCTVC-P1006, 16th JCT-VC Meeting, San Jose, CA, January 2014.

[5]	T. Chujoh, E. Sasaki and T. Ikai, Non-CE9: An improvement of early termination for BDOF, Joint Video Experts Team Document, JVET-O0208, Gothenburg, July 2019.

[6]	T. Chujoh and T. Ikai, Non-CE9: A speed up method of BDOF, Joint Video Experts Team Document, JVET-O0211, Gothenburg, July 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).