| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0211-v1 |
| --- | --- |

| Title: | Non-CE9: A speed up method of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

BDOF (bidirectional optical flow) is a tool to improve the bi-prediction value by using optical flows between two predictions; however the computational complexity has not been negligible yet. In this contribution, a method to improve the processing speed of BDOF is proposed. In the current specification of deriving the gradient values of optical flow, 32-bit operation for each pixel is requested. By changing the order of subtraction and shift, 16-bit operations is realized. When SIMD (single instruction / multiple data) is used, the number of parallel operations become double and the speed of the processing is upped. As an experimental result on CTC, the average of encoding times became 1% faster and the average gains of coding efficiency were -0.01%/-0.05%/-0.02% for Y/Cb/Cr respectively.

Introduction

Although BDOF is a very effective coding tool, the reduction of computational complexity should have been still encouraged. In the current VVC specification [1], the dynamic range of interpolated prediction value is 16-bit. Although the gradient values for deriving the optical flow are 16-bit precision, the processes defined four equations from 8-769 to 8-772 in subsection 8.5.6.4 are not 16-bit operations. Because the subtraction value between two 16-bit values is 17-bit. In this contribution, the order of subtraction and shift is changed and firstly, the two prediction values is shifted to right respectively, and secondly, those two values are subtracted. Since the shift value is 6, the precision after the subtraction is 11-bit. Therefore, 16-bit operation is realized. The pixel or block based image processing by SIMD is very effective and a lot of parts on VTM software including BDOF are implemented by SIMD code. The number of parallel operations can be doubled if 16-bit operation instead of 32-bit operation by SIMD can be realized. As an experimental result, there is no loss and the reduction of computational complexity has been achieved.

Proposal changes of specification

The working draft [1] should be changed by the followings.

#### 8.5.6.4	Bidirectional optical flow prediction process

From

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-771)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-772)

to

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[x][y] = (predSamplesL0[hx + 1][vy]>> shift1)− (predSampleL0[hx − 1][vy]>> shift1)	(8-769)

gradientVL0[x][y] = (predSampleL0[hx][vy + 1]>> shift1)− (predSampleL0[hx][vy − 1]>> shift1)	(8-770)

gradientHL1[x][y] = (predSamplesL1[hx + 1][vy]>> shift1)− (predSampleL1[hx − 1][vy]>> shift1)	(8-771)

gradientVL1[x][y] = (predSampleL1[hx][vy + 1]>> shift1)− (predSampleL1[hx][vy − 1]>> shift1)	(8-772)

Experimental results

The proposed method has been implemented on VTM-5.0 software. The simulation was based on VTM configuration JVET-N1010 [2]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, the average of encoding time became 1% faster. There was a little gain or no loss on average.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | -0.04% | 99% | 100% |
| Class A2 | -0.01% | -0.05% | -0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.02% | 99% | 100% |
| Class C | -0.02% | -0.02% | -0.07% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.02% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.15% | 99% | 99% |
| Class F | 0.00% | 0.00% | -0.01% | 99% | 100% |

Conclusion

In this contribution, an improvement method of BDOF was proposed. By the changing the order of subtraction and shift, the number of parallel operations becomes double and it is possible to use SIMD efficiently. The average of encoding time became 1% faster and there is a little gain or no loss of coding efficiency on average. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v8, Geneva, June 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-N1010, Geneva, April. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).