| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0214 |
| --- | --- |

| Title: | Non CE6: Harmonization of implicit MTS and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The normal implicit MTS is a tool which implicitly selects transform matrix between DST7 and DCT2 depending on CU shape and size in intra non sub-partition block. However it is asserted that it does not appropriately work under LFNST (low frequency non separable transform) enabled condition. This proposal is to solve this problem by restricting implicit MTS only when the stIdx is equal to 0. i.e. DCT2 or DST7 selection is used when LFNST is not applied in intra non sub-partition block (If not applied, DCT2 is used). It is reported that the coding loss of the implicit MTS case in VTM5 compared with explicit MTS (CTC) case was 1.02 %, 0.38 % and 0.11 % in AI, RA, LB while that of the proposed implicit MTS case is 0.28 %, 0.05 %, 0.11 % in AI, RA, LB respectively. In deduction, about 0.74 %, 0.33 %, 0.00 % improvement is reported.

When compared with CTC (MTS on) anchor,

VTM5:    1.02 %, 0.38%, 0.11 % (EncT: 90 %, 92 %, 97 %) in AI, RA, LB

Proposed:   0.28 %, 0.05 %, 0.11 % (EncT: 88 %, 92 %, 97 %) in AI, RA, LB

(Test: --MTS=0 --MTSImplicit=1, Anchor: no additional options)

When compared with MTS off anchor,

VTM5:    0.18 %, 0.05%, -0.11 % (EncT: 90 %, 92 %, 97 %) in AI, RA, LB

Proposed:   -0.55 %, -0.28 %, -0.11 % (EncT: 101 %, 100 %, 100 %) in AI, RA, LB

where minus means gain.

(Test: --MTS=0 --MTSImplicit=1, Anchor: --MTS=0)

Introduction

The implicit MTS is capable to select DST7 and DCT2 depending on CU shape and size as follows. It is effective selection by utilizing statistical characteristics.

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-966)

However, in VTM5.0 where LFNST is applied, the implicit MTS shows coding loss of 0.18%, 0.05 %. -0.11 % on average in AI, RA and LB respectively as shown in Table 1.

implicitMTS (anchor MTS off)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 MTS off (Sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | -0.71% | -0.72% | 104% | 101% |
| Class A2 | -0.10% | -1.02% | -0.90% | 103% | 102% |
| Class B | 0.05% | -1.21% | -1.38% | 104% | 102% |
| Class C | 0.42% | -0.42% | -0.49% | 103% | 100% |
| Class E | 0.30% | -0.79% | -0.92% | 103% | 101% |
| Overall | 0.18% | -0.85% | -0.92% | 103% | 101% |
| Class D | 0.40% | -0.19% | -0.21% | 102% | 101% |
| Class F | 0.25% | -0.47% | -0.46% | 102% | 101% |

Proposal

This proposal solves this problem by enabling implicit MTS only when the stIdx is equal to 0. i.e. DST7 can be used when LFNST is not used. (DCT2 is always used in LFSNT is on).

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, and stIdx is equal to 0, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-966)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-17 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-16 depending on tu_mts_idx[ xTbY ][ yTbY ].

Experimental result

Table 2 shows the implicit MTS’s coding gain with the proposed modification, which is 0.55 %, 0.28 %. 0.11 % on average in AI, RA and LB respectively

Note: encoder option is:  --MTS=0 --MTSImplicit=1

If we use VTM 5 CTC anchor, the implicit MTS’s coding gain with the proposed modification is 0.28 %, 0.05 %. 0.11 % with encoding time of 88 %, 92 %, 97 % on average in AI, RA and LB respectively.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).