| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0216-v1 |
| --- | --- |

| Title: | Non-CE5: Modification of ALF coefficient signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution proposes fixed 3rd order Exponential golomb codes for signalling ALF filter coefficients. VTM-5.0 uses filter merging process wherein the filter coefficient of filter N are predicted from filter N-1 and furthermore the signalled filter coefficients can further be predicted from the fixed filters which are newly introduced in VTM-5.0. The “delta (residual)” is then transmitted using Kth order exponential golomb codes. Reportedly the range of filter coefficient “delta” values can be signalled with fixed 3rd order Exponential golomb codes instead of adaptive Kth Order exponential golomb codes without any coding efficiency impact.

The objective results with “fixed 3rd order Exp. golomb” signalling scheme are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP) with “CTC” QP: Luma BD-Rate of 0.00%, 0.01%, 0.02%, -0.01% is achieved.

The objective results with “fixed 3rd order Exp. golomb” signalling scheme are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP) with “low” QP values (12, 27, 22, 27): Luma BD-Rate of 0.01%, 0.00%, 0.02%, 0.02% is achieved.

The objective results with “fixed 3rd order Exp. golomb” signalling scheme are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP) with “high” QP values (32, 37, 42, 47): Luma BD-Rate of -0.01%, 0.01%, -0.01%, -0.02% is achieved.

# Introduction

VTM-5.0[1] uses filter merging process wherein the filter coefficient of filter N are predicted from filter N-1 and furthermore the signalled filter coefficients can be further be predicted from the fixed filters which are newly introduced in VTM-5.0. The “delta (residual)” is then transmitted using Kth order exponential golomb codes. Syntax elements (highlighted in yellow) in Table 1 are the ones which are which are used for the signalling of ALF coefficients using Kth order exponential golomb codes. The current proposal targets to simplify the signalling of ALF coefficients.

| alf_luma_min_eg_order_minus1 | ue(v) |
| --- | --- |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |

Table 1.  alf_luma_coeff_delta_abs and alf_chroma_coeff_abs are signalled using Kth order Exponential golomb codes in VTM-5.0

# Proposed Solution

The current proposal reports that the range of filter coefficient “delta” values can be signalled with fixed 3rd order Exponential golomb codes, instead of using adaptive Kth Order exponential golomb codes without any coding efficiency impact. Moreover using fixed 3rd order exponential golomb codes removes few unnecessary syntax elements and at the same time simplifies the ALF encoder logic for determining the order K of the exponential golomb codes. Syntax table 2 below shows the modified syntax after substituting the Kth order exponential golomb codes with fixed 3rd order exponential golomb codes.

| alf_luma_min_eg_order_minus1 | ue(v) |
| --- | --- |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) ue3(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) ue3(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |

Table 2.  ALF coefficients signalled using fixed 3rd order exponential golomb codes

# Simulation Results

The proposed signalling scheme is implemented on top of VTM-5.0 [1] and the simulation is performed under common test conditions as described in JVET-N1010 [2].

## Results with CTC QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | -0.02% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.01% | -0.01% | -0.05% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.02% | 101% | 101% |
| Class A2 | 0.02% | 0.08% | 0.03% | 101% | 101% |
| Class B | 0.02% | -0.02% | -0.07% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% | 101% | 101% |
| Class D | 0.05% | 0.12% | 0.18% | 100% | 101% |
| Class F | -0.01% | 0.04% | 0.02% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | -0.14% | 101% | 104% |
| Class C | 0.04% | -0.02% | -0.15% | 100% | 101% |
| Class E | 0.05% | 0.39% | -0.27% | 101% | 102% |
| Overall | 0.02% | 0.14% | -0.18% | 101% | 102% |
| Class D | 0.02% | -0.29% | 0.70% | 100% | 102% |
| Class F | -0.03% | -0.05% | -0.59% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.03% | -0.04% | 101% | 103% |
| Class C | 0.02% | -0.23% | 0.19% | 100% | 100% |
| Class E | -0.05% | 0.51% | -0.44% | 101% | 103% |
| Overall | -0.01% | 0.04% | -0.06% | 101% | 102% |
| Class D | 0.02% | -0.23% | 0.70% | 100% | 102% |
| Class F | -0.11% | -0.08% | 0.19% | 101% | 102% |

## Results with high QP (32, 37, 42, 47)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 99% |
| Class C | -0.01% | -0.03% | -0.03% | 101% | 103% |
| Class E | -0.01% | -0.03% | -0.05% | 100% | 99% |
| Overall | -0.01% | -0.02% | -0.02% | 100% | 99% |
| Class D | -0.01% | -0.04% | -0.07% | 100% | 97% |
| Class F | -0.01% | -0.03% | -0.02% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class B | 0.02% | 0.07% | -0.03% | 100% | 100% |
| Class C | 0.00% | 0.05% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | -0.03% | 100% | 100% |
| Class D | -0.01% | 0.16% | 0.02% | 100% | 100% |
| Class F | -0.01% | -0.06% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.01% | -0.35% | 100% | 99% |
| Class C | 0.00% | -0.35% | -0.50% | 99% | 96% |
| Class E | 0.04% | 0.12% | 0.04% | 101% | 103% |
| Overall | -0.01% | -0.09% | -0.30% | 100% | 99% |
| Class D | 0.12% | 0.00% | -0.24% | 100% | 99% |
| Class F | -0.12% | -0.67% | -0.13% | 100% | 97% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.08% | -0.61% | 100% | 100% |
| Class C | 0.06% | -0.16% | -0.14% | 102% | 105% |
| Class E | -0.14% | 0.67% | 0.27% | 100% | 99% |
| Overall | -0.02% | 0.09% | -0.23% | 101% | 101% |
| Class D | -0.03% | -0.53% | -1.31% | 100% | 99% |
| Class F | -0.13% | 0.29% | 0.45% | 101% | 101% |

## Results with low QP (12, 17, 22, 27)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.01% | 0.01% | 0.00% | 101% | 99% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.01% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.02% | 0.00% | -0.01% | 101% | 99% |
| Class F | 0.01% | 0.00% | 0.00% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class B | 0.01% | -0.03% | -0.06% | 100% | 99% |
| Class C | 0.01% | 0.04% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.00% | 100% | 100% |
| Class D | 0.05% | 0.09% | 0.02% | 100% | 100% |
| Class F | 0.00% | 0.02% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class C | 0.02% | 0.01% | 0.07% | 100% | 98% |
| Class E | 0.03% | 0.07% | -0.02% | 100% | 99% |
| Overall | 0.02% | 0.04% | 0.03% | 100% | 99% |
| Class D | 0.03% | -0.07% | 0.04% | 100% | 99% |
| Class F | 0.04% | 0.11% | 0.18% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | 0.00% | 100% | 99% |
| Class C | 0.01% | -0.06% | 0.07% | 100% | 99% |
| Class E | 0.05% | -0.02% | -0.14% | 101% | 103% |
| Overall | 0.02% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.03% | 0.07% | 0.21% | 99% | 96% |
| Class F | -0.07% | 0.02% | 0.07% | 102% | 106% |

# Conclusion

An alternative way of signalling ALF filter coefficients is proposed wherein the adaptive Kth order exponential golomb codes are substituted by fixed 3rd order exponential golomb codes. Objective results showed no BD-Rate impact with the main advantage of removing few syntax elements and also simplifying the ALF encoder logic for determining the order K for the Kth order exponential golomb codes.  It is suggested to adopt the fixed 3rd order exponential golomb coding scheme for signalling the ALF filter coefficients. Specification text changes are attached along with the contribution.

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).