| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0219-v1 |
| --- | --- |

| Title: | Non-CE6: On LFNST transform set selection for a CCLM coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, R. Sjöberg R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a method to improve chroma coding efficiency by improving LFNST (Low Frequency Non-Separable Transform) transform set selection when one of three CCLM (cross-component linear model) modes is used for a chroma block. The proposed method utilizes the co-located luma intra prediction mode as the transform set selection table index to select the LFNST transform set for the chroma block. The proposed method has Y / Cb / Cr BDR impact for AI of -0.01% / -0.40% / -0.34% compared to VTM5.0 and Y / Cb / Cr BDR impact for RA of 0.00% / -0.37% / -0.33% compared to VTM5.0 and decoding time impact for AI / RA of 100% / 100% compared to VTM5.0.

# Introduction

In the current VVC, when LFNST is determined to be used for the current block, a LFNST matrix is chosen from four transform sets, where each transform set consists of two transforms. Which transform set is applied is determined from intra prediction modes as follows:

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

Table 1. The transform set selection table

The index to access Table 1, predModeIntra, has a range of [-14, 83], which is a transformed mode index used for wide angle intra prediction.

As shown in table 1, if one of three CCLM modes (INTRA_LT_CCLM, INTRA_T_CCLM or INTRA_L_CCLM) is used for the current block (81 <= predModeIntra <= 83), transform set 0 is selected for the current chroma block. Transform set 0 is also selected by a block with an intra prediction mode 0 or 1, which specifies planar or dc intra prediction mode respectively.

When a block is predicted by one of three CCLM modes, the chroma predicted samples are derived by using a linear model on downsampled reconstructed luma samples. If the reconstructed luma samples have a directional pattern other than planar or dc, we can assume the predicted chroma block have a directional pattern similar to the reconstructed luma samples. Therefore, the current LSNFT transform set selection for a CCLM predicted chroma block may not well suitable when the current chroma block has a directional pattern other than planar or dc.

# Proposed method

This contribution proposes an alternative method of LSNFT transform set selection if one of three CCLM modes is used for the current chroma block.

The proposed method determines an IntraPredMode_CCLM for the current chroma block, where the IntraPredMode_CCLM has a range of [-14, 80]. The LSNFT transform set for the current chroma block is selected from table 1 by using the IntraPredMode_CCLM as the table index.

The IntraPredMode_CCLM is determined by the co-located luma intra prediction mode and the dimension of the current chroma block.

The co-located luma intra prediction mode (lumaIntraPredMode) is derived in the same way specified in current VVC WD:

8.4.4 Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is derived using IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] and sizeId

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The IntraPredMode_CCLM is further derived from wide angle intra prediction mapping by using co-located luma intra prediction mode and the dimension of the current chroma block.

Finally, one of four LSNFT transform sets in table 1 is selected by using IntraPredMode_CCLM as the table index. The selected LSNFT transform set is used for the current chroma block.

The transform coefficients transpose flag is determined by checking IntraPredMode_CCLM, where in the current VVC the transpose flag is always equal to 0 when one of three CCLM modes is used for the current chroma block.

The current VVC WD is changed as:

8.7.4 Transformation process for scaled transform coefficients

8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-953)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-954)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-955)

nLfnstSize = 1  <<  log2LfnstSize		(8-956)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-957)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-958)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-959)

- When sps_cclm_enabled_flag is equal to 1, and 81 <= predModeIntra <=83, the corresponding luma intra prediction mode lumaIntraPredMode derivation process as specified in clause 8.4.4 is invoked with a luma location ( xTbY, yTbY ), cbWidth, cbHeight, as inputs,  and the modified predModeIntra is equal to lumaIntraPredMode as output.

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-960)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-961)

u[ x ] = d[ xC ][ yC ] 			(8-962)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLOM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(8-963)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(8-964)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

# Simulation result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | -0,39% | -0,46% | 100% | 100% |
| Class A2 | 0,00% | -0,24% | -0,24% | 100% | 100% |
| Class B | 0,00% | -0,47% | -0,39% | 100% | 100% |
| Class C | -0,03% | -0,49% | -0,33% | 100% | 100% |
| Class E | -0,01% | -0,30% | -0,25% | 100% | 100% |
| Overall | -0,01% | -0,40% | -0,34% | 100% | 100% |
| Class D | -0,04% | -0,06% | -0,27% | 100% | 100% |
| Class F | -0,02% | -0,42% | -0,34% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,56% | -0,44% | 100% | 100% |
| Class A2 | 0,03% | -0,10% | -0,09% | 99% | 100% |
| Class B | 0,01% | -0,35% | -0,47% | 100% | 100% |
| Class C | -0,01% | -0,44% | -0,27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | -0,37% | -0,33% | 100% | 100% |
| Class D | 0,03% | -0,32% | -0,23% | 100% | 100% |
| Class F | -0,01% | -0,20% | -0,24% | 100% | 100% |

# Conclusions

The proposed method is a better LSNFT transform set selection for a chroma block when one of three CCLM modes is used for the current block. We propose to adopt the proposed method to VVC and VTM to improve the coding efficiency for chroma components.

# References

[1]. Versatile Video Coding (Draft 5)

[2]. CE6: Reduced Secondary Transform (RST) (CE6-3.1)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).