| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0220 |
| --- | --- |

| Title: | Non-CE4: On maximum number of subblock merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Du Liu Torshamnsgatan 23, 164 80 Stockholm, Sweden | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-5.0, subblock merge candidate list contains subblock temporal motion vector prediction (SbTMVP) candidate and affine candidates. For each inter (P/B) slice, a slice level variable MaxNumSubblockMergeCand is derived to control the length of the subblock merge candidate list. It is asserted that there are following two issues with regards to the MaxNumSubblockMergeCand. Suggested modifications are also presented.

- In the VTM-5.0 reference software, the parsing of merge_subblock_flag is conditioned on SPS enabling flags for SbTMVP and affine, i.e. sps_sbtmvp_enabled_flag and sps_affine_enabled_flag. Given the VTM-5.0 syntax, it can have MaxNumSubblockMergeCand = 0 for some slices. For those slices, there would be no presence of subblock merge candidates, which would make the merge_subblock_flag always 0.

It is suggested to change the VTM-5.0 software to make the parsing of merge_subblock_flag conditioned on MaxNumSubblockMergeCand instead.

- The MaxNumSubblockMergeCand is inferred to 1 when the SPS enabling flags sps_affine_enabled_flag = 0 and sps_sbtmvp_enabled_flag = 1. In this case, the intention is let the subblock merge candidate list contain one SbTMVP candidate. However, the SbTMVP candidate generation process also follows the slice_temporal_mvp_enabled_flag. When the slice_temporal_mvp_enabled_flag is 0, the slice disallows any temporal motion vector prediction. In other words, for the combination of sps_affine_enabled_flag = 0, sps_sbtmvp_enabled_flag = 1 and slice_temporal_mvp_enbaled_flag = 0, the subblock merge candidate list would only contain one less meaningfull zero motion candidate.

It is proposed to set the five_minus_max_num_subblock_merge_cand to be 5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag) when five_minus_max_num_subblock_merge_cand is not present.

# Introduction

In VTM-5.0, there is a slice level variable MaxNumSubblockMergeCand which controls the length of the subblock merge candidate list. The value of MaxNumSubblockMergeCand is required to be within the range of 0 to 5. It is derived by as

MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand

The variable five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction (MVP) candidates supported in the slice subtracted from 5. When five_minus_max_num_subblock_merge_cand is not present, it is inferred to be equal to 5 − sps_sbtmvp_enabled_flag.

It is asserted that there are two issues with regards to the MaxNumSubblockMergeCand.

The first issue exists in the VTM-5.0 reference software. Currently the parsing of merge_subblock_flag is conditioned on SPS enabling flags sps_affine_enabled_flag and sps_sbtmvp_enabled_flag. For some of the slices, when MaxNumSubblockMergeCand is set to 0. There would be no presence of subblock merge candidates, which would make merge_subblock_flag always 0. Thus, the parsing of merge_subblock_flag is unnecessary.

The second issue exists in both VVC specification and VTM reference software. Currently the MaxNumSubblockMergeCand is inferred to 1 when the SPS enabling flags sps_affine_enabled_flag = 0 and sps_sbtmvp_enabled_flag = 1. In this case, the intention is let the merge candidate list contain one SbTMVP candidate. However, the SbTMVP candidate generation process also follows the slice_temporal_mvp_enabled_flag. When the slice_temporal_mvp_enabled_flag is 0, the slice disallows any temporal motion vector prediction including SbTMVP. In other words, for combination of sps_affine_enabled_flag = 0, sps_sbtmvp_enabled_flag = 1 and slice_temporal_mvp_enbaled_flag = 0, the subblock merge candidate list would only contain one less meaningfull zero motion candidate.

# Proposal

## Proposal #1: Change the VTM software such that the parsing merge_subblock_flag is dependent on the MaxNumSubblockMergeCand.

It is proposed to change the VTM-5.0 software to make the parsing of merge_subblock_flag depends on the slice level variable MaxNumSubblockMergeCand.

The following shows the code change where the modifications are encapsulated in the macro MAX_CAND_FOLLOW_SLICE.

In CABACReader.cpp:

void CABACReader::subblock_merge_flag( CodingUnit& cu ) { … #if MAX_CAND_FOLLOW_SLICE if (!cu.cs->slice->isIntra() && (cu.slice->getMaxNumAffineMergeCand() > 0) && cu.lumaSize().width >= 8 && cu.lumaSize().height >= 8) #else if ( !cu.cs->slice->isIntra() && (cu.cs->sps->getUseAffine() ¦¦ cu.cs->sps->getSBTMVPEnabledFlag()) && cu.lumaSize().width >= 8 && cu.lumaSize().height >= 8 ) #endif ... }

In CABACWriter.cpp:

void CABACWriter::subblock_merge_flag( const CodingUnit& cu ) { ... #if MAX_CAND_FOLLOW_SLICE if ( !cu.cs->slice->isIntra() && (cu.slice->getMaxNumAffineMergeCand() > 0) && cu.lumaSize().width >= 8 && cu.lumaSize().height >= 8 ) #else if ( !cu.cs->slice->isIntra() && (cu.cs->sps->getUseAffine() ¦¦ cu.cs->sps->getSBTMVPEnabledFlag()) && cu.lumaSize().width >= 8 && cu.lumaSize().height >= 8 ) #endif ... }

## Proposal #2: Change the VVC specification and VTM software to set MaxNumSubblockMergeCand also according to slice_temporal_mvp_enabled_flag.

The comination of sps_affine_enabled_flag = 0, sps_sbtmvp_enabled_flag = 1 and slice_temporal_mvp_enbaled_flag = 0 makes the subblock merge candidate list to only contain one less meaningfull zero candidate. It is proposed to set the MaxNumSubblockMergeCand to 0 instead.

The specification text change are shown as below.

… 7.4.6 Slice header semantics 7.4.6.1 General slice header semantics … five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction (MVP) candidates supported in the slice subtracted from 5. When five_minus_max_num_subblock_merge_cand is not present, it is inferred to be equal to 5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag). The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand is derived as follows: MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand (7-59) The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive. …

# Conclusion

It is proposed to adopt the proposed changes to the next version of VTM reference software and VVC specification text.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).