| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0225 |
| --- | --- |

| Title: | AHG17: On signalling of bottom right brick for rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Takahiro Nishi | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is proposed to include two modifications to the current syntax to signal rectangular slices in the picture parameter set:

- Change the semantics of the syntax element bottom_right_brick_idx_delta so that it is expressed in units of bricks for the first tile, then in units of tiles;

- Infer the value of the index of the bottom right brick for the last slice of the picture.

The authors believe that with these modifications, non-conformant slices cannot be signalled and the implementation of encoders and decoders may be simplified, since checks to make sure that constraints are fulfilled are not necessary anymore.

# Introduction

In VVC D5, a picture can be divided into tiles, which typically contain a rectangular set of CTU in the picture and share their sides with other tiles. A tile can be further divided into bricks that contain rows of CTUs inside the tile. A slice is a NAL unit that contains an integer number of bricks that are selected either in raster scan order of tiles and bricks or that contains a rectangular set of bricks. The second type of slices are called rectangular slices and are signalled in the Picture Parameter Set as illustrated in Figure 1, where for each rectangular slice of the picture, the top left brick index of the slice is signalled (called top_left_brick_idx in Figure 1) and a delta between the top left brick index and the bottom right brick index of the slice is signalled (called bottom_right_brick_idx_delta in Figure 1). For each rectangular slice of the picture, the bottom right brick index is identified as a sum of top_left_brick_idx and bottom_right_brick_idx_delta. Both syntax elements are signalled in units of bricks.

| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| --- | --- |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |

Figure 1: Current syntax in VVC D5 to signal rectangular slices

As an illustrating example, the following syntax elements are required to signal the two slices in Figure 2:

- num_slices_in_pic_minus1 = 1

- bottom_right_brick_idx_delta[0] = 7

- top_left_brick_idx[1] = 2

- bottom_right_brick_idx_delta[1] = 9

Figure 2: Example picture with 2 slices, 4 tiles and 12 bricks

The current syntax to signal the position of the bottom right brick is redundant, since:

- Not all slice signalling allowed by this syntax is allowed by the standard: there is a constraint that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile. This constraint means that additional checks are required at both encoder and decoder to make sure that the constraint is applied.

- The bottom right brick of the last slice of the picture is always the last brick of the picture, so that there is no need to signal it.

# Change semantics of syntax element signalling the bottom right brick

## Proposal to change the unit of bottom_right_brick_idx_delta

The first proposed modification is to change the semantics of the syntax element bottom_right_brick_idx_delta so that the constraint mentioned above (rectangular slices shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile) is automatically fulfilled. This can be achieved by modifying the unit in which this syntax element is expressed based on the value of the syntax element itself:

- If the value of bottom_right_brick_idx_delta is less than the number of bricks in the tile containing the first brick of the slice (i.e. the brick signalled by top_left_brick_idx), then bottom_right_brick_idx_delta is expressed in units of bricks.

- All values of bottom_right_brick_idx_delta greater than or equal to the number of bricks in the tile containing the first brick of a slice are expressed in units of tiles instead of units of bricks.

The following syntax elements are now required to signal the illustrative example in Figure 2:

- num_slices_in_pic_minus1 = 1

- bottom_right_brick_idx_delta[0] = 3 (one brick for first tile, two tiles)

- top_left_brick_idx[1] = 2

- bottom_right_brick_idx_delta[1] = 5 (three bricks for first tile, two tiles)

With this modification, the syntax for slices implements the constraint on what a slice may contain, thereby avoiding non-conformant slices and simplifying the implementation of encoders and decoders, since no check is necessary to make sure that the constraint is satisfied.

## Specification text

The syntax of the pic_parameter_set_rbsp syntax structure are not modified, only the semantics are modified as follows with additional text in yellow.

In 7.4.3.4 Picture parameter set RBSP semantics:

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ] in number of bricks for the first tile, then in number of tiles. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ], botRightBkIdx[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice, the index of the brick located at the bottom right corner of the i-th slice and the mapping of bricks to slices, are derived as follows:

for( t = 0; t < NumTilesInPic; t++ ) {
		tileX = t % ( num_tile_columns_minus1 + 1 )
		tileY = t / ( num_tile_columns_minus1 + 1 )
		if( BrickColBd[ top_left_brick_idx[ i ] ]  = =  tileColBd[ tileX ] && 
					BrickRowBd[ top_left_brick_idx[ i ] ] >= tileRowBd[ tileY ] &&
					BrickRowBd[ top_left_brick_idx[ i ] ] <= tileRowBd[ tileY ] + RowHeight[ tileY ] )
			tIdx = t
}
if( bottom_right_brick_idx_delta[ i ] <= num_brick_rows_minus1[ tIdx ] )
		botRightBkIdx[ i ] = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
else 
{
		botRightBkIdx[ i ] = top_left_brick_idx[ i ] + num_brick_rows_minus1[ tIdx ]
		BTDelta = bottom_right_brick_idx_delta[ i ] – num_brick_rows_minus1[ tIdx++ ]
		while( BTDelta > 0 ) {
			botRightBkIdx[ i ]  + =  num_brick_rows_minus[ tIdx++ ] + 1
			BTDelta  – =  1
		}
}
NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx[ i ] ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-36)
					BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx[ i ] ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

In 7.4.6.1 General slice header semantics:

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0.

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  slice_id[ sliceIdx ] )
		sliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = top_left_brick_idx[ sliceIdx ]
	botBrickIdx = brickIdx + bottom_right_brick_idx_delta[ sliceIdx ]
	for( bIdx = 0; brickIdx  <=  botBrickIdx botRightBkIdx[ sliceIdx ]  ; brickIdx++ )	(7-40)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

# Infer position of bottom right brick

## Modify the syntax to infer the value of the bottom right brick index

The second proposed modification is to infer the value of the index for the bottom right brick for the last slice of the picture, since it is always the last brick of the picture.

The following syntax elements are now required to signal the illustrative example in Figure 2:

- num_slices_in_pic_minus1 = 1

- bottom_right_brick_idx_delta[0] = 7

- top_left_brick_idx[1] = 2

## Specification text

The syntax of the pic_parameter_set_rbsp syntax structure is modified as follows:

| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| --- | --- |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| if( i < num_slices_in_pic_minus1 ) |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |

In 7.4.3.4 Picture parameter set RBSP semantics, the semantics of bottom_right_brick_idx_delta is modified as follows:

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

The value of bottom_right_brick_idx_delta[ num_slices_in_pic_minus1 ] is inferred to be equal to NumBricksInPic minus one minus top_left_brick_idx[ num_slices_in_pic_minus1 ].

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-36)
					BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

# Combination of both ideas

## Combination of changing the unit of bottom_right_brick_idx_delta and inferring its value for the last slice

Both ideas presented in this document can be combined. The illustrative example of Figure 2 now requires the following syntax elements in the bitstream:

- num_slices_in_pic_minus1 = 1

- bottom_right_brick_idx_delta[0] = 3 (one brick for first tile, two tiles)

- top_left_brick_idx[1] = 2

## Specification text

The syntax of the pic_parameter_set_rbsp syntax structure is modified as follows:

| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| --- | --- |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| if( i < num_slices_in_pic_minus1 ) |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |

In 7.4.3.4 Picture parameter set RBSP semantics:

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ] in number of bricks for the first tile, then in number of tiles. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ], botRightBkIdx[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice, the index of the brick located at the bottom right corner of the i-th slice and the mapping of bricks to slices, are derived as follows:

for( t = 0; t < NumTilesInPic; t++ ) {
		tileX = t % ( num_tile_columns_minus1 + 1 )
		tileY = t / ( num_tile_columns_minus1 + 1 )
		if( BrickColBd[ top_left_brick_idx[ i ] ]  = =  tileColBd[ tileX ] && 
					BrickRowBd[ top_left_brick_idx[ i ] ] >= tileRowBd[ tileY ] &&
					BrickRowBd[ top_left_brick_idx[ i ] ] <= tileRowBd[ tileY ] + RowHeight[ tileY ] )
			tIdx = t
}
if( i < num_slices_in_pic_minus1 
					&& bottom_right_brick_idx_delta[ i ] <= num_brick_rows_minus1[ tIdx ] )
		botRightBkIdx[ i ] = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
else if( i < num_slices_in_pic_minus1 )
{
		botRightBkIdx[ i ] = top_left_brick_idx[ i ] + num_brick_rows_minus1[ tIdx ]
		BTDelta = bottom_right_brick_idx_delta[ i ] – num_brick_rows_minus1[ tIdx++ ]
		while( BTDelta > 0 ) {
			botRightBkIdx[ i ]  + =  num_brick_rows_minus[ tIdx++ ] + 1
			BTDelta  – =  1
		}
}
else 
		botRightBkIdx[ i ] = NumBricksInPic – 1
NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx[ i ] ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-36)
					BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx[ i ] ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

In 7.4.6.1 General slice header semantics:

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0.

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  slice_id[ sliceIdx ] )
		sliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = top_left_brick_idx[ sliceIdx ]
	botBrickIdx = brickIdx + bottom_right_brick_idx_delta[ sliceIdx ]
	for( bIdx = 0; brickIdx  <=  botBrickIdx botRightBkIdx[ sliceIdx ]  ; brickIdx++ )	(7-40)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

# Conclusion

It is proposed to include two modifications to the current syntax to signal rectangular slices in the picture parameter set:

- Change the semantics of the syntax element bottom_right_brick_idx_delta so that it is expressed in units of bricks for the first tile, then in units of tiles;

- Infer the value of the index of the bottom right brick for the last slice of the picture.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).