| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0227 |
| --- | --- |

| Title: | AHG17: Parsing dependency of picture timing SEI message on SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is suggested to remove a parsing dependency, believed to be not intended by the author, of the picture timing SEI message on the active SPS by signalling the number of temporal sub-layers of the bitstream within the picture timing SEI message.

# Problem statement and proposal

At the last JVET meeting in March 2019, it was decided that it would be beneficial to remove the existing parsing dependency of the picture timing SEI message on the active SPS. The author of the present document fully supports this decision which makes implementation of the HRD much easier. At the same meeting, it was also decided to include a loop on temporal sub-layers within the picture timing SEI message, which is also supported fully by the author of the present document. Unfortunately, this decision created a new parsing dependency of the picture timing SEI message on the active SPS, which went unnoticed at the time, but which was never intended. It is therefore suggested to remove this parsing dependency by signalling the number of temporal sub-layers of the bitstream within the picture timing SEI message.

# Specification text

It is suggested to modify the syntax of the picture timing SEI message as follows:

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| frame_field_info_present_flag | u(1) |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| pt_max_sub_layers_minus1 | u(3) |
| cpb_removal_delay_minus1[ spspt_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < spspt_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| } |  |
| } |  |

It is suggested to add the following to the semantics of the picture timing SEI message:

pt_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of pt_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

It is a requirement of bitstream conformance that the value of pt_max_sub_layers_minus1 in the picture timing SEI message is equal to the value of sps_max_sub_layers_minus1 in the active SPS.

# Conclusion

It is suggested to remove an unintended parsing dependency of the picture timing SEI message on the active SPS by signalling the number of temporal sub-layers of the bitstream within the picture timing SEI message.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).