| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0229 |
| --- | --- |

| Title: | CE3-related: Simplifications of PDPC for diagonal and adjacent diagonal directions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Fabien Racapé, Fabrice Urban | Tel: Email: | +33.2.56.85.62.76 gagan.rath@InterDigital.com fabien.racape@InterDigital.com fabrice.urban@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

In VTM 5.0, PDPC is applied differently to the top-right and the bottom-left diagonal modes (modes 2 and 66) than to their adjacent angular modes. In this contribution, it is first shown that the PDPC applied to these two modes can be simplified to the same form as applied to their adjacent modes. Thus the two PDPC methods can be merged into a single PDPC method. In the second step, it is also shown that the resulting merged method can be simplified further by simplifying intermediate computations.  The proposed merging leads to identical BD-rate performances with reduced encoding and decoding times.

# Introduction

# In VTM 5.0 position dependent intra prediction combination (PDPC) is used  to smoothen the discontinuity at target block boundary for certain prediction modes. The prediction modes include the top-right mode (mode 2), and its adjacent angular modes (modes 3-10 and the wide angular modes -1, -2, … ), the bottom-left mode (mode 66), and its adjacent angular modes (modes 58-65, and the wide angular modes 67, 68, …), and the PLANAR, the DC, the vertical, and the horizontal modes. The top-right and the bottom-left modes (modes 2 and 66) are treated separately from their adjacent angular modes as the PDPC is applied differently on the target block. This contribution shows that both methods can be unified to a single method, which can be further simplified without losing any coding performance.

# Problem definition

For the sake of generality, a rectangular target block with width W pixels and height H pixels is considered. Let refMain and refSide represent the reference arrays on the top and the left respectively.

In modes 2 and 66, PDPC finds  two reference samples, one on each reference array, along the prediction direction, and then combines their values with the prediction value. Figure 1 shows the two reference samples for mode 66. The same figure is obtained for mode 2, if the top and left reference are  swapped, along with the height and width of the target block.

Figure 1: PDPC for bottom-left mode (mode 66). The top reference sample and the left reference samples are located on diagonally opposite directions. Target pixels for which the left reference samples are unavailable are not treated with PDPC.

Let Ptop and Pleft denote the values of the top and left reference pixels corresponding to target pixel A.  Then, Ptop=refMain[c+1], and Pleft=refSidec+1, where c=1+x+y. If Ppred(x,y) denotes the predicted value, the final predicted value at pixel A is obtained as

Px,y=Clip64-wT-wL*Ppredx,y+ wT*Ptop+wL*Pleft+32 ≫6, …………..(1)

where wT=16≫min31,y≪1≫scale, and wL=16≫min31,x≪1≫scale , with the scale pre-calculated as scale=log2W-2+log2H-2+2 ≫2.

For a target pixel, if the second reference sample on refSide is located beyond its length, its initial predicted value remains unchanged.

PDPC is also applied to the angular modes adjacent to modes 2 and 66. In particular, 8 regular modes adjacent to each of modes 2 and 66 are considered for PDPC. Furthermore, all wide angular modes are considered for PDPC. Thus, as illustrated in Figure 2, for an exemplary flat rectangular block with W=2*H, modes 58 to 65 as well as the wide angular modes 67-72 undergo PDPC.

Figure 2: Adjacent modes of bottom-left mode (mode 66) which are treated with PDPC for a rectangular block with W=2H.

In VTM 5.0, the PDPC tool finds the left reference pixel on refSide by extending the prediction direction, as illustrated in Figure 3. If the left reference pixel does not coincide with an integral sample location, the nearest reference pixel is taken.

Figure 3: PDPC for mode 68, which is an adjacent mode of vertical diagonal mode. The left reference samples are localized by extending the prediction direction till the left reference array. The nearest neighbor of the intersection point is selected as the left reference pixel.

Let Pleft denote the value of the left reference pixel corresponding to target pixel A at co-ordinate (x,y). Pleft is obtained as follows:

deltaPos = ((1+x)invAngle + 2) >> 2;

deltaInt = deltaPos >> 6;

deltaFrac = deltaPos & 63;

deltay = 1 + y + deltaInt;

Pleft =refSide[deltay+(deltaFrac>>5)]

If Ppred(x,y) denotes the initial predicted value, the final predicted value at pixel A is obtained as

Px,y=Clip64-wL*Ppredx,y+ wL*Pleft+32 ≫6, ……………………………(2)

where wL = 32 >> min(31, ((x << 1) >> scale)).

For a target pixel, if the left reference sample on refSide is located beyond its length, its initial predicted value remains unchanged.

It is apparent from methods (1) and (2) above that they are different. Furthermore method (1) seems more complex. It is shown in the following that method (1) can be simplified to method (2).

# Proposed solutions

Referring to Figure 1, it is observed that,  the predicted value for any target pixel is equal to the top reference pixel for that pixel. That is, Ppred is equal to Ptop.  Substituting the value of Ptop in method  (1) ,

Px,y=Clip64-wT-wL*Ppredx,y+ wT*Ppred(x,y)+wL*Pleft+32 ≫6.

Upon cancellation of the term wT*Ppredx,y, the above expression simplifies to

Px,y=Clip64-wL*Ppredx,y+ wL*Pleft+32 ≫6,

where wL = 16 >> min(31, ((x << 1) >> scale));

This expression is identical to Method (2) except for the value of the weight wL. It is seen that  there is no need to compute wT, nor the term Ptop, accordingly the term wT*Ptop.  It is now possible to merge the case of PDPC for mode 2 and mode 66 with the case of PDPC for their adjacent modes:

Px,y=Clip64-wL*Ppredx,y+ wL*Pleft+32 ≫6, …………………….(3)

where wL = wLmax >> min(31, ((x << 1) >> scale)), where wLmax = 16 if intraPredAngle = 32, wLmax = 32, otherwise.

It is also to note that the position of the left reference sample for mode 2 or 66 does not change even if the two cases are merged. As the value of invAngle for mode 2 and 66 is equal to 256,

deltaPos = ((1+x)*256 + 2) >> 2 = (1+x)*64;

deltaInt = deltaPos >> 6 = (1+x);

deltaFrac = deltaPos & 63 = 0;

deltay = 1 + y + deltaInt = 1 + y + 1 + x;

Pleft = refSide[1 +y+1+x] =refSide[c+1] where c = 1+x+y;

Thus, the above merging will result in identical results as the current VTM 5.0. As a matter of fact, the term deltaFrac really does not serve any purpose. In the initial proposal for PDPC [1], deltaFrac was included as the position of the left reference sample was linearly interpolated based on deltaFrac’s value. In a subsequent JVET meeting, the linear interpolation was replaced by the nearest neighbor [2]. However, for this, the computation of the deltaFrac term is not necessary.

The derivation of the left reference sample involves computing the distance deltaPos:

deltaPos = ((1+x)invAngle + 2) >> 2;

In VTM 5.0, deltaPos is computed as a recursive sum, which eliminates the multiplication by x:

invAngleSum = 2;

For 0 <=x < W

invAngleSum += invAngle;

deltaPos = invAngleSum >> 2;

deltaInt = deltaPos >> 6;

deltaFrac = deltaPos & 63;

deltay = 1 + y + deltaInt;

Pleft =refSide[deltay+(deltaFrac>>5)]

These computations can be simplified as

invAngleSum = 128;

for 0 <= x < W

invAngleSum += invAngle;

deltay= 1 + y + (invAngleSum >> 8),

Pleft =refSide[deltay]

In the above simplification,  the following three steps have been merged into one:

deltaPos = invAngleSum >> 2,

deltaInt = deltaPos >> 6

deltay = 1 + y + deltaInt;

Initializing invAngleSum with 128 instead of 2 results in the nearest integer automatically after the bit-shifting operation  (invAngleSum >> 8). This avoids calculating the parameter deltaFrac. Furthermore, as a side effect, the last reference pixel on refSide, that is, refSide[2H] will also be used as Pleft  when the actual position of the left reference sample is between 2H and 2H+1, and is closer to refSide[2H]. In the current PDPC implementation, this case will be simply skipped as deltay > 2H, and consequently the initial predicted value of the current pixel will remain unchanged. Except for this special case, the proposed simplification will result in identical predicted values for target pixels as the VTM 5.0.

Method (3) can be simplified with the observation that the weighted sum does not require a clipping:

Px,y= 64-wL*Ppredx,y+ wL*Pleft+32 ≫6

Since wL is also a power of 2 when non-zero, this can be equivalently expressed as:

Px,y=  Ppredx,y+((((Pleft-Ppredx,y)≪ log2wL)+32)≫6)…………….(4)

which does not require any multiplication operation.

# Experimental Results

To ensure that the simplifications are correct, two tests were conducted. In the first test, PDPC for diagonal modes 2 and 66 was merged with that for adjacent diagonal modes but Pleft was still computed using deltaFrac. In the second test, further simplifications were done without using deltaFrac.  In both tests, the simplified method in (4) was used to derive the final predicted value. Simulations were performed according to CTC described in [3]. AI and RA results are reported below.  The results of Test 1 confirm that the BD-rate performances after merging  are identical  to those in VTM 5.0, but the merging leads to less encoding and decoding times. Test 2 results show that further simplifications result in almost the same BD-rate performances and encoding and decoding times.

## Test 1

## Test 2

# Conclusion

This contribution document reports the results of  merging the PDPC for diagonal directions with that for adjacent diagonal directions. Further simplifications show that the PDPC for both diagonal and adjacent diagonal modes do not require  any multiplication operation or clipping.   Based on the experimental results, it can be concluded that the proposed merging leads to identical BD-rate performances with reduced encoding and decoding times.  Thus it is proposed to replace the existing PDPC for diagonal and adjacent diagonal modes in VTM 5.0.

# References

- “Simplified PDPC,” G. V. der Auwera, V. Seregin, A. Said, A. K. Ramasubramonian, and M. Karczewicz, JVET-K0063, Ljubljana, SI, July 2018.

- “Simplification of PDPC,” J. Lee, H. Lee, S.-C. Lim, J. Kang, and H. Y. Kim, JVET-L0152, Macao, CN, Oct. 2018.

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, 13th Meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).