| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0230 |
| --- | --- |

| Title: | CE3-related: Interpolation filtering for intra prediction in non-diagonal directions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Fabien Racapé, Fabrice Urban, Fabrice Le Léannec | Tel: Email: | +33.2.56.85.62.76 gagan.rath@InterDigital.com fabien.racape@InterDigital.com fabrice.urban@InterDigital.com fabrice.leleannec@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

For intra prediction along non-diagonal directions, this contribution reports the results of using simple linear approximations for interpolation with four reference samples instead of the 4-tap Gaussian and cubic filters. Since models using four samples cannot model object boundaries accurately, it is also proposed to use the conventional interpolation with two reference samples if their magnitudes are relatively close. Besides BD-rate performance gain, the advantages of the proposed method are lower complexity, no memory storage of filter coefficients, and a uniform method for both Luma and Chroma components. Besides reduced encoding and decoding times, BD-rate gains of 0.09% for Luma and 0.25% for chroma are reported.

# Introduction

# In VTM 5.0, for intra prediction along non-diagonal directions except the horizontal and vertical directions, the predicting reference samples are interpolated using either a 4-tap Gaussian filter or a 4-tap cubic filter. Though such an interpolation leads to higher coding gain than the conventional linear interpolation, it comes at the expense of additional complexity, and memory requirements for storing the 32x4 cubic and Gaussian filter coefficient arrays. This contribution proposes to use simpler linear models with four reference samples instead of the 4-tap Gaussian and the cubic filters. As the VTM5.0 already uses the conventional linear interpolation for chroma components, the proposed method is applied to both Luma and chroma components.

# Problem definition

For the sake of generality, a rectangular target block with width W pixels and height H pixels is considered. Let refMain and refSide represent the reference arrays on the top and the left of the target block, respectively. For horizontal prediction modes, that is, if mode < DIA_IDX,  refMain and the refSide denote the top and the left reference arrays after they have been swapped. Since this contribution deals with only non-diagonal directions, both refMain and refSide consist of unfiltered decoded samples in VTM 5.0.

For any target pixel at (x,y), the predictor reference sample on refMain is identified through two parameters deltaInt and deltraFract, which are determined as deltaPos = (1+y) * predIntraAngle, deltaInt = deltaPos >> 5, and deltraFract = deltaPos & 31, where predIntraAngle corresponds to the prediction mode.

Let  P0, P1, P2, and P3 denote the four reference samples nearest to the predictor reference sample, as shown in Figure 1.  They are identified as:P0 ≡ refMain[x+deltaInt],  P1 ≡ refMain[x+deltaInt+1],  P2 ≡ refMain[x+deltaInt + 2],  and P3 ≡refMainx+deltaInt +3.

Figure 1: interpolation of predictor for intra prediction in non-diagonal directions. P0, P1, P2, and P3 are the nearest reference samples which are used to interpolate the predictor with either a Gaussian or a cubic filter for the Luma component. For the Chroma component, the predictor is linearly interpolated using P1 and P2.

For a Luma target block, the predictor sample value is computed as:

Ppred(x,y) = (F[0] * P0 + F[1] * P1 + F[2] * P2 + F[3] * P3 + 32) >> 6 …… (1)

where F[i] denotes the ith filter coefficient. The chosen filter depends on the value deltaFract and a Boolean parameter called useFilter. If useFilter is enabled, then a Gaussian filter is used, else a cubic filter is used. The value of the useFilter is decided based on several intra prediction parameters such as the block size,  the prediction direction, MRL index, etc. As the specified cubic filter has both negative and positive coefficient values, when the useFilter is disabled, a clipping operation is additionally performed to keep the predicted value within the valid dynamic range of the Luma component:

Ppred(x,y) = Clip((F[0] * P0 + F[1] * P1 + F[2] * P2 + F[3] * P3 + 32) >> 6) …… (2)

For a chroma target block, only the two reference samples P1 and P2 are used for interpolating the predictor. The predictor value is linearly interpolated, as in HEVC:

Ppred(x,y) = ((32-deltaFract) * P1 +deltaFract * P2+  16) >> 5; …… (3)

Since deltaFract has integral values in the range [0-31], VTM 5.0 includes 32 Gaussian and 32 cubic filters corresponding to  32 deltaFract values, which are pre-computed and stored in memory.  It is clear that each Luma target block requires four multiplications per pixel.

# Proposed solutions

As the reference arrays refMain and refSide are unfiltered, to distinguish between the two cases when the flag useFilter is enabled (a Gaussian filter is used in VTM 5.0) , and when it is disabled (a cubic filter is used in VTM 5.0),  the same filter, i.e. [1 2 1]/4, is applied to P1 and P2 when  useFilter if enabled.

Thus, if useFilter flag is true

P1 = (refMain[x + deltaInt] + 2*refMain[x + deltaInt + 1] + refMain[x + deltaInt + 2] + 2) >> 2 ;

P2 = (refMain[x + deltaInt+1] + 2* refMain[x + deltaInt + 2]+ refMain[x + deltaInt + 3] + 2) >> 2 ;

This step would not be necessary if filtered versions refMain and refSide were used for prediction in the case useFilter is enabled.

In the first method, the predictor value is simply computed as:

Ppred(x,y) = Clip(P1  + P2 – P1<< 2* deltaFract + P1 + P2 – P0 – P3 * deltaFract2  + 64>> 7)

where deltaFract2=deltaFract  if deltaFract<16, deltaFract2= 32-deltaFract , otherwise.

In the second method, the absolute difference between P1 and P2 is compared with a pre-set threshold. If the value is less than the  threshold, the conventional linear interpolation is performed using P1 and P2, else the predictor is interpolated as above. Denoting the preset threshold by Th,

If |(P1-P2)| <= Th

Ppred(x,y) = (P1 * (32 – deltaFract)   + P2 * deltaFract + 16) >> 5;

Else

Ppred(x,y) = Clip(P1  + P2 – P1<< 2* deltaFract + P1 + P2 – P0 – P3 * deltaFract2  + 64>> 7)

The interpolation is derived from the piece-wise linear approximation of a 2nd order polynomial that uses four samples p0, p1, p2, and p3 , as shown in Figure 2.

Figure 2: Piece-wise linear approximation of a quadratic function. In each half of the interval [0, 1], the function is approximated with a straight line, where the slope of the line depends on the four given sample values p0, p1, p2, and p3.

# Experimental Results

Simulations were performed according to CTC described in [1]. AI and RA results are reported below.  Test 1 and Test 2 results correspond to the first  and the second  methods, respectively. In Test 2, the threshold values used for the Luma and Chroma components were 16 and 8 respectively. It is interesting to observe that Method 2 results in BD-rate gain for all test classes. Looking at the results for individual test sequences, it is also observed that, Method 2 yields Luma BD-rate gain for almost all test sequences under AI configuration.

## Test 1

## Test 2

# Conclusion

This contribution reports the results of using  simple linear approximations for interpolation in intra prediction along non-diagonal directions. The approximations are proposed to replace the Gaussian and the cubic filtering for Luma and the conventional linear interpolation for chroma, in VTM 5.0. Experimental results show  BD-rate gains of 0.01% for Luma and 0.25% for Chroma over VTM 5.0 in AI configuration when no threshold is used. With threshold, BD-rate gains of 0.09% for Luma and 0.25% for Chroma are reported. Both cases report reduced encoding and decoding times. Considering this, and other advantages, Method 2 is proposed for intra prediction in VTM as a unified method for Luma and Chroma.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, 13th Meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).