| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0233 |
| --- | --- |

| Title: | CE2-related: Linear LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In VTM5, the luma mapping with chroma scaling (LMCS) requires in-loop mapping of the luma component based on up to 16 adaptive piecewise linear models. For the chroma components, luma-dependent chroma residual scaling needs to be applied, and it causes dependency between luma and chroma. This contribution proposes a simplified version of LMCS for SDR, referred as linear LMCS, with only one linear piece between the start and end flat pieces. The benefit is that it reduces the number of the inverse scale factor derivation (which requires division) and does not need to identify piece index for chroma residual scaling. This reduces the complexity of luma inverse mapping and chroma residual scaling. In addition, it removes dependency of chroma residual scaling on corresponding luma block, and therefore eliminates the latency problem of chroma residual scaling.

With linear LMCS, it achieves performance close to LMCS in VTM5. Results are 0.05%, 0.93%, 1.05% for Y,U,V respectively in AI configuration; 0.04%, -0.05%, -0.07% for Y, U, V respectively in RA configuration, and 0.11%, -0.16%, -0.29% for Y, U, V respectively in LD configuration.

Results show that for SDR, the code word variations among pieces are not the main contributor to the gain. The min and max indices and overall code word of LMCS resulted in majority of the gain. Linear LMCS has smaller signalling overhead. This makes it feasible to have more frequent update of LMCS model and potentially can get more gains.

# Introduction

In VTM5, luma mapping with chroma scaling (LMCS) is added as a new processing block before the loop filters. LMCS has two main components: 1) in-loop mapping of the luma component based on adaptive piecewise linear models; 2) for the chroma components, luma-dependent chroma residual scaling is applied. The in-loop mapping of the luma component adjusts the dynamic range of the input signal by redistributing the codewords across the dynamic range to improve compression efficiency. Luma mapping makes use of a forward mapping function, FwdMap, and a corresponding inverse mapping function, InvMap. The FwdMap function is signalled using a piecewise linear model with 16 equal pieces. InvMap function does not need to be signalled and is instead derived from the FwdMap function.

The 16 PWL model are represented by 16 bins and 17 pivots points. In forward mapping, the input original luma values are equally divided into 16 bins. For bin_idx < lums_min_bin_idx  and bin_idx > LmcsMaxBinIdx, delta code words are 0, so the start and end pieces are flat. Code words for bins between lmcs_min_bin_idx and LmcsMaxBinIdx may have some variations.

An example is shown in Fig. 1 below. It shows the actual LMCS forward and inverse mapping of the first frame for MarketPlace coded at QP 37 in RA. In this example, for the forward mapping, there are 16 bins and 17 pivot points, where lmcs_min_bin_idx=1 and LmcsMaxBinIdx =14. In Fig 1, labels p1 and p15 mark the 1st and 15th pivot points (i.e. pivot point at lmcs_min_bin_idx and LmcsMaxBinIdx +1). For simplicity, pivot point at lmcs_min_bin_idx and LmcsMaxBinIdx +1 are referred as p_min and p_max.

In inverse mapping, the input is reshaped value, those are not equally spaced. Therefore inverse mapping requires an iterative process to find the piece index. Chroma residual scaling factor depends on the piece index in which the corresponding luma value belongs to, therefore it results in latency of chroma processing. The piecewise index identification process is as described 8.8.2.3 in VVC5 draft text [1] .

The variable idxYInv is derived as follows:

if ( lumaSample < LmcsPivot[ lmcs_min_bin_idx + 1 ] )
	idxYInv = lmcs_min_bin_idx
else if ( lumaSample >= LmcsPivot[ LmcsMaxBinIdx ] ) 
	idxYInv = LmcsMaxBinIdx
else {					
	for( idxYInv = lmcs_min_bin_idx; idxYInv < LmcsMaxBinIdx; idxYInv++ ) {
		if( lumaSample < LmcsPivot [ idxYInv + 1 ] )
			break
	}
}

For each of the 16 bin, a scale factor and an inverse scale factor are derived. Inverse scale factor derivation requires up to 16 divisions.

LmcsPivot[ 0 ] = 0;
for( i = 0; i <= 15; i++ ) {
	  LmcsPivot[ i + 1 ] = LmcsPivot[ i ] + lmcsCW[ i ] 
	  ScaleCoeff[ i ] = ( lmcsCW[ i ] * (1 << 11) + (1 << (Log2(OrgCW) − 1))) >> (Log2(OrgCW)) 	(7-93)
	  if ( lmcsCW[ i ] = = 0 )
		 InvScaleCoeff[ i ] = 0
	  else
		InvScaleCoeff[ i ] = OrgCW * (1 << 11) / lmcsCW[ i ]
}

# Proposed Method

This contribution proposes to use a simplified version of LMCS model for SDR with a linear line between LMCS beginning and end flat pieces. The linear LMCS can be regarded as a special case of current 16-piece PWL LMCS model. This is motivated by the observations that for SDR sequences, many of the LMCS PWL mappings are close to straight lines between the lmcs_min_bin_idx and (LmcsMaxBinIdx + 1) pivot points in the forward mapping. This is shown in the example in Fig. 1. The code word variations among pieces are small.

Illustrations of proposed LMCS are shown in Fig. 2. It keeps the start and end two flat pieces, and only make the portion between pivot lmcs_min_bin_idx and LmcsMaxBinIdx+1 linear. In our test, we mainly reused the existing LMCS encoder-side parameter estimation, and did a linear approximation to the PWL mapping determined by current LMCS encoder algorithm.

The linear approximation is done as following. Keep p_min the same, and only adjust the mapped value at p_max, the pivot point after adjustment is denoted as pL. PL is determined by minimizing the weighted error between the PWL pivot points and the virtual pivot point if connecting the p_min and p_max. The weight is based on histogram of a piece. Histogram is already known during LMCS SDR statistics collection. The linear approximation line is then equally divided into (LmcsMaxBinIdx- lums_min_bin_idx +1) pieces, then it can easily reuse existing 16-piece LMCS structure.

Fig. 2 Illustrations of linear LMCS model

The benefit of linear LMCS is that now each piece will have same scale and inverse scale. So the scale and inverse scale only need to be calculated once per LMCS model.

For forward mapping, all the pieces between p1 and pL have same scale factor. They are all equal to

ScaleCoeffSingle = ( lmcsCW[ lmcs_min_idx+1 ] * (1 << 11) + (1 << (Log2(OrgCW) − 1))) >> (Log2(OrgCW))

For inverse mapping, there is only one scale factor between p1 and pL. They are all equal to

InvScaleCoeffSingle = OrgCW * (1 << 11) / lmcsCW[ lmcs_min_idx+1  ]

InvScaleCoeff[ i ]  = InvScaleCoeffSingle

In our test, to reuse the existing LMCS structures, the inverse luma sample derivation is kept unchanged:

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * 
	( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

, while now all InvScaleCoeff[ i ] are the same.

An alternative way to derive the inverse luma sample is to always use InputPivot[ lmcs_min_idx ] as the starting point. Then the piecewise index identificaton process may be removed too.

For Chroma residual scaling, with single scale factor, chroma residual scaling does not depend on corresponding luma block, therefore it eliminates the latency problem of chroma residual scaling. In addition it reduces chroma residual scaling complexity since there is no need to identify the piecewise index of corresponding luma block and the process of mapping and calculating the average of corresponding luma block can be removed.

## Signal the Linear LMCS

Linear LMCS is targeted to SDR content. It is not applied to HDR content. Currently, a flag lmcs_is_linear is added to lmcs_data to enable linear LMCS or not. In the future, it may be enabled based on profile. e.g. SDR or HDR profile. When linear LMCS is enabled, only one set of lmcsCW[i] information is signaled. The syntax table is shown in Table 1.

Table 1. syntax table for signaling linear LMCS

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_is_linear_flag | u(1) |
| lmcs_delta_cw_linear_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinEnd; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] |  |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |

lmcs_min_bin_idx specifies the minimum bin index used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive.

lmcs_delta_max_bin_idx specifies the delta value between 15 and the maximum bin index LmcsMaxBinIdx used in the luma mapping with chroma scaling construction process. The value of lmcs_delta_max_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMaxBinIdx is set equal to 15 − lmcs_delta_max_bin_idx. The value of LmcsMaxBinIdx shall be greater than or equal to lmcs_min_bin_idx.

lmcs_is_linear_flag equal to 1 specifies that LMCS model is linear model, otherwise, equal to 0 specifies that LMCS model is regular 16-piece PWL model.

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive

Variables LmcsMaxBinEnd is derived as below:

if (lmcs_is_linear_flag==1)

LmcsMaxBinEnd  = lmcs_min_bin_idx

else

LmcsMaxBinEnd  = LmcsMaxBinIdx

When lmcs_is_linear_flag is true, the start index and end index of the loop to receive lmcs_delta_abs_cw[ i ]  are the same, therefore the loop is only entered once, only one set of delta codeword  is received, and is stored in lmcs_delta_abs_cw[lmcs_min_bin_idx ] and lmcs_delta_sign_cw[lmcs_min_bin_idx].

lmcs_delta_abs_cw[ i ] specifies the absolute delta codeword value for the ith bin.

lmcs_delta_sign_cw_flag[ i ] specifies the sign of the variable lmcsDeltaCW[ i ]  as follows:

- If lmcs_delta_sign_cw_flag[ i ] is equal to 0, lmcsDeltaCW[ i ] is a positive value.

- Otherwise ( lmcs_delta_sign_cw_flag[ i ] is not equal to 0 ), lmcsDeltaCW[ i ] is a negative value.

When lmcs_delta_sign_cw_flag[ i ] is not present, it is inferred to be equal to 0.

The variable OrgCW is derived as follows:

OrgCW = (1 << BitDepthY ) / 16

The variable lmcsDeltaCW[ i ], with i = lmcs_min_bin_idx..LmcsMaxBinIdx, is derived as follows:

if lmcs_is_linear_flag is true,

lmcsDeltaCW[i] = ( 1 − 2 * lmcs_delta_sign_cw_flag[lmcs_min_bin_idx] ) *        lmcs_delta_abs_cw[lmcs_min_bin_idx]

else

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_delta_sign_cw_flag[ i ] ) * lmcs_delta_abs_cw[ i ]

Linear signaling has less overhead than current 16-piece PWL model. It makes it feasible to enable more frequent update of LMCS model, and can potential have more gains.

When lmcs_is_linear_flag is true, all the lmcsDeltaCW[ i ] between lmcs_min_bin_idx  and LmcsMaxBinIdx  are having the same values. This leads to lmcsCW[ i ] of all pieces between lmcs_min_bin_idx  and LmcsMaxBinIdx have the same values. Scale and inverse scale and Chroma scale of all bins between lmcs_min_bin_idx and lmcsMaxBinIdx are the same.

# Experimental Results

Proposed linear LMCS has been implemented into VTM5.0 and tested following CTC test condition [1]. Results are as below.

Table 2. Experimental results of linear LMCS

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 1.57% | 2.12% | 102% | 101% |
| Class A2 | -0.09% | 1.90% | 1.93% | 102% | 102% |
| Class B | 0.13% | 1.31% | 1.41% | 100% | 102% |
| Class C | 0.06% | -0.49% | -0.54% | 99% | 99% |
| Class E | 0.03% | 0.56% | 0.64% | 100% | 99% |
| Overall | 0.05% | 0.93% | 1.05% | 101% | 101% |
| Class D | 0.03% | -0.43% | -0.68% | 99% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.39% | 0.62% | 101% | 101% |
| Class A2 | -0.14% | 0.62% | 0.52% | 100% | 101% |
| Class B | 0.04% | -0.25% | -0.22% | 100% | 101% |
| Class C | 0.20% | -0.65% | -0.83% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.05% | -0.07% | 100% | 100% |
| Class D | 0.00% | 0.05% | 0.18% | 99% | 100% |
| Class F | 0.17% | -0.41% | -0.44% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.15% | -0.17% | 100% | 101% |
| Class C | 0.16% | -0.24% | -0.45% | 99% | 102% |
| Class E | -0.02% | -0.59% | -0.29% | 100% | 103% |
| Overall | 0.11% | -0.16% | -0.29% | 100% | 102% |
| Class D | 0.02% | -0.44% | 0.60% | 99% | 101% |
| Class F | 0.14% | 0.01% | 0.27% | 100% | 103% |

As shown the results, with linear LMCS, it can achieve gain very close to VTM5. That means for SDR, Codeword variation among the pieces are not the main contributor to the gain of LMCS tool, the min and max indices and overall code word resulted in the majority of the gain.

# Conclusion

This contribution proposes to simplify LMCS with linear model LMCS. This linear LMCS has only one forward, inverse and chroma residual scale factor per picture. It simplifies the inverse luma mapping and chroma residual scaling processing, and also eliminates the dependency problem of Chroma residual scaling on corresponding luma block. Linear LMCS has less signalling overhead, this makes it feasible to enable more frequent update of LMCS model and can potential get more gains.

More importantly, the linear LMCS helps us understand for SDR where the gain of LMCS tool comes from. With linear LMCS, it achieves performance close to LMCS in VTM5. Results are 0.05%, 0.93%, 1.05% for Y,U,V respectively in AI configuration; 0.04%, -0.05%, -0.07% for Y, U, V respectively in RA configuration, and 0.11%, -0.16%, -0.29% for Y, U, V respectively in LD configuration.

Results show that Code word variations among the pieces are not the main contributor to the gain of LMCS tool on SDR, the min and max indices and overall code word resulted in majority of the gain. We propose to adopt the linear LMCS into VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N0010, Geneva, CH, Mar 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).