#### Luma mapping with chroma scaling data syntax

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_is_linear_flag | u(1) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <=LmcsMaxBinEnd ; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

#### Luma mapping with chroma scaling data semantics

lmcs_min_bin_idx specifies the minimum bin index used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive.

lmcs_delta_max_bin_idx specifies the delta value between 15 and the maximum bin index LmcsMaxBinIdx used in the luma mapping with chroma scaling construction process. The value of lmcs_delta_max_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMaxBinIdx is set equal to 15 − lmcs_delta_max_bin_idx. The value of LmcsMaxBinIdx shall be greater than or equal to lmcs_min_bin_idx.

lmcs_is_linear_flag equal to 1 specifies that LMCS model is linear model, otherwise, equal to 0 specifies that LMCS model is regular 16-piece PWL model.

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive.

Variables LmcsMaxBinEnd is derived as below:

if (lmcs_is_linear_flag==1)

LmcsMaxBinEnd  = lmcs_min_bin_idx

else

LmcsMaxBinEnd  = LmcsMaxBinIdx

When lmcs_is_linear_flag is true, the start index and end index of the loop to receive lmcs_delta_abs_cw[ i ]  are the same, therefore the loop is only entered once, only one set of delta codewords  is received, and is stored in lmcs_delta_abs_cw[lmcs_min_bin_idx ] and lmcs_delta_sign_cw[lmcs_min_bin_idx].

lmcs_delta_abs_cw[ i ] specifies the absolute delta codeword value for the ith bin.

lmcs_delta_sign_cw_flag[ i ] specifies the sign of the variable lmcsDeltaCW[ i ]  as follows:

- If lmcs_delta_sign_cw_flag[ i ] is equal to 0, lmcsDeltaCW[ i ] is a positive value.

- Otherwise ( lmcs_delta_sign_cw_flag[ i ] is not equal to 0 ), lmcsDeltaCW[ i ] is a negative value.

When lmcs_delta_sign_cw_flag[ i ] is not present, it is inferred to be equal to 0.

The variable OrgCW is derived as follows:

OrgCW = (1 << BitDepthY ) / 16	(7-88)

The variable lmcsDeltaCW[ i ], with i = lmcs_min_bin_idx..LmcsMaxBinIdx, is derived as follows:

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_delta_sign_cw_flag[ i ] ) * lmcs_delta_abs_cw[ i ]	(7-89)

The variable lmcsCW[ i ] is derived as follows:

- For i = 0.. lmcs_min_bin_idx − 1, lmcsCW[ i ] is set equal 0.

- For i = lmcs_min_bin_idx..LmcsMaxBinIdx, the following applies:

lmcsCW[ i ] = OrgCW + lmcsDeltaCW[ i ]	(7-90)

The value of lmcsCW[ i ] shall be in the range of (OrgCW>>3) to (OrgCW<<3 − 1), inclusive.

- For i = LmcsMaxBinIdx + 1..15, lmcsCW[ i ] is set equal 0.

It is a requirement of bitstream conformance that the following condition is true:

i=015lmcsCW[ i ] <= (1 << BitDepthY ) − 1	(7-91)

The variable InputPivot[ i ], with i = 0..16, is derived as follows:

InputPivot[ i ] = i * OrgCW	(7-92)

The variable LmcsPivot[ i ] with i = 0..16, the variables ScaleCoeff[ i ] and InvScaleCoeffSingle, InvScaleCoeff[ i ] with i = 0..15, are derived as follows:

LmcsPivot[ 0 ] = 0;

if (lmcs_is_linear_flag==1)

InvScaleCoeffSingle = OrgCW * (1 << 11) / lmcsCW[ lmcs_min_bin_idx ]
for( i = 0; i <= 15; i++ ) {
	LmcsPivot[ i + 1 ] = LmcsPivot[ i ] + lmcsCW[ i ] 
	ScaleCoeff[ i ] = ( lmcsCW[ i ] * (1 << 11) + (1 << (Log2(OrgCW) − 1))) >> (Log2(OrgCW)) 	(7-93)
	if ( lmcsCW[ i ] = = 0 )
		InvScaleCoeff[ i ] = 0

else if (lmcs_is_linear_flag==1)

InvScaleCoeff[ i ] = InvScaleCoeffSingle
	else
		InvScaleCoeff[ i ] = OrgCW * (1 << 11) / lmcsCW[ i ]
}

The variable ChromaScaleCoeff[ i ], with i = 0…15, is derived as follows:

if (lmcs_is_linear_flag==1)

ChromaScaleCoeffSingle = InvScaleCoeffSingle

else {

if ( lmcsCW[ i ] = = 0 )
	ChromaScaleCoeff[ i ] = (1 << 11)
    else					(7-94)
	ChromaScaleCoeff[ i ] = InvScaleCoeff[ i ]

}

#### Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the transform block width,

- a variable nCurrSh specifying the transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

Output of this process is a reconstructed chroma picture sample array recSamples.

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one of the following conditions is true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1C( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- slice_chroma_residual_scale_flag is equal to 0

- nCurrSw * nCurrSh is less than or euqal to 4

- tu_cbf_cb [ xCurr ][ yCurr ] is equal to 0 and tu_cbf_cr [ xCurr ][ yCurr ] is equal to 0

- Otherwise, the following applies:

- if lmcs_is_linear_flag is true, the variable varScale is set to ChromaScaleCoeffSingle

- Otherwise the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

invAvgLuma = Clip1Y( ( k=0SubWidthC*nCurrSw-1l=0SubHeightC*nCurrSh-1PredMapSamples[ k ][ l ]
	 + nCurrSw * nCurrSh * ( ( SubWidthC * SubHeightC ) / 2 ) ) / 	(8-996)
		( nCurrSw * nCurrSh *SubWidthC * SubHeightC ) )

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with the variable lumaSample set equal to invAvgLuma as the input and idxYInv as the output.

- The variable varScale is derived as follows:

varScale = ChromaScaleCoeff[ idxYInv ]	(8-997)

- The recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepthC ), ( 1 << BitDepthC ) − 1, resSamples[ i ][ j ] )	(8-998)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C( predSamples[ i ][ j ] + 	(8-999)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tu_cbf is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C(predSamples[ i ][ j ] )	(8-1000)

