| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0234 |
| --- | --- |

| Title: | AHG17: Slice header extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal claims that the JVET-N1001-v8 VVC specification includes extension fields at the end of the parameter set syntax tables for the DPS, VPS, SPS, PPS and APS. This is claimed to be similar to HEVC which also includes a slice header extension mechanism. However, this proposal points out that slice header extension is not part of the VVC specification and proposes to include it. The proponents state that a slice header extension feature may prove to be useful in future versions of VVC and point out that HEVC makes use of the slice header extension in Annex F “Common specifications for multi-layer extensions” and Annex I “3D high efficiency video coding”.

Similar to HEVC, the extension field is proposed to be preceded by a length parameter in order to provide a version 1 VVC decoder with the information needed to find the start of the slice data. The presence of the extension field in the slice header is proposed to be conditioned on a flag in the PPS that has to be set to 0 in bitstreams conforming to the first version of VVC, to not add any bit overhead in the slice headers.

The proponents claim that the proposal is essentially identical to the HEVC slice header extension mechanism.

Specification text on top of JVET-N1001-v8 is attached with the contribution.

# Introduction

In the JVET-N1001-v8 VVC specification, all parameter sets have an extension field at the end of their syntax table. The presence of the extension fields is controlled by extension data flags.

These extension fields provide VVC with the possibility to add new syntax elements to the parameter sets in the future while still being backwards compatible with legacy decoders as they will simply ignore the new syntax elements. Since the NumBytesInRBSP is known for all parameter sets there is no problem for a legacy decoder to calculate the length of an extension field (or detect the end of it).

Example from sequence parameter set:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Problem

The slice header of VVC does not contain an extension field.  In future extensions of VVC it might be useful to be able to add syntax elements to the slice header that a legacy decoder ignores but a decoder that conforms to the extension makes use of. It would be an advantage to be able to add such syntax elements as straight-forward as possible (e.g. without having to add prefix NAL units, new NAL unit types or SEI messages).

However, it is not possible to simply add an extension field to the slice header in the same manner as in the parameter sets since the length of the slice header is not known. A slice consists of a slice header and slice data and it is only the total length of the slice that is known (through NumBytesInRBSP) not the individual lengths of the slice header and slice data respectively. Thus, if new slice header syntax elements were added in an extension to VVC, a legacy decoder would not be able to conclude where the slice header ends and the slice data begins.

Slice layer RBSP syntax:

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

Another problem with extensibility in the slice header is the bit cost. Having to spend even a single bit can be somewhat costly if it must be done in all slice headers in all VVC bitstreams.

# Proposed solution

We propose to add a slice header extension field similar to the HEVC design. The slice header extension field is proposed to be conditioned on a flag in the PPS and to include a length parameter representing the length of the actual slice header extension data.

Proposed additional syntax element in PPS:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Proposed slice header extension:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

The length parameter provides the legacy decoder with sufficient information for finding the start of the slice data. Putting the slice header extension present flag in the PPS makes the bit cost for the extensibility very small. For bitstreams that conform to the current profile(s) of VVC the cost will be a single bit per PPS.

The proposed semantics for the three new syntax elements are in line with the semantics of the extension fields of the parameter sets:

slice_header_extension_present_flag equal to 0 specifies that no slice header extension syntax elements are present in the slice headers for coded pictures referring to the PPS. slice_header_extension_present_flag equal to 1 specifies that slice header extension syntax elements are present in the slice headers for coded pictures referring to the PPS. slice_header_extension_present_flag shall be equal to 0 in bitstreams conforming to this version of this Specification.

slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling slice_header_extension_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive. When not present, the value of slice_header_extension_length is inferred to be equal to 0.

slice_header_extension_data_byte may have any value. Decoders conforming to this version of this Specification shall ignore the value of slice_header_extension_data_byte. [Ed. (RSS): One of the following sentences should be considered if profiles are specified in a future version of VVC: “Its value does not affect decoder conformance to profiles specified in Annex A” or “Its value does not affect decoder conformance to profiles specified in this version of this Specification”]

# Comparison of the proposal with HEVC version 5 (02/2018)

| HEVC version 5 | This proposal |
| --- | --- |
| pic_parameter_set_rbsp( ) {. .... slice_segment_header_extension_present_flag; u(1). slice_segment_header( ) {. .... if(slice_segment_header_extension_present_flag) {. slice_segment_header_extension_length; ue(v). for(i=0;i<slice_segment_header_extension_length;i++). slice_segment_header_extension_data_byte[ i ]; u(8). }. .... | pic_parameter_set_rbsp( ) {. .... slice_header_extension_present_flag; u(1). slice_segment_header( ) {. .... if(slice_header_extension_present_flag) {. slice_header_extension_length; ue(v). for(i=0;i<slice_header_extension_length;i++). slice_header_extension_data_byte[ i ]; u(8). }. .... |
| slice_segment_header_extension_present_flag equal to 0 specifies that no slice segment header extension syntax elements are present in the slice segment headers for coded pictures referring to the PPS. slice_segment_header_extension_present_flag equal to 1 specifies that slice segment header extension syntax elements are present in the slice segment headers for coded pictures referring to the PPS. | slice_header_extension_present_flag equal to 0 specifies that no slice header extension syntax elements are present in the slice headers for coded pictures referring to the PPS. slice_header_extension_present_flag equal to 1 specifies that slice header extension syntax elements are present in the slice headers for coded pictures referring to the PPS. slice_header_extension_present_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. |
| slice_segment_header_extension_length specifies the length of the slice segment header extension data in bytes, not including the bits used for signalling slice_segment_header_extension_length itself. The value of slice_segment_header_extension_length shall be in the range of 0 to 256, inclusive. When not present, the value of slice_segment_header_extension_length is inferred to be equal to 0. | slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling slice_header_extension_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive. When not present, the value of slice_header_extension_length is inferred to be equal to 0. |
| slice_segment_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in Annex A. | slice_header_extension_data_byte may have any value. Decoders conforming to this version of this Specification shall ignore the value of slice_header_extension_data_byte. [Ed. (RSS): One of the following sentences should be considered if profiles are specified in a future version of VVC: “Its value does not affect decoder conformance to profiles specified in Annex A” or “Its value does not affect decoder conformance to profiles specified in this version of this Specification”] |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).