| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0236-v1 |
| --- | --- |

| Title: | AHG17: Making PPS parsing independent of SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The proponents of this contribution have the opinion that keeping the PPS parsing independent of the SPS is a desirable property in VVC assuming VVC will end up in an HEVC-like process for parameter set activation where parameter sets are activated by VCL NAL units. If the parsing of the PPS then depends on the SPS, PPSes may need to be stored in coded format by a decoder until a VCL NAL unit triggers a PPS activation.

If the VVC design would differ from the HEVC design and allow parsing dependencies between the PPS and SPS, the proponents do not propose to make any of the changes described in this contribution.

Two claimed PPS parsing dependency issues in the draft VVC specification, JVET-N1001-v8, were identified by the proponents.

The first issue is that the number of bits for the u(v) syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] are determined by the picture size which is specified in the SPS. The proposed solution is to replace

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x[ i ] * 8

by

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_coarse_x[ i ] * 512 + pps_virtual_boundaries_fine_x[ i ] * 8

where pps_virtual_boundaries_coarse_x is a ue(v) syntax element and pps_virtual_boundaries_fine_x is a u(6) syntax element. The same is proposed for PpsVirtualBoundariesPosY.

The second issue is that currently the for-loop count for parsing brick syntax elements is equal to NumTilesInPic. If uniform_tile_spacing_flag is equal to 1, then NumTilesInPic depends on the pic_width_in_luma_samples  and pic_heigh_in_luma_samples syntax elements in the SPS. This contribution proposes to add a PPS syntax element num_tiles_in_pic_minus1 in case uniform_tile_spacing_flag and brick_splitting_present_flag are both equal to 1 where the value of num_tiles_in_pic_minus1 shall be set equal to NumTilesInPic – 1, with syntax as follows:

| brick_splitting_present_flag | u(1) |
| --- | --- |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| ... |  |

# Introduction and problem statement

Keeping the PPS parsing independent of the SPS is a desirable property in VVC assuming VVC will end up in an HEVC-like process for parameter set activation where parameter sets are activated by VCL NAL units. If the PPS is not parsing independent, PPSes may need to be stored in coded format until a VCL NAL unit activates it.

If the VVC design would differ from the HEVC design and allow parsing dependencies between the PPS and SPS, the proponents propose not to make any of the changes described in this contribution.

Two PPS parsing dependency issues were identified in JVET-N1001-v8. The first concerns virtual boundary signaling and the second concerns uniform tile signaling as described below.

# Virtual boundary signaling

The problem here is that the number of bits for the pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] syntax elements are Ceil( Log2( pic_width_in_luma_samples ) − 3 ) and Ceil( Log2( pic_height_in_luma_samples ) − 3 ) respectively. This makes the number of bits to parse dependent on the picture size which is signaled in the SPS.

The proposed solution in this contribution is to signal the positions of the virtual boundaries by codewords that are not dependent on the picture size as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) { |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_virtual_boundaries_coarse_x[ i ] | ue(v) |
| pps_virtual_boundaries_fine_x[ i ] | u(6) |
| } |  |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) { |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| pps_virtual_boundaries_coarse_y[ i ] | ue(v) |
| pps_virtual_boundaries_fine_y[ i ] | u(6) |
| } |  |
| ... |  |

pps_num_ver_virtual_boundaries specifies the number of pps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the PPS. When pps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_num_ver_virtual_boundaries specifies the number of pps_virtual_boundaries_coarse_x[ i ] and pps_virtual_boundaries_fine_x[ i ] syntax elements that are present in the PPS. When pps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_virtual_boundaries_pos_x[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_x[ i ] is Ceil( Log2( pic_width_in_luma_samples ) − 3 ). pps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the vertical virtual boundary PpsVirtualBoundariesPosX[ i ] is derived as follows:

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x[ i ] * 8	(7-37)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

pps_virtual_boundaries_coarse_x[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples.

pps_virtual_boundaries_fine_x[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ].

The location of the vertical virtual boundary PpsVirtualBoundariesPosX[ i ] is derived as follows:

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_coarse_x[ i ] * 512 + pps_virtual_boundaries_fine_x[ i ] * 8			(7-37)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

PpsVirtualBoundariesPosX[ i ] shall be in the range of 8 to pic_width_in_luma_samples – 1, inclusive.

pps_num_hor_virtual_boundaries specifies the number of pps_virtual_boundaries_pos_y[ i ] syntax elements that are present in the PPS. When pps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_num_hor_virtual_boundaries specifies the number of pps_virtual_boundaries_coarse_y[ i ] and pps_virtual_boundaries_fine_y[ i ] syntax elements that are present in the PPS. When pps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_virtual_boundaries_pos_y[ i ] is used to compute the value of PpsVirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_y[ i ] is Ceil( Log2( pic_height_in_luma_samples ) − 3 ). pps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the horizontal virtual boundary PpsVirtualBoundariesPosY[ i ] is derived as follows:

PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_pos_y[ i ] * 8	(7-38)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

pps_virtual_boundaries_coarse_y[ i ] is used to compute the value of PpsVirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples.

pps_virtual_boundaries_fine_y[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ].

The location of the horizontal virtual boundary PpsVirtualBoundariesPosY[ i ] is derived as follows:

PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_coarse_y[ i ] * 512 + pps_virtual_boundaries_fine_y[ i ] * 8			(7-38)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

PpsVirtualBoundariesPosY[ i ] shall be in the range of 8 to pic_height_in_luma_samples – 1, inclusive.

# Uniform tile signaling

For uniform tile splits, there is a parsing dependency if uniform_tile_spacing_flag and brick_splitting_present_flag are both equal to 1. The reason is that the number of times to parse the brick syntax elements is equal to NumTilesInPic that in this case depends on the pic_width_in_luma_samples  and pic_heigh_in_luma_samples syntax elements in the SPS.

The proposed solution to this parsing dependency problem is to add a PPS syntax element num_tiles_in_pic_minus1 in case uniform_tile_spacing_flag and brick_splitting_present_flag are both equal to 1, as shown in the following:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

num_tiles_in_pic_minus1 plus 1 specifies the number of tiles in each picture referring to the PPS. The value of num_tiles_in_pic_minus1 shall be set equal to NumTilesInPic – 1 and when not present, the value of num_tiles_in_pic_minus1 is inferred to be equal to NumTilesInPic − 1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).