| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0237 |
| --- | --- |

| Title: | AHG17: Compact NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes two changes related to the NAL unit header in VVC.

Firstly, it is proposed to signal an extension flag in the NAL unit header that specifies if the NAL unit header is signaled with one byte or if the NAL unit header is extended with one additional byte. The extension byte is needed to express a NuhLayerId larger than 0 or for certain less commonly used NAL unit types (e.g. reserved or unspecified NAL unit types).

More specifically, the following changes to the NAL unit header are proposed in this contribution:

- Signal a vcl_flag indicating whether the NAL unit is a VCL or non-VCL NAL unit

- Signal nal_unit_type_lsb with 2 bits instead of 4

- Signal a nuh_extension_flag specifying whether a 1-byte NAL unit header extension is present

- If the extension is present signal nuh_layer_id_plus1, nal_unit_type_ext_bit and nuh_reserved_zero_bit in a second NAL unit header byte, else infer nuh_layer_id_plus1 to be equal to 1 and nal_unit_type_ext_bit and nuh_reserved_zero_bit to be equal to 0

- Signal nuh_layer_id_plus1 with 6 bits as in HEVC instead of 7 bits

- Set NALUnitType equal to ( zero_tid_required_flag  <<  4 ) + (vcl_flag << 3 ) + ( nal_unit_type_ext_bit << 2 ) + nal_unit_type_lsb

The estimated luma BD rate numbers by the change are reported to be 0.0%/0.0%/-0.1%/-0.1% for AI/RA/LDB/LDP respectively and the proponents claim higher savings when multiple slices per pictures are used.

Secondly, it is claimed by the proponents that in the current VVC draft there are no reserved IRAP NAL unit type values and that the unspecified NAL unit types are required to have temporal ID equal to 0. This contribution proposes to reserve two IRAP NAL unit type values and to allow two of the four unspecified NAL unit types to have a non-zero temporal ID, at the cost of removing two of the reserved non-IRAP NAL unit type values.

# Introduction

At the JVET meeting in Geneva a NAL unit header design was adopted from JVET-N0067 which assigns a wider range of NUT values while still avoiding MPEG-2 PES start code emulation, eliminating the forbidden_zero_bit and recapturing the bits allocated to temporal ID for NUTs which require temporal ID equal to 0. Further, a layer concept without inter-layer referencing was adopted from JVET-N0278-v3.

The resulting NAL unit header as specified in the VVC draft (JVET-N1001-v8) is signaled using two bytes as shown in the syntax table below.

#### 7.3.1.2	NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| nuh_layer_id_plus1 | u(7) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

It can be expected that for the mainstream usage of the VVC codec, layer ID larger than 0 may not be frequently used and that the second byte of the NAL unit header then most of the time would be equal to 0.

In addition, the draft VVC specification has a capacity of 32 NAL unit types, whereof 18 have been defined and the rest are either reserved or unspecified. Thus, in version 1 of VVC, a large part of the available NAL unit slots may not be allocated.

The authors therefore believe that it would be beneficial to signal the more commonly used information of the NAL unit header with one byte instead of two and signal the less commonly used information, layer ID and rarely used NAL unit types, in a NAL unit header extension byte.

# Proposal

This contribution proposes two changes related to the NAL unit header for VVC. These changes are not related to each other and may be considered separately.

## Proposed change 1 – Compact NAL unit header

It is proposed to signal an extension flag in the NAL unit header that specifies if the NAL unit header is signaled with one byte or if the NAL unit header is extended with one additional byte. The extension byte is needed to express a NuhLayerId larger than 0 or for certain less commonly used NAL unit types (e.g. reserved or unspecified NAL unit types).

More specifically, the following changes to the NAL unit header are proposed in this contribution:

- Signal a vcl_flag indicating whether the NAL unit is a VCL or non-VCL NAL unit

- Signal nal_unit_type_lsb with 2 bits instead of 4

- Signal a nuh_extension_flag specifying whether a 1-byte NAL unit header extension is present

- If the extension is present, signal nuh_layer_id_plus1, nal_unit_type_ext_bit and nuh_reserved_zero_bit in a second NAL unit header byte, else infer nuh_layer_id_plus1 to be equal to 1 and nal_unit_type_ext_bit and nuh_reserved_zero_bit to be equal to 0

- Signal nuh_layer_id_plus1 with 6 bits as in HEVC instead of 7 bits

- Set NALUnitType equal to ( zero_tid_required_flag  <<  4 ) + ( vcl_flag << 3 ) + ( nal_unit_type_ext_bit << 2 ) + nal_unit_type_lsb

### Design motivation

The proposal suggests compacting the NAL unit header to one byte in the most common cases instead of two bytes as in the current VVC design. The obvious advantage is bitrate savings which is further discussed in section 2.1.2 below.

It can be observed in Table 7-1 of the VVC draft that in the current NAL unit header design the first bit of the nal_unit_type_lsb corresponds to whether the NAL unit is a non-VCL NAL unit or if it is a VCL NAL unit or has an unspecified type. We propose to make this property explicit by using an explicit vcl_flag in the NAL unit header. This is purely editorial and does not play a role in compacting the NAL unit header to one byte.

It can also be seen from Table 7-1 that when the second bit of nal_unit_type_lsb is set to 1, the NAL unit type is often reserved or unspecified. Thus, this bit is rarely set to 1. We therefore suggest signaling this bit in a NAL unit header extension byte and use the bit in the first byte of the NAL unit header to indicate if the extension byte is present or not. The NAL unit type can be easily derived by shifting and adding the codewords for zero_tid_required_flag, vcl_flag, nal_unit_type_ext_bit and nal_unit_type_lsb.

In the VVC draft, there are currently 18 specified NAL unit types, 10 NAL unit types reserved for future use and 4 unspecified types. With the proposed design, the order of NAL unit types of table 7-1 results in that all reserved and unspecified values are signaled using two bytes for the NAL unit header. All specified values may be signaled with only one byte in the NAL unit header, except for the APS_NUT and the VPS_NUT which would require that the NAL unit header is signaled with two bytes. It could of course be possible to rearrange the order of the NAL unit types to adjust which NAL unit types may be signaled with only one byte for the NAL unit header and which NAL unit types would require two bytes. For instance, the positions of APS_NUT and AUD_NUT in Table 7-1 could be swapped.

A consequence of the proposed design is that the layer ID is signaled with 6 bits as in HEVC, instead of 7 bits as in the current VVC design. The proponents believe that having 62 layer IDs is sufficient for all use cases.

### Estimated bitrate savings

It is assessed that in the mainstream usage of the VVC codec the NAL unit type can be signaled with one byte instead of two as in the current version of the VVC draft. Only when a NuhLayerId larger than 0 or a less common NAL unit type is used (e.g. reserved or unspecified), the NAL unit header is signaled with two bytes.

The estimated bitrate savings varies and depends on the application, with varying settings for the overall bitrate, frame rate and the number of NAL units per picture, where the number of NAL units per picture depends on factors such as the number of slices per picture, if APSs are used, if additional SEI messages are used, etc.

We have estimated the bitrate savings for each sequence by counting the number of NAL units X in the decoder trace file for each anchor sequence and deducted 8*X bits from the total number of bits for each sequence.

The estimated bitrate savings for VTM-5.0 CTC is shown in Table 1 below. In the estimation, we have swapped the positions of APS_NUT and AUD_NUT in Table 7-1 such that the APS NAL unit header is coded with one byte.

Note that there are no losses or noise in these numbers, since the gains do not come from change in quality but purely from saving one byte per NAL unit header. For the Johnny sequence at QP37 in LDB, the bitrate savings are estimated to be 0.58%.

Table 1	Estimated results for encoding NAL unit headers with 1 byte instead of 2 bytes for the CTC

In the CTC, there is always one slice per picture, SEI messages and access unit delimiters are not used and APSs are used for every picture for AI and on average for about every third picture for RA, LDB and LDP. This may not be realistic for all common use cases. Below are some estimated bitrate savings for some additional non-CTC use cases:

|  | 4k video streaming 10 Mbps @ 60 fps | 1080p video 2 Mbps @ 60 fps | 4k 360 video 8 Mbps @ 60 fps |
| --- | --- | --- | --- |
| Slices per picture | 20 | 5 | 96 |
| SEI messages per picture | 2 | 1 | 2 |
| APSs per picture | 0.33 | 0.33 | 0.33 |
| bit savings (bits per second): | 10 718 | 3038 | 47 198 |
| bit savings (%): | 0.11 | 0.15 | 0.59 |

Table 2 Estimated bitrate savings for some additional non-CTC use cases

In JVET-N0051 from Tencent it was also proposed to signal the NAL unit header with one byte instead of two. In that contribution the expected bitrate savings were discussed and estimated for different applications, with estimates ranging from 0.048% to well above 1%.

### VVC specification text for proposed change 1

The proposed changes on top of the VVC draft (JVET-N1001-v8) is marked in yellow.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| vcl_flag | u(1) |
| nal_unit_type_lsb | u(4)u(2) |
| nuh_extension_flag | u(1) |
| if nuh_extension_flag { |  |
| nuh_layer_id_plus1 | u(7)u(6) |
| nal_unit_type_ext_bit | u(1) |
| nuh_reserved_zero_bit | u(1) |
| } |  |
| } |  |

#### NAL unit header semantics

< The semantics of zero_tid_required_flag and nuh_temporal_id_plus1 are unchanged. >

vcl_flag equal to 1 indicates that the NAL unit is a VCL NAL unit or that the NAL unit type is unspecified. vcl_flag equal to 0 indicates that the NAL unit is a non-VCL NAL unit.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type.

nuh_extension_flag equal to 1 specifies that the NAL unit header has an extension. nuh_extension_flag equal to 0 specifies that there is no NAL unit header extension.

nal_unit_type_ext_bit specifies the extension bit for the NAL unit type. If not present, nal_unit_type_ext_bit is inferred to be equal to 0.

The variable NalUnitType, which specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1, is derived as follows:

NalUnitType = ( zero_tid_required_flag  <<  4 ) + ( vcl_flag << 3 ) + ( nal_unit_type_ext_bit << 2 ) + nal_unit_type_lsb		(7-2)

NAL units that have NalUnitType in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 3 – NAL unit types in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of NalUnitType is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these NalUnitType values, and in the design of decoders that interpret the content of NAL units with these NalUnitType values. This Specification does not define any management for these values. These NalUnitType values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same NalUnitType value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of NalUnitType.

NOTE 4 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 4 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 6 5..7 | RSV_NVCL65.. RSV_NVCL7 | Reserved | non-VCL |
| 8 | TRAIL_NUT | Coded slice of a non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 9 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 10 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 11 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 12..15 | RSV_VCL_12.. RSV_VCL_15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 21..23 | RSV_NVCL21.. RSV_NVCL23 | Reserved | non-VCL |
| 24 25 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 26 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

NOTE 5 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 6 – An instantaneous decoding refresh (IDR) picture having NalUnitType equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having NalUnitType equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

nuh_layer_id_plus1 specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id_plus1 shall be in the range of 1 to 62126, inclusive. The value of 63127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 63127 for nuh_layer_id_plus1 in a NAL unit.

NOTE 7 – The value of 63127 for nuh_layer_id_plus1 may be used to indicate an extended layer identifier in a future extension of this Specification.

The variable NuhLayerId is derived as follows:

NuhLayerId = nuh_layer_id_plus1 − 1							      (7-3)

The value of NuhLayerId shall be the same for all VCL NAL units of a coded picture. The value of NuhLayerId of a coded picture is the value of the NuhLayerId of the VCL NAL units of the coded picture. If not present, the value of nuh_layer_id_plus1 is inferred to be equal to 1.

nuh_reserved_zero_bit shall be equal to '0' when present. The value 1 of nuh_reserved_zero_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with nuh_reserved_zero_bit equal to '1'.

## Proposed change 2 – Fix for reserved IRAP and unspecified NAL unit types

It is claimed by the proponents that in the current VVC draft there are no reserved IRAP NAL unit type values and that all the unspecified NAL unit types are required to have temporal ID equal to 0. This contribution proposes to reserve two IRAP NAL unit type values and to allow two of the four unspecified NAL unit types to have a non-zero temporal ID, at the cost of removing two of the reserved non-IRAP NAL unit type values. The current design and the proposed change are illustrated in the tables below.

| NalUnitType | Name of NalUnitType | Description | tID constraints |
| --- | --- | --- | --- |
| 12, 13 | RSV_VCL_12 RSV_VCL_13 | Reserved non-IRAP VCL | No constraint |
| 14, 15 | RSV_VCL_14 RSV_VCL_15 | Reserved non-IRAP VCL | No constraint |
| 28, 29 | UNSPEC28 UNSPEC29 | Unspecified non-VCL | tID = 0 |
| 30, 31 | UNSPEC30 UNSPEC31 | Unspecified non-VCL | tID = 0 |

Table 3	NAL unit types 12-15 and 28-31 in the current VVC design.

| NalUnitType | Name of NalUnitType | Description | tID constraints | Nature of change |
| --- | --- | --- | --- | --- |
| 12, 13 | RSV_VCL_12 RSV_VCL_13 | Reserved non-IRAP VCL | No constraint | No change |
| 14, 15 | UNSPEC14 UNSPEC15 | Unspecified non-VCL | No constraint | Moved to enable tID > 0 |
| 28, 29 | RSV_VCL_28 RSV_VCL_29 | Reserved IRAP VCL | tID = 0 | Converted from reserved non-IRAP |
| 30, 31 | UNSPEC30 UNSPEC31 | Unspecified non-VCL | tID = 0 | No change |

Table 4	Proposed change for NAL unit types 12-15 and 28-31 in VVC.

### VVC specification text for proposed change 2

The proposed changes on top of the VVC draft (JVET-N1001-v8) is marked in yellow.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type.

The variable NalUnitType, which specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1, is derived as follows:

NalUnitType = ( zero_tid_required_flag  <<  4 ) + nal_unit_type_lsb	(7-2)

NAL units that have NalUnitType equal to UNSPEC14, UNSPEC15, UNSPEC30 or UNSPEC31,  in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 3 – NAL unit types equal to UNSPEC14, UNSPEC15, UNSPEC30 or UNSPEC31in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of NalUnitType is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these NalUnitType values, and in the design of decoders that interpret the content of NAL units with these NalUnitType values. This Specification does not define any management for these values. These NalUnitType values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same NalUnitType value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of NalUnitType.

NOTE 4 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 4 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 5..7 | RSV_NVCL5.. RSV_NVCL7 | Reserved | non-VCL |
| 8 | TRAIL_NUT | Coded slice of a non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 9 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 10 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 11 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 12..13 | RSV_VCL_12.. RSV_VCL_1315 | Reserved non-IRAP VCL NAL unit types | VCL |
| 14..15 | UNSPEC14 UNSPEC15 | Unspecified | non-VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 21..23 | RSV_NVCL21.. RSV_NVCL23 | Reserved | non-VCL |
| 24 25 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 26 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28..29 | RSV_VCL_28 RSV_VCL_29 | Reserved IRAP VCL NAL unit types | VCL |
| 3028..31 | UNSPEC3028.. UNSPEC31 | Unspecified | non-VCL |

##### …

##### Order of NAL units and coded pictures and their association to access units

…

-	NAL units with NalUnitType equal to UNSPEC14, UNSPEC15, UNSPEC30 or UNSPEC31, in the range of UNSPEC28..UNSPEC29 (when present).

…

-	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, APS NAL units, prefix SEI NAL units, NAL units with  NalUnitType equal to RSV_NVCL_5, RSV_NVCL_6, RSV_NVCL_21, or RSV_NVCL_22, or NAL units with NalUnitType equal to UNSPEC14, UNSPEC15, UNSPEC30 or UNSPEC31, in the range of UNSPEC28..UNSPEC29 are present, they shall not follow the last VCL NAL unit of the access unit.

…

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).