| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0238-v3 |
| --- | --- |

| Title: | AHG17: Parameters in PPS or slice headers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Per Wennersten Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.comper.wennersten@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The proponents report that some of the slice parameters in VVC stay constant for all slices in the bitstream for some CTC configurations. This contribution proposes a mechanism to enable signaling some of the slice parameters in VVC either in the PPS or in the slice header.

More specifically, the following changes to VVC are proposed in this contribution:

- Decode a pps_or_slice_flag in the PPS that specifies whether the parameter values of the below listed syntax elements may be conditionally signalled in the PPS or slice headers, or if they are always signaled in slice headers.

  - dep_quant_enabled_flag,

  - ref_pic_list_sps_flag,

  - slice_temporal_mvp_enabled_flag,

  - mvd_l1_zero_flag,

  - collocated_from_l0_flag,

  - six_minus_max_num_merge_cand

  - five_minus_max_num_subblock_merge_cand

  - max_num_merge_cand_minus_max_num_triangle_cand

- If pps_or_slice_flag is equal to 1, then for each of the above syntax elements:

  - Decode a corresponding syntax element in the PPS.

  - If the value of the corresponding syntax element in the PPS is equal to 0, then the parameter value is signalled by a syntax element in the slice header. Otherwise, the parameter value for the slice header syntax element is derived as the value of the corresponding syntax element in the PPS minus 1.

The luma BD-rate numbers for the CTC are reported to be 0.0%/0.0%/-0.1%/0.0% for AI/RA/LDB/LDP respectively and the proponents claim higher gains when multiple slices per picture are used.

Specification text on top of JVET-N1001-v8 is attached with the contribution.

In version 2 of the contribution the presented slide set is attached.

In version 3 of the contribution the global signalling of parameters has been moved from SPS to PPS and the descriptor tu(v) has been replaced with u(2) as per decision in the JVET plenary.

# Introduction

It has been observed by the proponents that some parameter values in the slice header is constant in all slice headers in the entire CTC. The table below shows in which CTC configurations using VTM 5.0 certain parameters stay constant in the CTC.

| Slice parameter name | Constant in |  |  |  |
| --- | --- | --- | --- | --- |
|  | AI | RA | LDB | LDP |
| dep_quant_enabled_flag | x | x | x | x |
| short_term_ref_pic_set_sps_flag | x |  | x | x |
| slice_temporal_mvp_enabled_flag |  |  | x | x |
| mvd_l1_zero_flag |  |  | x |  |
| collocated_from_l0_flag |  |  | x |  |
| six_minus_max_num_merge_cand | x | x | x | x |
| five_minus_max_num_affine_merge_cand |  | x | x | x |
| max_num_merge_cand_minus_max_num_triangle_cand |  | x | x |  |

Table 1	Slice parameters that stay constant for each CTC configuration using VTM 5.0

It is noted that a couple of the slice parameter names differ between VTM 5.0 and the draft VVC specification, JVET-N1001-v8. We therefore suggest to change the name of short_term_ref_pic_set_sps_flag and five_minus_max_num_affine_merge_cand to ref_pic_list_sps[0], ref_pic_list_sps[1] and five_minus_max_num_subblock_merge_cand in the VTM software.

# Proposal

We believe it is redundant to send parameters in the slice header which stay constant for all slice headers in the bitstream. We therefore propose a mechanism to enable signaling some of the slice parameter values either in the PPS or in the slice header.

More specifically, the following changes to VVC are proposed in this contribution:

- Decode a pps_or_slice_flag in the PPS that specifies whether the parameter values of the below listed syntax elements may be conditionally signalled in the PPS or slice headers, or if they are always signaled in slice headers.

  - dep_quant_enabled_flag,

  - ref_pic_list_sps_flag,

  - slice_temporal_mvp_enabled_flag,

  - mvd_l1_zero_flag,

  - collocated_from_l0_flag,

  - six_minus_max_num_merge_cand

  - five_minus_max_num_subblock_merge_cand

  - max_num_merge_cand_minus_max_num_triangle_cand

- If the pps_or_slice_flag is equal to 1, then for each of the above syntax elements:

  - Decode a corresponding syntax element in the PPS.

  - If the value of the corresponding syntax element in the PPS is equal to 0, then the parameter value is signalled by a syntax element in the slice header. Otherwise, the parameter value for the slice header syntax element is derived as the value of the corresponding syntax element in the PPS minus 1.

## Proposed changes to the VVC specification

The proposed changes on top of the VVC draft (JVET-N1001-v8) are marked in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| … |  |
| } |  |

sps_temporal_mvp_enabled_flag equal to 0 specifies that slice_temporal_mvp_enabled_flag is present in the slice headers of slices with slice_type not equal to I in the CVS. sps_ temporal_mvp_enabled_flag equal to 1 specifies that slice_temporal_mvp_enabled_flag is not present in slice headers and that temporal motion vector predictors are not used in the CVS.

sps_sbtmvp_enabled_flag equal to 1 specifies that subblock-based temporal motion vector predictors may be used in decoding of pictures with all slices having slice_type not equal to I in the CVS. sps_sbtmvp_enabled_flag equal to 0 specifies that subblock-based temporal motion vector predictors are not used in the CVS. When sps_sbtmvp_enabled_flag is not present, it is inferred to be equal to 0.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_or_slice_flag | u(1) |
| if ( pps_or_slice_flag ) { |  |
| pps_or_slice_dep_quant_enabled_flag | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_or_slice_ref_pic_list_sps_flag[ i ] | u(2) |
| pps_or_slice_temporal_mvp_enabled_flag | u(2) |
| pps_or_slice_mvd_l1_zero_flag | u(2) |
| pps_or_slice_collocated_from_l0_flag | u(2) |
| pps_or_slice_six_minus_max_num_merge_cand | ue(v) |
| pps_or_slice_five_minus_max_num_subblock_merge_cand | ue(v) |
| pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if( pps_or_slice_temporal_mvp_enabled_flag != 1 ) |  |
| pps_sbtmvp_enabled_flag | u(1) |
| … |  |
| } |  |

pps_or_slice_flag equal to 0 specifies that pps_or_slice_dep_quant_enabled_flag, pps_or_slice_ref_pic_list_sps_flag[i], pps_or_slice_temporal_mvp_enabled_flag, pps_or_slice_mvd_l1_zero_flag,  pps_or_slice_collocated_from_l0_flag, pps_or_slice_six_minus_max_num_merge_cand, pps_or_slice_five_minus_max_num_subblock_merge_cand and pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand are inferred to be equal to 0. pps_or_slice_flag equal to 1 specifies that these syntax elements are present in the PPS.

pps_or_slice_dep_quant_enabled_flag equal to 0 specifies that dep_quant_enabled_flag is present in slice headers. pps_or_slice_dep_quant_enabled_flag equal to 1 or 2 specifies that dep_quant_enabled_flag is inferred to be equal to  pps_or_slice_dep_quant_enabled_flag minus 1.  pps_or_slice_dep_quant_enabled_flag equal to 3 is reserved for future use.

pps_or_slice_ref_pic_list_sps_flag[ i ] equal to 0 specifies that ref_pic_list_sps[ i ] is present in slice headers. pps_or_slice_ref_pic_list_sps_flag[ i ] equal to 1 or 2 specifies that ref_pic_list_sps[ i ] is inferred to be equal to pps_or_slice_ref_pic_list_sps_flag[ i ] minus 1.  pps_or_slice_ref_pic_list_sps_flag[ i ] equal to 3 is reserved for future use.

pps_or_slice_temporal_mvp_enabled_flag equal to 0 specifies that slice_temporal_mvp_enabled_flag is present in the slice headers of slices with slice_type not equal to I in the CVS. pps_or_slice_temporal_mvp_enabled_flag equal to 1 or 2 specifies that slice_temporal_mvp_enabled_flag is inferred to be equal to  pps_or_slice_temporal_mvp_enabled_flag minus 1. pps_or_slice_temporal_mvp_enabled_flag equal to 3 is reserved for future use.

pps_or_slice_mvd_l1_zero_flag equal to 0 specifies that mvd_l1_zero_flag is present in slice headers. pps_or_slice_mvd_l1_zero_flag equal to 1 or 2 specifies that mvd_l1_zero_flag is inferred to be equal to  pps_or_slice_mvd_l1_zero_flag minus 1.  pps_or_slice_mvd_l1_zero_flag equal to 3 is reserved for future use.

pps_or_slice_collocated_from_l0_flag equal to 0 specifies that collocated_from_l0_flag is present in slice headers. pps_or_slice_collocated_from_l0_flag equal to 1 or 2 specifies that collocated_from_l0_flag is inferred to be equal to pps_or_slice_collocated_from_l0_flag minus 1. pps_or_slice_collocated_from_l0_flag equal to 3 is reserved for future use.

pps_or_slice_six_minus_max_num_merge_cand equal to 0 specifies that six_minus_max_num_merge_cand is present in slice headers. pps_or_slice_six_minus_max_num_merge_cand larger than 0 specifies that six_minus_max_num_merge_cand is inferred to be equal to pps_or_slice_six_minus_max_num_merge_cand minus 1. The value of pps_or_slice_six_minus_max_num_merge_cand shall be in the range of 0 to 6, incusive.

pps_or_slice_five_minus_max_num_subblock_merge_cand equal to 0 specifies that five_minus_max_num_subblock_merge_cand is present in slice headers. pps_or_slice_five_minus_max_num_subblock_merge_cand larger than 0 specifies that five_minus_max_num_subblock_merge_cand is inferred to be equal to pps_or_slice_five_minus_max_num_subblock_merge_cand minus 1. The value of pps_or_slice_five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 6, inclusive.

pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice headers. pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand larger than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand minus 1. The value of pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand shall be in the range of 0 to MaxNumMergeCand – 1.

pps_sbtmvp_enabled_flag equal to 1 specifies that subblock-based temporal motion vector predictors may be used in decoding of pictures with all slices having slice_type not equal to I in the CVS. pps_sbtmvp_enabled_flag equal to 0 specifies that subblock-based temporal motion vector predictors are not used in the CVS. When pps_sbtmvp_enabled_flag is not present, it is inferred to be equal to 0.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( NalUnitType != IDR_W_RADL && NalUnitType != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_or_slice_ref_pic_list_sps_flag[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| … |  |
| if ( slice_type != I ) { |  |
| if( !pps_or_slice_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B && !pps_or_slice_mvd_l1_zero_flag ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_or_slice_collocated_from_l0_flag ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| … |  |
| if( !pps_or_slice_six_minus_max_num_merge_cand ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag && !pps_or_slice_five_minus_max_num_subblock_merge_cand ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_or_slice_max_num_merge_cand_minus_max_num_triangle_cand ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } else if ( sps_ibc_enabled_flag && !pps_or_slice_six_minus_max_num_merge_cand ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| … |  |
| if ( !pps_or_slice_dep_quant_enabled_flag ) |  |
| dep_quant_enabled_flag | u(1) |
| … |  |
| byte_alignment( ) |  |
| } |  |

# Results

The BD-rate numbers for the CTC using VTM-5.0 is shown below. For AI, sps_or_slice_flag is set to 0 since an SPS is sent for each picture. Note that there is no noise in these BD-rate numbers, since the gains do not come from change in quality but purely from saving bits by signaling constant parameters in the SPS instead of in the slice headers. For the Johnny sequence in LDB, the BD-rate number is -0.37%.

## Estimated results for non-CTC configurations

In a non-CTC configuration with multiple slices the bit savings are expected to be higher. A rough estimation is done in Table 5 below by multiplying the CTC bit savings with the number of slices:

|  | Estimated BD-rate (Y) |  |  |
| --- | --- | --- | --- |
| X slices per picture | RA | LDB | LDP |
| 6 | -0.06% | -0.48% | -0.18% |
| 24 (OMAF example) | -0.24% | -1.92% | -0.72% |

Table 2	Estimated luma BD-rate for multiple slices per picture

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).