| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0240-v1 |
| --- | --- |

| Title: | AHG 8: Adaptive Resolution Change (ARC) with downsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Samuelsson, S. Deshpande, A. Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution relates to Adaptive Resolution Change (ARC). The contribution asserts that several of the use-cases for ARC involves downsampling and it is suggested to use filters with low-pass characteristics for the cases where a reference picture is of higher resolution than the current picture. For the one-step approach of ARC this would mean to use a different interpolation filter in the Motion Compensation process. For the two-step approach of ARC (where a new reference picture is created from a reference picture with different resolution) it would mean to generate a downsampled reference picture using filters adjusted for downsampling. The contribution includes two suggested sets of interpolation filters: one for 2:1 scaling and one for 4:1 scaling. Both are integer versions of 8-tap Lanczos filters. In the second version of the proposal, an example with 2x scaling factor is added as Annex B, and an example of 4x scaling factor using the SHVC 12-tap filter is added as Annex C for reference.

# Introduction

The general concept of Adaptive Resolution Change (ARC) has existed for a long time in video compression, with Annex P of H.263 [1] as the first standardized approach for ARC. There have been several different proposals in JVET to add support for ARC in VVC and it can be seen from the meeting notes of the 14th JVET meeting [2] that “It was agreed in principle to … Having the resolution of the stored reference picture being different from that of the current picture”. This can be realized using a block-based one-step approach for ARC through taking account for the scaling factor during motion compensation as proposed in JVET-N0279 [3].

ARC can be used in many different scenarios and has a lot of different use case. Several of the different use cases involves predicting from a reference picture with higher spatial resolution than the current picture, for example:

- The coded resolution is reduced due to a (significant) drop in available bandwidth.

- The rendered resolution is reduced due to events triggered by the system, such as change of active speaker in a multiparty conversation, or switch between “main video” and screen sharing.

- The rendered resolution is reduced due to events triggered by the user, such as switching from full-screen to window/thumbnail, or rotating a handheld device from landscape to portrait.

- The received resolution is reduced in an Adaptive Bit Rate (ABR) streaming service when switching from a higher bitrate representation to a lower bitrate representation. If the representations are encoded with open-GOP Intra pictures, the leading pictures will use reference pictures of higher resolution for prediction.

Given that there are several important use cases that involves predicting from a refence picture with higher spatial resolution, it is suggested to ensure that the ARC scheme in VVC provides support for efficient handling of ARC with downsampling.

# Downsampling for Adaptive Resolution Change

In document JVET-N0279 [3] it is proposed to integrate support for ARC by including calculation of a scaling factor in motion compensation so that inter-prediction can be performed between pictures of different resolutions. This is a lightweight approach for ARC that does not involve any separate resampling process and does not require any additional storage of resampled reference pictures.

The motion interpolation filter in VVC does not constitute a low-pass filter and does not provide good quality when applied in a downsampling scenario. For scaling ratios close to 1, the VVC motion interpolation filter will provide acceptable result, but for large scaling ratios, the result will show severe aliasing artifacts. For integer scaling ratios, such as 2:1 and 4:1, no filtering is performed, resulting in that the downsampling filter corresponds to a Nearest Neighbor filter. The cropped part from the CatRobot sequence, shown in Figure 1., is used to demonstrate the effect of the downsampling.

Figure 1. Original cropped part from the CatRobot sequence.

Figure 2. shows the cropped part from Figure 1. downsampled by a factor of 4 and then upsampled again using the 8-tap interpolation filter from VVC.

Figure 2. 4x downsampling and 4x upsampling using VVC 8-tap interpolation filters.

Figure 3. shows the cropped part from Figure 1. downsampled by a factor of 4 using the proposed interpolation filters and then upsampled again using the 8-tap interpolation filter from VVC.

Figure 3. 4x downsampling using proposed 8-tap filter and 4x upsampling using VVC 8-tap filter

# Proposed interpolation filters

If the downsampling factor is larger than 1.5:1 it is proposed to use the following filter coefficients for motion compensation:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | −5 | 4 | 37 | 56 | 37 | 4 | −5 | 0 |
| 1 | −5 | 3 | 34 | 57 | 39 | 6 | −5 | −1 |
| 2 | −5 | 2 | 32 | 57 | 41 | 7 | −5 | −1 |
| 3 | −4 | 1 | 30 | 56 | 42 | 9 | −5 | −1 |
| 4 | −4 | 0 | 28 | 55 | 45 | 11 | −5 | −2 |
| 5 | −4 | −1 | 25 | 55 | 47 | 13 | −5 | −2 |
| 6 | −3 | −2 | 23 | 53 | 48 | 15 | −4 | −2 |
| 7 | −3 | −3 | 21 | 53 | 50 | 17 | −4 | −3 |
| 8 | −3 | −4 | 19 | 52 | 52 | 19 | −4 | −3 |
| 9 | −3 | −4 | 17 | 50 | 53 | 21 | −3 | −3 |
| 10 | −2 | −4 | 15 | 48 | 53 | 23 | −2 | −3 |
| 11 | −2 | −5 | 13 | 47 | 55 | 25 | −1 | −4 |
| 12 | −2 | −5 | 11 | 45 | 55 | 28 | 0 | −4 |
| 13 | −1 | −5 | 9 | 42 | 56 | 30 | 1 | −4 |
| 14 | −1 | −5 | 7 | 41 | 57 | 32 | 2 | −5 |
| 15 | −1 | −5 | 6 | 39 | 57 | 34 | 3 | −5 |

Table 1. Proposed interpolation filter for 2:1 downsampling

If the downsampling factor is larger than 3:1 it is proposed to use the following filter coefficients for motion compensation:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | 7 | 17 | 26 | 28 | 26 | 17 | 7 | 0 |
| 1 | 7 | 16 | 25 | 28 | 25 | 17 | 8 | 2 |
| 2 | 6 | 15 | 24 | 29 | 26 | 18 | 8 | 2 |
| 3 | 6 | 15 | 24 | 28 | 26 | 18 | 9 | 2 |
| 4 | 5 | 14 | 23 | 28 | 27 | 19 | 9 | 3 |
| 5 | 5 | 13 | 23 | 28 | 27 | 19 | 10 | 3 |
| 6 | 5 | 13 | 22 | 28 | 27 | 20 | 10 | 3 |
| 7 | 4 | 12 | 22 | 28 | 27 | 20 | 11 | 4 |
| 8 | 4 | 12 | 21 | 27 | 27 | 21 | 12 | 4 |
| 9 | 4 | 11 | 20 | 27 | 28 | 22 | 12 | 4 |
| 10 | 3 | 10 | 20 | 27 | 28 | 22 | 13 | 5 |
| 11 | 3 | 10 | 19 | 27 | 28 | 23 | 13 | 5 |
| 12 | 3 | 9 | 19 | 27 | 28 | 23 | 14 | 5 |
| 13 | 2 | 9 | 18 | 26 | 28 | 24 | 15 | 6 |
| 14 | 2 | 8 | 18 | 26 | 29 | 24 | 15 | 6 |
| 15 | 2 | 8 | 17 | 25 | 28 | 25 | 16 | 7 |

Table 2. Proposed interpolation filter for 4:1 downsampling

Both filters are integer versions of Lanczos filters for scaling factors 2:1 and 4:1, with cut-off frequencies of 0.9π and 0.8π, respectively.

Corresponding 4-tap filters for chroma are included in Annex A.

# Proposed text

The contribution JVET-N0279 [3], proposes a process for determining corresponding positions in reference pictures of different resolution with a fixed-point representation of the scaling factor. This contribution proposes to extend that process so that different motion compensation interpolation filters are used depending on the scaling factor. Using the fixed-point precision from JVET-N0279 [3], where scaling factor values have 14 binary decimals, the proposed thresholds would be 24576 (for 1.5:1) and 49152 (for 3:1), respectively.

Thus, in addition to the derivation of the full-pel and fractional-pel location proposed in JVET-N0279, the following is proposed.

If scale_fp > 49152, luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 18-3.

Otherwise, if scale_fp > 24576, luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 18-4.

Otherwise, luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 18-5.

Where scale_fp is the fixed-point scaling factor for the respective direction (horizontal or vertical) and where Table 1 and Table 2 above would be added to the WD as Table 18-4 and Table 18-3, respectively.

Corresponding changes would apply for chroma using the tables presented in Annex A.

## Downsampling in two-step ARC approach

Adaptive resolution change can be realized with a two-step approach in which additional reference pictures are created by resampling existing reference pictures. The additional reference pictures could either be stored in the Decoded Picture Buffer (DPB) or in a temporary memory buffer. The filters proposed above for the one-step ARC approach can also be used in the two-step ARC approach to provide additional reference pictures that do not suffer from aliasing artifacts.

# References

- ITU-T H.263: Video coding for low bit rate communication, Annex P “Reference Picture Resampling”.

- JVET-N_Notes_dD, https://www.itu.int/wftp3/av-arch/jvet-site/2019_03_N_Geneva/JVET-N_Notes_dD.docx

- Peisong Chen, Tim Hellman, Brian Heng, Wade Wan, Minhua Zhou, “AHG 19: Adaptive Resolution Change", JVET-N0279, March 2019.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex A. Proposed 4-tap downsampling filters for chroma.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 32 | 64 | 32 | 0 |
| 1 | 30 | 64 | 33 | 1 |
| 2 | 28 | 64 | 35 | 1 |
| 3 | 27 | 63 | 36 | 2 |
| 4 | 25 | 64 | 37 | 2 |
| 5 | 24 | 63 | 39 | 2 |
| 6 | 22 | 63 | 40 | 3 |
| 7 | 21 | 62 | 42 | 3 |
| 8 | 20 | 61 | 43 | 4 |
| 9 | 18 | 60 | 45 | 5 |
| 10 | 17 | 60 | 46 | 5 |
| 11 | 16 | 59 | 47 | 6 |
| 12 | 15 | 58 | 48 | 7 |
| 13 | 13 | 57 | 50 | 8 |
| 14 | 12 | 56 | 52 | 8 |
| 15 | 11 | 55 | 53 | 9 |
| 16 | 10 | 54 | 54 | 10 |
| 17 | 9 | 53 | 55 | 11 |
| 18 | 8 | 52 | 56 | 12 |
| 19 | 8 | 50 | 57 | 13 |
| 20 | 7 | 48 | 58 | 15 |
| 21 | 6 | 47 | 59 | 16 |
| 22 | 5 | 46 | 60 | 17 |
| 23 | 5 | 45 | 60 | 18 |
| 24 | 4 | 43 | 61 | 20 |
| 25 | 3 | 42 | 62 | 21 |
| 26 | 3 | 40 | 63 | 22 |
| 27 | 2 | 39 | 63 | 24 |
| 28 | 2 | 37 | 64 | 25 |
| 29 | 2 | 36 | 63 | 27 |
| 30 | 1 | 35 | 64 | 28 |
| 31 | 1 | 33 | 64 | 30 |

Table A.1. Proposed chroma interpolation filter for 2:1 downsampling

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 38 | 52 | 38 | 0 |
| 1 | 34 | 49 | 36 | 9 |
| 2 | 33 | 49 | 36 | 10 |
| 3 | 32 | 48 | 37 | 11 |
| 4 | 31 | 48 | 38 | 11 |
| 5 | 30 | 48 | 38 | 12 |
| 6 | 29 | 48 | 38 | 13 |
| 7 | 28 | 47 | 39 | 14 |
| 8 | 27 | 47 | 40 | 14 |
| 9 | 27 | 46 | 40 | 15 |
| 10 | 26 | 46 | 40 | 16 |
| 11 | 25 | 46 | 41 | 16 |
| 12 | 24 | 45 | 42 | 17 |
| 13 | 23 | 45 | 42 | 18 |
| 14 | 22 | 44 | 43 | 19 |
| 15 | 21 | 44 | 43 | 20 |
| 16 | 20 | 44 | 44 | 20 |
| 17 | 20 | 43 | 44 | 21 |
| 18 | 19 | 43 | 44 | 22 |
| 19 | 18 | 42 | 45 | 23 |
| 20 | 17 | 42 | 45 | 24 |
| 21 | 16 | 41 | 46 | 25 |
| 22 | 16 | 40 | 46 | 26 |
| 23 | 15 | 40 | 46 | 27 |
| 24 | 14 | 40 | 47 | 27 |
| 25 | 14 | 39 | 47 | 28 |
| 26 | 13 | 38 | 48 | 29 |
| 27 | 12 | 38 | 48 | 30 |
| 28 | 11 | 38 | 48 | 31 |
| 29 | 11 | 37 | 48 | 32 |
| 30 | 10 | 36 | 49 | 33 |
| 31 | 9 | 36 | 49 | 34 |

Table A.2. Proposed chroma interpolation filter for 4:1 downsampling

Annex B. Example 2x downsampling and 2x upsampling.

Figure B.1. 2x downsampling and 2x upsampling using VVC 8-tap interpolation filters.

Figure B.2. 2x downsampling using proposed 8-tap filter and 2x upsampling using VVC 8-tap filter

Annex C. Example 4x downsampling and 4x upsampling using SHVC filters.

In SHVC software and anchor generation a 12-tap filter is used for downsampling. The result of using that downsampling filter is included here for reference.

Figure C.1. 4x downsampling using SHVC 12-tap filter and 4x upsampling using VVC 8-tap filter